3GPP TSG-WG SA2 Meeting #155 	S2-2303091

Athens, Greece, Feburary 20th – 24th, 2023

Source:	Intel

Title:	TS 23.586: Procedures for Service Authorization and Provisioning to UE

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper proposes Procedures for Service Authorization and Provisioning to UE for Ranging based services and sidelink positioning.

# 1. Introduction/Discussion

This paper proposes Procedures for Service Authorization and Provisioning to UE for Ranging based services and sidelink positioning.

# 2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

## 6.2	Procedures for Service Authorization and Provisioning to UE

### 6.2.1	General

The procedures for service authorization and provisioning to UE may be initiated by the PCF (as described in clause 6.2.2), by the UE (as described in clause 6.2.4).

### 6.2.2	PCF based Service Authorization and Provisioning to UE

For PCF based Service Authorization and Provisioning to UE, the Registration procedures as defined in clause 4.2.2.2 of TS 23.502 [5], UE Policy Association Establishment procedure as defined in clause 4.16.11 of TS 23.502 [5] and UE Policy Association Modification procedure as defined in clause 4.16.12 of TS 23.502 [5] apply with the following additions:

-	If the UE indicates Ranging/SL positioning Capability in the Registration Request message and if the UE is authorized to use Ranging/SL positioning service based on subscription data, the AMF selects the PCF which supports Ranging/SL positioning Policy/Parameter provisioning as described in clause 6.2.3 and establishes a UE policy association with the PCF for Ranging/SL positioning Policy/Parameter delivery.

-	If the UE supports one of Ranging/SL positioning capabilities and it does not have valid Ranging/SL positioning policy/parameters, the UE includes the UE Policy Container with indicating the Ranging/SL positioning Policy Provisioning Request during registration procedure. The UE may also include UE capabilities as defined in clause 4.3.1 in the Ranging/SL positioning Policy Provisioning Request and PCF may determine the Ranging/SL positioning Policy/Parameters for the UE based on received UE capabilities.

-	If the UE indicates the Ranging/SL positioning Policy Provisioning Request in the UE Policy Container, the PCF determines whether to provision Ranging/SL positioning Policy/parameters to the UE, as specified in clause 6.1.2.2.2 of TS 23.503 [9], and the PCF provides the Ranging/SL positioning Policy/parameters (see clause 5.2) to the UE by using the procedure as defined in clause 4.2.4.3 "UE Configuration Update procedure for transparent UE Policy Delivery" in TS 23.502 [5].

The PCF may update the Ranging/SL positioning Policy/parameters to the UE in following conditions:

-	UE Mobility, e.g. UE moves from one PLMN to another PLMN. This is achieved by using the procedure of UE Policy Association Modification initiated by the AMF, as defined in clause 4.16.12.1 of TS 23.502 [5].

-	When there is a subscription change in the list of PLMNs where the UE is authorized to perform Ranging/SL positioning services. This is achieved by using UE Policy Association Modification initiated by the PCF procedure as defined in clause 4.16.12.2 of TS 23.502 [5].

-	When the UE determines that the Ranging/SL positioning Policy/parameter(s) is invalid and performs UE triggered Policy Provisioning procedure to the PCF.

If the serving PLMN is removed from the list of PLMNs in the service authorization parameters, the service authorization is revoked in the UE.

When the UE is roaming, the change of subscription resulting in updates of the service authorization parameters are transferred to the UE by H-PCF via V-PCF.

The UE may perform UE triggered Policy Provisioning procedure to the PCF as specified in clause 6.2.4 when the UE determines the Ranging/SL positioning Policy/Parameter is invalid (e.g. Policy/Parameter is outdated, missing or invalid).

When the UE disables a Ranging/SL positioning capability, the PCF may stop updating the corresponding Ranging/SL positioning Policy/parameter(s) and when the UE enables a Ranging/SL positioning capability the PCF may need to provide or update the corresponding Ranging/SL positioning Policy/parameter(s).

### 6.2.3	PCF discovery

PCF discovery and selection mechanism defined in clause 6.3.7.1 of TS 23.501 [4] applies with the following addition to enable a PCF instance is selected for Ranging/SL positioning service and for UE:

-	Based on the indication from the UE and/or UE subscription data during the Registration procedure, the AMF may include the Ranging/SL positioning service indication in the Nnrf_NFDiscovery_Request message as the optional input parameter. If provided, the NRF takes the information into account for discovering the PCF instance.

### 6.2.4	Procedure for UE triggered Policy/parameters provisioning

The UE triggered Policy/parameters provisioning procedure is initiated by the UE to request Ranging/SL positioning Policy/Parameter from the PCF.

Figure 6.2.4-1: UE triggered Ranging/SL positioning Policy/parameters provisioning procedure

1.	The UE sends the UL NAS TRANSPORT message carrying the UE Policy Container (UE Policy Provisioning Request to request Ranging/SL positioning policies) to the AMF.

2.	The AMF sends Namf_Communication_N1MessageNotify request to the PCF including the UE Policy Container received from UE.

3.	The PCF receives UE Policy Container which indicates UE Policy Provisioning Request to request Ranging/SL positioning policies. If the Ranging/SL positioning policies are authorized based on AMF input, the PCF performs the UE Policy delivery procedure as defined in clause 4.2.4.3 of TS 23.502 [5].

# * * * * End of changes * * * *