3GPP TSG-WG SA2 Meeting #155 	S2-2303725

# Athen, 20-24 February 2023	(revision of S2-2302590)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3795 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Group QoS request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, SK Telecom, LG Uplus, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduce the support of creating AF sessions for a list of UEs and -activating Consolidated-MBR Monitoring as concluded in TR 23.700-80. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of QoS request for AF sessions for a list of UEs and Consolidated-MBR Monitoring |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support of this feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.X.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (ALL NEW TEXT)

4.15.6.X.1	Procedures for Creating a Multi-member AF session with required QoS for a list of UEs

Figure 4.15.6.X.1-1: Procedures for creating Multi-member AF sessions with required QoS for a list of UEs

1.	The AF sends a request to reserve resources for AF sessions for a list of UEs `using Nnef_MultiMemberAFsessionWithQoS_Create request message (a list of UE address, AF Identifier, Flow description information or External Application Identifier, QoS Reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), QoS parameter(s) to be measured, Reporting frequency, Target of reporting, optional an indication of local event notification, Consolidated-MBR Threshold, DNN, S-NSSAI) to the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20]. The AF may provide QoS parameter(s) to be measured as defined in clause 5.X of TS 23.501 [2], Reporting frequency, Target of reporting, optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20]. To activate the Consolidated-MBR monitoring, the AF shall include the UL bitrate or DL bitrate as defined in 5.X of TS 23.501 [2] in the QoS parameter(s) to be measured, Aggregated-MBR Threshold, and the Target of reporting set to the NEF.

Editor’s Note: Proper reference to the UPF Event Exposure needs further clarification.

Editor’s Note: Whether and how to leverage direct or indirect UPF event notification is FFS

Editor’s Note: Whether and how to activate the Consolidated-MBR monitoring during the create procedure is FFS

Editor’s Note: The name of the service is FFS.

2.	The NEF authorizes the AF request and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all steps (except step 5) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed. The NEF generates the Transaction Reference ID and associate the list of UE address received from the AF to the Transaction Reference ID.

3-4.The NEF uses the list of UE address to discover the serving PCFs from the BSF(s).

5.  Step 5-9 applies for each UE address in the list of UE addresses. The NEF provides the UE address and the received parameters in step 1 to the PCF in the Npcf_PolicyAuthorization_Create request.

6.  	Step 4 in Figure 4.15.6.6-1 applies. The PCF generates authorized QoS Monitoring policy according to the information provided from the NEF in step 5 and provides PCC rules with the policy to the SMF as described in clause 6.1.3.21 of TS 23.503 [20]. The SMF configures the UPF to perform QoS Monitoring as described in clause 5.X of TS 23.501 [2].

Editor’s Note: Proper reference to the UPF Event Exposure needs further clarification.

7.	If the NEF receives the Npcf_PolicyAuthorization_Create response, the NEF sends a Nnef_MultiMemberAFsessionWithQoS_Create response message (Transaction Reference ID, Result for list of UE address) to the AF. Result for list of UE address includes whether the request is granted or not for every UE address in the list of UE address.

8.  Step 6 in Figure 4.15.6.6-1 applies.

9.  	Step 7 in Figure 4.15.6.6-1 applies.

Editor’s Note: Whether and how to leverage direct or indirect UPF event notification is FFS

10. The NEF sends Nnef_MultiMemberAFsessionWithQoS_Notify message with the aggregated resource allocation status events to the AF.

11. Based on the parameters received in Step 5, the QoS Monitoring events are reported to the NEF.

Editor’s Note: Proper reference to the UPF Event Exposure needs further clarification.

12. The NEF sends Nnef_MultiMemberAFsessionWithQoS_Notify message with the aggregated QoS Monitoring events to the AF. If the NEF received Consolidated-MBR Threshold, the NEF aggregates the bitrate received from Step 10 and if the aggregated bit rate exceeds the Consolidated-MBR Threshold, the NEF sends the event report for Consolidated-MBR Monitoring which includes the aggregated QoS Monitoring event for bitrates and the indication whether the aggregated bitrate is exceeded or not. If the Reporting Frequency received from the AF is periodic, the NEF periodically sends the event report for Consolidated-MBR Monitoring to the AF.

The AF may send Nnef_MultiMemberAFsessionWithQoS_Revoke request containing the Transaction Reference ID to the NEF in order to revoke all the AF sessions related to the Transaction Reference ID. The NEF authorizes the revoke request and triggers the Npcf_PolicyAuthorization_Delete and the Npcf_PolicyAuthorization_Unsubscribe operations for every UE in the list of UE address associated with the Transaction Reference ID.

END of CHANGES