3GPP TSG-SA2 Meeting #155	S2-2302356

# Athens, Greece February 20 - 24, 2023										(revision of S2-2300148)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3672 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.502 Solution for new service to support QoS Resource Allocation and QoS Monitoring operation for group of AF Sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Oracle, Toyota, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to enable 5GC to assist the Application AI/ML operation traffic to comply to the SLA between the MNO and ASP on the aggregated traffic volume during the group data transmissions, it is essential to optimize and to minimize signaling overhead while allowing MNO to be aware/alerted of/to any crossing of the aggregated throughput threshold which is defined by Group-MBR threshold. Furthermore, it is important to recognize the short lifecycle of the group transmission of the Application AI/ML operation traffic, and that in case of the FL, the AF selected member UEs who participate in the group data transmission could be different and are not static during the entire specific FL operation. The Group-MBR threshold monitoring operation as described by this CR is initiated by the AF towards 5GC to keep track of the overall aggregated group data transmission volume via the support by NEF to perform the Group-MBR monitoring. The request of the Group-MBR monitoring is converted into individual UPF QoS Monitoring event for Data Rate monitoring for the set of selected QoS flows. When the NEF receives the set of the Data Rate monitoring report, it will then aggregate the reported bit rates and compare the aggregated bit rate against the Group-MBR threshold specified by the AF. If the threshold is exceeded by aggregated bit rate, the NEF will then notify the AF for the outcome. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduces a new service Nnef_AFGroupOfSessionsWithQoS to enable resource allocation for a group of AF sessions and to support the control of the Group QoS monitoring operation including the Group-MBR monitoring operation for a set of QoS flows of a selected group of UEs’ PDU Sessions. - Describes the procedures on the 4 operations (i.e. create, update, revoke and notify) of the new service - Describes how the new service to be converted by the NEF to be partitioned into individual UPF QoS Monitoring event for Data Rate Monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide a reliable and optimal mechanism for the group based QoS flows monitoring to address the operator’s requirement to monitor the Application AI/ML traffic volume to determine the impacts towards non-Application AI/ML traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.x, 4.15.3.2.x.1, 5.2.6.1, 5.2.6.X.1, 5.2.6.X.2, 5.2.6.X.3, 5.2.6.X.4, 5.2.6.X.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y | X | Other core specifications |  |  |  | TS/TR 23.503. CR 0903 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * (All New)

##### 4.15.3.2.X	Information flow for Group of AF Sessions with required QoS resource allocations and group QoS monitoring procedures

This clause describes the NEF service to support the network resource allocation as well as various QoS monitoring operation for a group of AF sessions which have been set up for a group of UEs selected by the AF.     Each AF session within the group, if authorized by the PCF, is allocated with the same set of QoS resources as requested by the AF.   If the QoS monitoring is also requested for the group, the group of QoS flows for the corresponding UEs within the group which participate in the same application is to be monitored as requested by the AF.  One or more QoS parameters to be measured are specified by the AF to imply the type of QoS monitoring measurement (e.g. delay, jitter and/or data rate etc.).

This NEF service coordinates the AF requests for the group of AF sessions with the respective 5GC NFs’ responses and notifications between the AF and the 5GC.  More specifically, the NEF verifies the authenticity of the AF’s request and partitions the AF’s request for the group of AF sessions into individual N5 request as well as to identify the proper serving PCF of the UE for the AF session within the group to process the N5 request. The UE’s serving PCF further authorizes the N5 request before proceeding to derive the PCC rules for the target QoS flow as indicated in the AF’s request.  The UE’s serving PCF performs the PCC rule binding with the target QoS flow requested by the AF and, through the support of the UE’s serving SMF, the PCF updates the N4 policy corresponding to the QoS resources and the QoS monitoring configurations indicated to the UE’s serving UPF.

When the QoS Monitoring for Data Rate for the group of QoS flows is requested by the AF for the group of AF sessions, the AF may also request the monitoring of a group of QoS flows against a specified maximum aggregated throughput which is the Group-MBR threshold and is included in the AF request.   If the Group-MBR monitoring is requested, the group of the QoS Monitoring for Data Rate reports will be sent to the AF only if the aggregated bit rate exceed the Group-MBR threshold.  Otherwise, the group of QoS Monitoring for Data Rate report will be sent to the AF according to the reporting frequency.

The following describes the mapping of the 4 types of NEF service requests of the Group AF sessions and the N5 service request.

| Nnef_GroupOfAFSessionWithQoS Request | N5 request |
| --- | --- |
| Create | Requesting to allocate a network resource for a group of QoS flows corresponding to a group of AF sessions associated with a group of selected UEs. In addition, the QoS monitoring for a group of QoS flows can be initiated as well |
| Update | Updating (a) the list of UEs and their respective QoS flows of the corresponding AF Sessions in the group, (b) the QoS resource allocation and/or (c) the type of QoS monitoring for the group of QoS flows. More than one of the above operations can be requested at the same time. |
| Revoke | Deleting the establishment of the group or specific AF session within the group. |
| Notify | Notifying AF with the bulk QoS monitoring report for all the AF sessions within the group |

The Group Maximum Bit Rate (Group-MBR) threshold is described as follows:

-	The Group-MBR threshold provides the upper bound of the aggregate bit rate across all GBR QoS Flows corresponding to the group of PDU Sessions of the UEs who participate in a group of transmissions concurrently (e.g. Federated Learning (FL) operation) with active user planes. The anchor UPF of the group of transmissions (e.g. FL operation) measures the aggregated bit rate across the set of selected QoS Flows against the Group-MBR threshold provided by the associated anchor SMF of the corresponding group of PDU sessions that support the group of transmissions. The Group-MBR is provided by the AF to initiate the Group-MBR monitoring for a set of QoS flows of a group of PDU sessions supporting a group of concurrent transmissions.

In case of the Group QoS monitoring, either Group-MBR monitoring or group reporting for individual QoS flow data rate and not both will be supported at the same time by the NEF.

NOTE-1: If Group-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the bit rate shall be provided.

NOTE-2: When the event report is for Group-MBR monitoring, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Group-MBR threshold is exceeded.

###### 4.15.3.2.X.1  	Procedures for a group of AF sessions with required QoS for resource allocation and/or QoS monitoring support

Figure 4.15.3.2.X-1: Information Flows of a group of AF sessions with required QoS for resource allocation and/or QoS monitoring support

1.	The AF which controls the operation for a group of AF Sessions invokes the Nnef_GroupOfAFSessionWithQoS_Create/Update/Revoke Request with 5G Core via the support of NEF to allocate, update or revoke, respectively, for the resource allocations and/or to direct the QoS monitoring for a group of QoS flows for a selected group of UEs.  The group of QoS flows corresponding to a group of UEs’ PDU sessions which may be selected to serve a group of data transfer for a specific Application (e.g. FL operation).

- If the AF is to set up the resource allocation for an initial group of UEs which may or may not include the request for specific QoS monitoring operation, it invokes the Nnef_GroupOfAFSessionWithQoS_Create request which includes the group of UE addresses, AF Identifier, Flow description information or External Application Identifier, QoS Reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), DNN, S-NSSAI) to the NEF. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. QoS parameter(s) to be measured (Optional). The Group-MBR threshold for Group-MBR monitoring (Optional). Regardless of whether the AF request is formulated using a QoS Reference or individual QoS parameters, the AF may also provide one or more of the following parameters that describe the traffic characteristics flow direction, Burst Arrival Time at UE (uplink) or UPF (downlink).

NOTE-1: If Group-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE-2: When the event report is for Group-MBR monitoring, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Group-MBR threshold is exceeded.

- If the AF is to update the UE list in the group and/or to update either the resource allocation and/or to update the type of QoS monitoring, the AF initiates the Nnef_GroupOfAFSessionWithQoS_Update request which includes the group of UE addresses, the AF Identifier, Transaction Reference ID, [Flow description information], [QoS Reference or individual QoS parameters], [Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20])]) to NEF for updating the reserved resources. The Transaction Reference ID provided in the AF session with required QoS update request message is set to the Transaction Reference ID that was assigned by the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. Regardless of whether the AF request is formulated using a QoS Reference or individual QoS parameters, the AF may also provide one or more of the following parameters that describe the traffic characteristics: flow direction, Burst Arrival Time at UE (uplink) or UPF (downlink). QoS parameter(s) to be measured (Optional). The Group-MBR threshold for Group-MBR monitoring if requested. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as specified in clause 6.1.3.22 of TS 23.503 [20].

NOTE-1: If Group-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE-2: When the event report is for Group-MBR monitoring, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Group-MBR threshold is exceeded.

- If the AF revokes the Group of AF sesssions, it initiates the Nnef_GroupOfAFSessionWithQoS_Revoke request by including the Transaction Reference Id which was assigned by the NEF to the AF for the corresponding target group.

2.&3. NEF authorizes the AF request.  The NEF partitions the AF request which contains the list of UEs into individual UE’s request to BSF.  The NEF determines the UE’s corresponding serving PCF address based on the local configuration or the NEF consumes Nbsf_Management Discovery service operation to find out the address of the UE’s serving PCF.

Editor’s note: Whether to optimize the BSF to support bulk request for a group of UEs is FFS.

4.&5. Once the UE(s)’ serving PCF(s) are identified, the NEF converts the AF’s request into individual N5 request to each UE’s serving PCF.  The UE’s serving PCF authorizes the NEF request and responds to NEF with Npcf_PolicyAuthorization response according to the success or failure of the authorization.

- If the AF requests for Nnef_GroupofAFSession_Create to set up the group of AF sessions with resource allocation for a group of UEs, the NEF requests the individual UE’s serving PCF to derive the required QoS parameters of the PCC rule based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration) and notifies the result to the NEF.  Step 4 as described in clause 4.15.6.6 is referred to further describe how the UE’s serving PCF derives the PCC rules towards the UE’s serving SMF when it receives the QoS reference or individual QoS parameters from the AF via NEF.

If the Nnef_GroupOfAFSession_Create includes also the QoS parameter to be measured and the Group-MBR Threshold,  NEF requests the PCF to initiate the QoS Monitoring event for Data Rate monitoring as specified in clauses 5.2.26 and 5.X.Y2 of TS 23.501 [2] for a specific QoS flow, the NEF invokes PCF with the Npcf_PolicyAuthorization_Create request and includes the UE IP address, DNN and S-NSSAI, Application Identifier, Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20] to be measured, flow direction, the Reporting frequency, Target of reporting (i.e. NEF), DNN, S-NSSAI, Transaction Reference Id, optional Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order).

NOTE-1: If Group-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE-2: When the event report is for Group-MBR monitoring, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Group-MBR threshold is exceeded.

- If the AF requests for Nnef_GroupofAFSession_Revoke to delete the entire Group AF session, the NEF invokes the Npcf_PolicyAuthorization_Delete service to the group of UEs’ respective serving PCFs, NEF notifies each UE’s serving PCF to remove the N4 policy by referring to the AF Transaction Id.  The UE’s serving PCF responds to the NEF according to the authorization status of the NEF request.

- If the AF requests for Nnef_GroupOfAFSessionWithQoS_Update in order to update the list of UEs in the group for the resource allocation, the NEF refers to the local context of the corresponding to the list of UEs in the group and determines for which the new UE is to be added to the group, for which of the existing UE is to be removed from the group and/or for which of the existing UE is to be updated by invoking one or more of the following Npcf_PolicyAuthorization procedures:

- If the group AF session update request involves a new UE is to be added to the group with resource allocation, the NEF initiates the Npcf_PolicyAuthorization_Create as described in A. above towards the PCF for the corresponding UE should be followed.

- If the group AF session update request involves an existing UE is to be removed to the group, the NEF initiates the Npcf_PolicyAuthorization_Delete as described above B. towards the PCF for the corresponding UE should be followed.

- If the group AF session update request involves either the update the QoS resources or QoS monitoring or both for the existing UE in the group, the NEF initiates the Npcf_PolicyAuthorization_Update to the UE’s serving PCF. The PCF processes the Npcf_PolicyAuthorization_Update request similar to the Group AF session create request as described in A. above.

6. 	NEF aggregates the authorization responses from the PCFs and sends the AF with the Nnef_AFGroupOfSessionsWithQoS Create/Update/Revoke response corresponding to the earlier Group AF session request.  The consolidated response for NEF provides specific success or failure response corresponding to each UE.

7.&8. 	If NEF requested Npcf_PolicyAuthorization_Create/Update is authorized by the PCF, and the PCF determines that the SMF needs updated policy information, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with updated policy information about the PDU Session as described in the PCF initiated SM Policy Association Modification procedure in clause 4.16.5.2.

Furthermore, if the QoS parameter to be measured and the Group-MBR monitoring are included in the Nnef_GroupOfAFSession_Update request, the PCF triggers the QoS Monitoring event for the Data Rate Monitoring as specified in clauses 5.2.26 and 5.X.Y2 of TS 23.501 [2] and updates the associated SMF with corresponding policy information as described in clauses 6.1.3.21 and 6.3.1 of TS 23.503 [20] to trigger the QoS Monitoring event for Data Rate Monitoring for the corresponding QoS flow by invoking Npcf_SMPolicyControl_UpdateNotify service operation towards the SMF.  The target of the QoS Monitoring report is NEF.

If NEF requested Npcf_PolicyAuthorization_Delete is authorized by the PCF, the PCF triggers Npcf_SMPolicyControl_Delete service operation towards the SMF.

9.&10.   If the SMF receives the create or the update of the policy information for the PDU session provided by the PCF as described in step 7 above, SMF initiates N4 Session Modification procedure to update the N4 session context of an existing PDU Session at the UPF as described in clause 4.4.1.3.  The updated N4 Session may require the SMF to initiate the requested action to trigger the QoS Monitoring event for Data Rate Monitoring as specified in clauses 5.2.26 and 5.X.Y2 of TS 23.501 [2] for the target QoS flow.

If the SMF receives the deletion of the policy information for the PDU session, the SMF notifies the PSA UPF to terminate the QoS Monitoring event for Data Rate Monitoring for the target QoS flow.

SMF acknowledges the PCF request with a Npcf_SMPolicyControl_UpdateNotify response.

11.  When received the PCF request, the SMF is then to provision the PSA UPF to remove the reporting, or to initiate the reporting of Data Rate Monitoring via Nupf_EventExposure_Notify to the NEF periodically in accordance with the Reporting frequency.

12.  If the QoS Monitoring event for Data Rate Monitoring is indicated by the SMF, the UPF reports the bit rate for the target QoS flow periodically to the NEF; otherwise, if the QoS Monitoring event for Data Rate Monitoring is revoked by the SMF, the UPF terminate the UPF reporting.

13. 	If receiving the Data Rate Monitoring report of the QoS Monitoring event for specific QoS flow from UPFs, and the Group-MBR threshold was provided by the prior Nnef_GroupOfAFSessionWithQoS request,  the NEF sums up the bit rates of all the active QoS flows through the duration of the Reporting Frequency.  If the given Application AI/ML traffic is asymmetric, the bit rate aggregation is done per uplink and downlink separately; otherwise, the bit rate aggregation is done per either uplink or downlink. NEF compares the aggregated bit rate of the uplink or downlink against the Group-MBR threshold for the uplink or downlink, accordingly.

If the Group-MBR threshold was not provided, the NEF collects all the UPF QoS monitoring reports through the duration of the Reporting Frequency.

14.  Only if the aggregate bit rate exceeds the Group-MBR threshold specified by the AF, the NEF will notify the Application AI/ML AF via Nnef_AFGroupOfSessionsWithQoS_Notify for the collected UPF QoS monitoring reports together with the AF Transaction Reference ID; otherwise, the NEF always notify the AF with the collected UPF QoS monitoring reports together with the AF Transaction Reference ID.

15.  AF may apply traffic mediation action in responding to the Group-MBR monitoring report (e.g. requesting 5G Core to apply traffic gating on specific QoS flow such as asking 5GC to block some of the flows) or may decide not to take any action other than using it for its own statistics or charging. The decision is local implementation decision is not within the scope of this procedure.

The UPF QoS Monitoring event for Data Rate Monitoring continues until AF terminates the Group-MBR monitoring. The UPF continues to perform the QoS Monitoring event for Data Rate Monitoring for the given QoS flow to the NEF periodically corresponding to the Reporting Frequency as described in steps 11-12 above.

If AF wants to terminate the Group-MBR monitoring, AF sends the Nnef_AFGroupOfSessionsWithQoS_Revoke request to NEF which includes the AF Transaction Reference ID.

* * * Next Changes * * *

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_GroupOfAFSessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |
| Nnef_PDTQPolicyNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |

* * * Next Changes * * * (All New)

#### 5.2.6.X	Nnef_GroupOfAFSessionWithQoS service

##### 5.2.6.X.1	General

See clause 4.15.X.

In addition to support the QoS resource allocation for a group of UEs, this service is also used to support subscription and notification of QoS Monitoring event for Data Rate Monitoring for a group of QoS flows, as described in clause 5.2.26.

##### 5.2.6.X.2	Nnef_GroupOfAFSessionWithQoS_Create service operation

Service operation name: Nnef_GroupOfAFSessionWithQoS Create

Description: The consumer requests the network to allocate QoS dedicated resources for a specific group of AF sessions and potentially to perform QoS monitoring (e.g. delay, jitter, bit rate or Group-MBR).

Inputs, Required: AF Identifier, a list of UE addresses (i.e. IP addresses), Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: Time Period, Traffic Volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Group-MBR Threshold, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available, flow direction.

NOTE-1: If Group-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE-2: When the event report is for Group-MBR monitoring, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Group-MBR threshold is exceeded.

Outputs, Required: Transaction Reference ID, result for the success or failure of the request corresponding to individual UE(s) in the list.

Output (optional): None.

##### 5.2.6.X.3	Nnef_GroupOfAFSessionWithQoS_Notify service operation

Service operation name: Nnef_GroupOfAFSessionWithQoS_Notify

Description: NEF reports the QoS Flow level event(s) to the consumer corresponding to the group of AF sessions.

Inputs, Required: Transaction Reference ID, Delivery reports of the events as defined in clause 6.1.3.18 of TS 23.503 [20].

NOTE: When the event report is for Group-MBR monitoring, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Group-MBR threshold is exceeded.

Inputs, Optional. When the delivery report is for Group-MBR Monitoring, includes Group-MBR Monitoring report

Outputs, Required: None.

Output (optional): None.

##### 5.2.6.X.4	Nnef_GroupOfAFSessionWithQoS_Revoke service operation

Service operation name: Nnef_GroupOfAFSessionWithQoS_Revoke

Description: The consumer requests the network to revoke the group of AF sessions.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Transaction Reference ID, result for the success or failure of the request corresponding to individual UE.

Output (optional): None.

##### 5.2.6.X.5	Nnef_GroupAFSessionWithQoS_Update service operation

Service operation name: Nnef_GroupOfAFSessionWithQoS_Update

Description: The consumer requests the network to update list of UEs (identified by IP addresses) of the group of AF sessions, the Service Requirement(s) and/or additional Alternative Service Requirement(s) for a group of AF sessions and/or the type of QoS monitoring operation.

Inputs, Required: Transaction Reference ID. AF Identifier, a list of UE addresses (i.e. IP addresses), Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier.

Inputs, Optional: QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Group-MBR Threshold, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], flow direction.

NOTE-1: If Group-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE-2: When the event report is for Group-MBR monitoring, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Group-MBR threshold is exceeded.

Outputs, Required: Transaction Reference ID, result for the success or failure of the operation execution result.

Output (optional): None.

* * * End of Changes * * *