3GPP TSG-WG SA2 Meeting #155 	S2-2302743

February 20 – 24, 2023, Athens, Greece	(revision of S2-220xxxx)

Source:	vivo

Title:	TS 23.586: Network assisted Sidelink Positioning

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper implements the the conclusions of KI#5 Network assisted SL Positioning.

1. Introduction/Discussion

This paper implements the following conclusions of KI#5 Network assisted SL Positioning.

The Normative work will be based on the following principles:

-	No normative work will be done by SA2 in Rel-18 to support inter PLMN Network Assisted SL Positioning i.e., when the Target UE and Located UE are registered in different PLMNs.

-	Roaming is supported when Target UE and Located UE are registered in the same PLMN i.e., each UE is either registered in VPLMN or HPLMN, but both are registered in the same PLMN.

-	Legacy functionality specified in TS 23.273 [11] for location services shall be reused including MO-LR, MT-LR for both regulatory and commercial use, when the Target UE has NAS connection with AMF (directly or indirectly via a ProSe UE-to-Network Relay).

NOTE 1:	When the target UE has pending LCS request e.g., per deferred MT-LR and if there is no direct connection available, the target UE can try to discover U2N Relay (layer-2 or layer-3 with N3IWF) and setup indirect connection for handling the pending LCS request, e.g., location report.

-	The Location Service is triggered via the AMF that serves the Target UE, the request comes either from an application server via the GMLC, a 5G NF, or the Target UE.

-	The Target UE can initiate MO-LR or the LMF can initiate MT-LR based on legacy procedures.

-	Positioning protocol LPP endpoints in this case are the Target UE and the LMF.

NOTE 2:	RAN WGs will determine whether and what enhancements and the subset functionalities of LPP are needed to support Network assisted SL positioning including an MT-LR and MO-LR.

-	Either the Target UE or LMF determines if network assisted SL positioning will be applied. When LMF determines that network assisted SL positioning is used, LMF may trigger the Target UE to perform the discovery of Located UE(s).

-	The Target UE and Located UE(s) perform ranging/SL positioning, as defined in KI#4.

-	The Target UE reports the identity of the Located UE(s) to the LMF.

NOTE 3:	It will be determined during normative phase what UE identity of the Located UE that the Target UE provides to the LMF. Depending on which UE identity, the LMF or the GMLC may need to resolve this by query another NF. The security impact will be coordinated with SA3.

-	Target UE discovers Located UE(s) for network assisted SL positioning. To assist the Target UE the LMF may provide candidate list of Located UE to participate in the network assisted SL positioning for the target UE. The provisioning of candidate list of Located UE assumes that the capability and location information of the Located UE in the list are maintained by 5GC.

-	The LMF uses the location of Located UE(s) together with the ranging/SL positioning measurement data or result reported by Target UE and optionally also by Located UEs to estimate the location of the Target UE. The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same time (per TS 23.273 [11]) to improve the Target UE positioning accuracy.

NOTE 4:	The LMF may need to compensate the time difference of the positioning of Target UE and Located UE with additional information.

NOTE 5:	The Ranging/Sidelink positioning supports the scheduled location time feature.

-	The LMF provides the location estimate to the AMF which forwards it either to the Target UE over NAS or to the GMLC, or to the 5GC NF according to legacy functionality.

In this paper, the network assisted mechanism is considered that the Target UE is able to have NAS connection with AMF, the network is involved in the Located UE positioning based on reported Located UE ID. For the Target UE is not able to have NAS connection with AMF, the parts of the conclusion have no network involved in, even the network is involved, it is only used for the result calculation, it is not suitable to be called network assisted.

Another point is that the Network assisted SL Positioning can be carried out either by the Target UE or LMF, so it would better to use the separate clause from the LMF determined Network assisted SL Positioning and Target UE determined Network assisted SL Positioning perspectives. If to use the MO/MT perspective, there may be multiples case to be described.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

5.X	Network assisted Sidelink Positioning

5.2.1	General

Network assisted Sidelink Positioning is defined as the process to assist the UE, 5GC NFs and Application server for obtaining the Ranging/SL Positioning and Uu positioning results of the Target UE.

Either the UE and LMF determines if network assisted SL positioning will be applied, when to carry out the network assisted SL positioning, the legacy functionality and the MO-LR, MT-LR procedure specified in TS 23.273 [8] for location services are reused as basis.

5.2.2	LMF determined Network assisted Sidelink Positioning

When the LMF determines that network assisted SL positioning is used (e.g., is not able to obtain enough measurement data), the LMF may trigger the Target UE to perform the Located UE(s) discovery if the Target UE has NAS connection with AMF (directly or indirectly via a ProSe UE-to-Network Relay).

The Target UE reports the identity of the Located UE(s) to the LMF for Located UE(s) Uu positioning after the Located UE(s) discovery and selection. To assist the Located UE(s) discovery and selection, the LMF may provide candidate list of Located UE to participate in the network assisted SL positioning for the target UE.

LMF triggers the Ranging/SL Positioning procedure between Target UE and the selected Located UE(s), and uses the location of Located UE(s) together with the ranging/SL positioning measurement data or result reported by Target UE and optionally also by Located UEs to estimate the location of the Target UE.

LMF provides the location estimate to the AMF which forwards it either to the Target UE over NAS or to the GMLC, or to the 5GC NF according to legacy functionality.

The Location Service is triggered via the AMF that serves the Target UE, the Location Service request can come either from an application server via the GMLC, a 5G NF, or the Target UE.

The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same time to improve the Target UE positioning accuracy

5.2.3	Target UE determined Network assisted Sidelink Positioning

The Target UE determined Network assisted Sidelink Positioning requires that the Target UE is able to establish NAS connection with AMF. During the process:

-	Target UE performs the Located UE(s) discovery and selection, and reports the selected Located UE(s) ID to the LMF for located UE(s) positioning.

-	LMF performs the Located UE(s) positioning procedure and uses the location of Located UE(s) together with the ranging/SL positioning measurement data or result reported by Target UE and optionally also by Located UEs to estimate the location of the Target UE.

-	LMF provides the location estimate to the AMF which forwards it either to the Target UE over NAS or to the GMLC, or to the 5GC NF according to legacy functionality.

# * * * * End of changes * * * *