3GPP TSG-SA2 Meeting #155 	S2-2303017

# Athens, Greece, February 20th -24th, 2023	                              (revision of S2-2301460)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0628 | rev | 6 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for Federated Learning among Multiple NWDAFs in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile], Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To address the 3 Editor’s notes left in 6.2C Editor’s Note: Procedure of MTLF registration and discovery with respect to FL is FFS. - Propose the registration and discovery procedure for FL in 6.2C.1. MTLF (FL server or FL client) register s to NRF with its NF profile, so the FL server and FL clients can be selected when FL is needed. Editor’s Note: Definition of FL capability is FFS. - Clarify that the FL capability type can be FL server or FL client. Editor’ Note: Further details on providing the accuracy of FL model process is for FFS. - Propose the exchanging process of model metric among consumer, FL server and FL client. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Federated Learning procedure in 6.2C to remove 3 ENs |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

## 6.2C	Federated Learning among Multiple NWDAFs containing MTLFs

### 6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Federated Learning technique to train an ML model.

### 6.2C.2	Procedures

#### 6.2C.2.1	Registration and Discovery procedure for Federated Learning

Figure 6.2X.2.1-1: Registration and Discovery procedure for Federated Learning

1-3.  NWDAF containing MTLF as Server NWDAF or Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability information including FL capability Type (i.e. FL server or FL client) and Time interval supporting FL as described in clause 5.2.

Editor's note:	Definition of FL capability and whether FL capability includes computation capability is FFS.

4-6  NWDAF containing MTLF determines ML model requires FL based on Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as Server NWDAF, the MTLF first discovers and selects Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, Model filter information as defined in TS 23.288 [5], FL capability Type (i.e. FL server), If Server NWDAF is currently doing a FL for the Analytics ID, Time Period of Interest, Service Area.

Once the Server NWDAF (the requested or the selected one) is determined, it discovers and selects other NWDAF(s) containing MTLF as Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, Data availability by the Client NWDAF, Time Period of Interest, Interoperability Indicator.

Editor’s Note: Additional criteria for FL server to discover FL clients from the NRF (e.g. data availability) is FFS.

7.  Server NWDAF sends Federated Learning preparation request to the Client NWDAF(s) with Interoperability information. In the preparation request, the following parameters may be added: Available data requirement, availability time requirement.

Editor’s Note: Other parameters in the preparation request and the definition of “Available data requirement” and “availability time requirement” are FFS.

FL Server includes initial ML Model Information, a Federated Learning execution flag, FL Correlation ID, guideline information (e.g., maximum response time for FL Client to provide interim local ML model information) in the Federated Learning Preparation message.

8.  Client NWDAF(s) decides whether to join the Federated Learning process based on its availability, computation and communication capability, and Interoperability information.

9.  Client NWDAF(s) sends response to Server NWDAF indicating if it will join the FL procedure.

10.  Server NWDAF conducts selection of Client NWDAF(s).

#### 6.2C.2.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF) sends a subscription request to NWDAF containing MTLF to   retrieve a ML model, using Nnwdaf_MLModelProvision service as defined in TS 23.288 [5], including Analytics ID, model metric (e.g., accuracy, precision, recall, etc), pre-determined status (e.g. requested accuracy level and/or total training time) and/or periodically reporting condition (e.g. training round/time). The NWDAF discovery and selection procedure in clause 6.2C.2.1 is applicable.

NOTE 1: The requested accuracy level can be used to indicate the target accuracy level of the training process, and the FL server NWDAF may stop the training process when the requested accuracy level is achieved during the training process.

1.	Server NWDAF sends Nnwdaf_MLModel_DistributedTraining request (Analytics ID(s), (set of) file address of the initial ML model, ML model reporting time limit) to the selected NWDAF containing MTLF(Client NWDAF) that participates in the Federated learning to perform the local ML model training for Federated Learning, including model metric.

2.	Each Client NWDAF collects its local data by using the current mechanism as described in clause 6.2.

3.	During Federated Learning training procedure, each Client NWDAF containing MTLF further trains the retrieved ML model from the Server NWDAF containing MTLF in step 1, based on its own data. Each Client NWDAF also computes local model metric and reports it to the Server NWDAF.

Client NWDAF containing MTLF sends Nnwdaf_MLModel_DistributedTraining response (Analytics ID(s), Result Indication, (set of) file address of the local ML model, validity period). The validity period indicates the time over which the local trained ML model is valid.

NOTE Y: The validity period sent from the Client NWDAF containing MTLF to the Server NWDAF containing MTLF may be used by the Server NWDAF to determine whether the Client NWDAF may be selected for the next iteration of the FL during the FL training procedure.

Editor's note:	The ML model information exchanged between the Client NWDAF(s) and the Server NWDAF is FFS.

4.	The Server NWDAF containing MTLF aggregates the local ML model  received from NWDAF containing MTLF in  step 3, to generate the global ML model for the given Analytics ID in step 0.  The Server NWDAF may also compute the global model metric.

Editor's note: It is FFS how the global model metric is derived during the procedure for Federated Learning among Multiple NWDAF.

5.  The Server NWDAF containing MTLF determines whether to continue the FL training iteration or reselect the list of client NWDAF(s) containing MTLF based on either the ML model accuracy information sent by NF consumer (NWDAF containing AnLF) or based on local configuration in the Server NWDAF.

6.  [Conditional] The Server NWDAF containing MTLF sends Nnwdaf_MLModel_DistributedTraining request (Analytics ID(s), (set of) file address of the aggregated ML model) to each NWDAF containing MTLF.

7.	[Conditional] The Client NWDAF containing MTLF updates the local ML model based on the aggregated ML model sent by the Server NWDAF containing MTLF in step 6.

8a.	[Optional] Based on the NF consumer (NWDAF containing AnLF) request in step 1, the Server NWDAF containing MTLF updates the training status (e.g., global model metric) to the consumer periodically (e.g. a certain number of training rounds or every 10 min) or dynamically when some pre-determined status is achieved (e.g. the requested accuracy level is achieved or training time expires).

8b.	[Optional]The NF consumer (NWDAF containing AnLF) decides whether the current model can fulfil the requirement, e.g. global model metric is satisfactory for the consumer. The consumer modifies subscription (stops or continues the training process) according to the consumer decision.

NOTE:	The steps 2-8b should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

9.   The Server NWDAF containing MTLF sends Nnwdaf_MLModelProvision_Notify/Nnwdaf_MLModelInfo response to the NF consumer (NWDAF containing AnLF) including Analytics ID(s), (set of) file address of the globally optimal ML model.

* * * Next Changes (all new)* * *

## 7.X	Nnwdaf_MLModel_DistributedTraining service

### 7.X.1	General

Service description: this service enables the consumer to request and get from NWDAF containing MTLF ML Model Information for distributed training of ML model.

NOTE: In this release of the specification, this service is applicable to Federated Learning between NWDAFs containing MTLF.

### 7.X.2	Nnwdaf_MLModel_DistributedTraining_Request service operation

Service operation name: Nnwdaf_MLModel_DistributedTraining_Request

Description: The consumer requests NWDAF ML Model Information required for distributed ML model training.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2, address (e.g. URL or FQDN) of Model file).

Inputs, Optional: ML Model Filter Information to indicate the conditions for which ML model for the analytics is requested, and Target of ML Model Reporting to indicate the object(s) for which ML model is requested (e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs)), ML Model Reporting Information (including e.g. ML Model Target Period), ML model accuracy information

Outputs, Required: Set of the tuple (Analytics ID, Result Indication, address (e.g. URL or FQDN) of Model file).

Outputs, Optional: Validity period.

## END of CHANGES