SA WG2 Meeting #155	S2-2303063

# 20 - 24 February, 2023, Athens, Greece			(revision of S2-230xxxx)

Source:	Nokia, Nokia Shanghai-Bell

Title:	TMGI allocation for MOCN RAN sharing based on configuration

Document for:	Approval

Agenda Item:	9.10.2

Work Item / Release:	5MBS_Ph2 / Rel-18

Abstract: This contribution discusses how to handle of location dependent sessions for MOCN network sharing. Related CR are in S2-2303077 and S2-2303083.

# 1. Introduction

Procedures for Resource sharing across multiple broadcast MBS Sessions during network sharing were discussed in the S2-2300590 thread at the last meeting but postponed.

One of the issues that needs to be considered as part of those procedures is the TMGI allocation if RAN nodes identify related MBS sessions based on a configured mapping of TMGIs.

# 2. Discussion

As part of the key issue 2 conclusions that NG RAN nodes can identify MBS session subject to resource sharing based on a configured mapping between TMGIs. It is thus necessary that TMGIs that are mapped to each other in RAN nodes are allocated for related MBS sessions.

One possibility would be that arbitrary TMGIs are allocated to AFs, and the content provider then informs the operators about the allocated TMGIs in order to allow them to configure a mapping in shared RAN nodes. This is a cumbersome procedure that would need to be performed separately for each MBS service subject to RAN sharing

It has already been agreed that RAN nodes may be configured with ranges of TMGIs (one for each PLMN participating in RAN sharing) that are mapped to each other to avoid such a per-MBS-service configuration. However, such ranges cannot be used unless AFs obtain appropriate TMGIs (that are mapped to each other in RAN nodes) from those ranges.

It is thus suggested to extend TMGI allocation procedures with a TMGI index value that can be provided by the AF to request a particular TMGI from a range of TMGIs for shared services reserved in the MB-SMF.

Proposal: Extend the TMGI allocation and MBS session creation procedure with a TMGI index value that can be provided by the AF to request a particular TMGI from a range of TMGIs for shared services reserved in the MB-SMF.