3GPP TSG-SA2 Meeting #155	S2-2303139

# Athens, Greece, 20 – 24 February 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4185 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access to SNPN via wireline access network by using FN-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Access to SNPN via wireline network has been specified in relevant SA2 specifications. In this case, SNPN is implicitly selected by wired physical connectivity between 5G-RG or FN-RG and W-AGF. Change 1: To support FN-RG (FN-BRG or FN-CRG) with two or more SNPN subscriptions, it is reuqired FN-BRG or FN-CRG to use a separate PEI for each network subscriptinon when it registers to the netowrk. However, FN-RG might not have globally unique PEI. A NOTE is added in Clause 5.30.2.0 to clarify this situation. Change 2: Clause 5.30.2.14 in TS 23.501 specifies Access to SNPN services via wirelin access network by using FN-RG. FN-RG could be FN BRG or FN CRG. #### GLI is used to identify the line connecting the FN BRG to the nextwork. GLI is defined in TS 23.003 and in BBF TR-470. #### GCI is used to identify to identify the line connecting the FN CRG to the network. GCI is defined in TS 23.003 and in CableLabs WR-TR-5WWC-ARCH. For the sake of completeness, the change adds GLI in the sentence to avoid confusion about only GCI is supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two changes are added to further clarify the details about access to SNPN servics via wireline access network |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details about access to SNPN services via wireline acces network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.0, 5.30.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.30.2	Stand-alone non-public networks

#### 5.30.2.0	General

SNPN 5GS deployments are based on:

-	the architecture depicted in clause 4.2.3;

-	the architecture for 5GC with Untrusted non-3GPP access (Figure 4.2.8.2.1-1) for either access to SNPN services via a PLMN (and vice versa) or for direct access to SNPN via non-3GPP access;

-	-	the architecture for 5GC with Trusted non-3GPP access (Figure 4.2.8.2.1-2); and

-	the additional functionality covered in clause 5.30.2.

Alternatively, a Credentials Holder (CH) may authenticate and authorize access to an SNPN separate from the Credentials Holder based on the architecture specified in clause 5.30.2.9. Idle and connected mode mobility is supported as defined in clause 5.30.2.11.

Clauses 5.30.2.1 to 5.30.2.11 specify the common SNPN aspects applicable to both 3GPP and non-3GPP access, except where stated differently.

Aspects specific to Untrusted non-3GPP access for SNPN are specified in clause 5.30.2.12.

Aspects specific to Trusted non-3GPP access for SNPN are specified in clause 5.30.2.13.

The following 5GS features and functionalities are not supported for SNPNs:

-	Interworking with EPS.

-	Also, emergency services when the UE accesses the SNPN over NWu via a PLMN.

-	Roaming, e.g. roaming between SNPNs. However, it is possible for a UE to access an SNPN with credentials from a CH as described in clause 5.30.2.9 and to move between equivalent SNPNs.

-	Handover between SNPN and PLMN or PNI NPN.

-	CIoT 5GS optimizations.

-	CAG.

-	Proximity based Services (ProSe) as defined in TS 23.304 [128].

-	5G NSWO.

A UE with two or more network subscriptions, where one or more network subscriptions may be for a subscribed SNPN, can apply procedures specified for Multi-USIM UEs as described in clause 5.38. The UE shall use a separate PEI for each network subscription when it registers to the network.

NOTE 1:	The number of preconfigured PEIs for a UE is limited. If the number of network subscriptions for a UE is greater than the preconfigured number of PEIs, the number of network subscriptions that can be registered with the network simultaneously is restricted by the number of pre-configured number of PEIs.

NOTE 2:	For access to SNPN services via wireline access network, W-AGF might need to be involved to provide qualified PEIs for FN-RG with multiple SNPN subscriptions. Further details on PEIs provided by W-AGF for FN-RG are defined in Clause 4.7.7 of TS 23.316 [84].

# * * * * Next change * * * *

#### 5.30.2.14	Access to SNPN services via wireline access network

Access to SNPN services via a wireline access network follows the same procedures used for accessing a PLMN via a wireline access network as defined in clause 4.2.1 of TS 23.316 [84]. The SNPN is implicitly selected by wired physical connectivity between 5G-RG or FN-RG and W-AGF. The Global Cable Identifier (GCI) and the Global Line Identifier (GLI) used by W-AGF for providing access to SNPN services is specified in Clause 28.15 of TS 23.003 and Clause 28.16 of TS 23.003 [19], respectively.

# * * * * End of changes * * * *