SA WG2 Meeting #S2-156E	S2-2303944

17 - 21 April, 2023, Electronic

3GPP TSG RAN WG1 #112			R1-2302074

Athens, Greece, February 27 – March 3, 2023

Title:	LS reply on the requirement for LPHAP

Response to:	S2-2303414

Release:	Release 18

Work Item:	NR_pos_enh2, 5G_eLCS_Ph3

Source:	RAN1

To:	SA2

Cc:

Contact Person:

#### Name:	Jinhuan Xia

#### Tel. Number:

#### E-mail Address:	Jinhuan.xia@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1 thanks SA2 regarding the question about LPHAP. Some relevant agreements reached in RAN1 include:

Agreement

Confirm that use case 6 defined in TS 22.104 is the single representative use case for the study of LPHAP.

Agreement

- In Rel-18 low power and high accuracy positioning, adopt the following requirement:

  - Horizontal positioning accuracy < 1 m for 90% of UEs

  - Positioning interval / duty cycle of 15-30 s

  - UE battery life of 6 months – 1 year

- Note: Setting an exact value each from the set of positioning interval / duty cycle and UE battery life in the evaluation and identification of performance gap will be discussed separately when necessary.

In addition, the WID (RP-223549) for Rel-18 WI ‘Expanded and Improved NR Positioning’ regarding LPHAP clearly states to enable LPHAP use-case 6 as defined in TS 22.104:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

Therefore, from RAN1 perspective, considering only ‘low power’ or only ‘high accuracy positioning’ is OUT of the release 18 RAN working scope.

# 2	Actions

To SA WG2

ACTION: 	RAN1 respectively requests SA2 to take RAN1 response into account in the normative work.

# 3	Dates of next RAN WG1 meetings

RAN1#112-bis-e	               17 – 26 April, 2023	                                       E-meeting

RAN1#113	               22 – 26 May, 2023	                                       Incheon, Korea