SA WG2 Meeting #S2-156E	S2-2303968

17 - 21 April, 2023, Electronic

3GPP TSG-SA3 Meeting #110 	S3-231387

Athens, Greece, 20 - 24 February 2023

Title:	LS reply to TSG SA on LS 5G-ACIA-LS-2022-005 on 5G capabilities exposure for factories of the future – identified gaps from 5G ACIA

Response to:	LS 5G-ACIA-LS-2022-005 on 5G capabilities exposure for factories of the future – identified gaps from 5G ACIA

Release:	not specified

Work Item:

Source:	TSG SA3

To:	TSG SA

Cc:	TSG SA WG1, 3GPP TSG SA WG2, 3GPP TSG SA WG5, 3GPP TSG SA WG6, TSG CT

Contact person:	Markus Staufer

markus.staufer@nokia.com

+49 151 5515 3469

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA3 would like to thank 5G-ACIA for their LS 5G-ACIA-LS-2022-005 on "5G capabilities exposure for factories of the future – identified gaps" and would like to thank WG SA for their effort to coordinate the response to 5G ACIA.

SA3 has discussed the LS from 5G ACIA with a focus on Gaps G.1, G.3 and limitation L.6, which have been identified as being potentially relevant for SA3. The following conclusions have been agreed:

G.1 Provisioning of subscriptions

SA3 did not standardize or does not have plans to standardize a mechanism to provision information related to the identity of a subscription (i.e., UE Ids or authentication keys) into the 5G core.

For the sake of clarity SA3 also points out that SA3 did not define and does not have plans to define a protocol for provisioning of subscription data to a UE.

G.3 Provision of security event/logging information

SA3 interprets the terms event and logging information in such way, that it refers to events and logging information originating from the 5G core, but not to events and logging information originating from the UE.

According to the understanding of SA3 security events and logging information include for instance information relating to successful and unsuccessful (primary, secondary, slice specific) authentications of UE and information relating to applied protection of user plane traffic.

SA3 did not and does not have plans to standardize exposure of these events or logging information. SA3 proposes to take responses from other working groups (e.g., SA2, SA5) into account, when assessing this gap.

Based on requirements from SA1 or proposals from member companies, SA3 can work on adding these events following the 3GPP SA3 working procedures.

Due to the sensitive nature of the information to be exposed, SA3 suggests being consulted, in case other working groups plan to standardize the exposure of the requested information.

L.6 Fine-grained authorization

SA3 points out that the authorization of Application Functions (AF) at the exposure point can be executed using the OAuth 2.0 procedures. The currently defined scopes allow authorization on the level of services.   Vendors might define and make available additional scopes in their specific products.

In addition to authorization based on OAuth 2.0, the exposure points might limit the scope of API calls in a fine granular way based on implementation specific policy checks as indicated in Clause 12.2 of TS 33.501.

# 2	Actions

To TSG SA

ACTION: 	SA3 kindly request SA to take above information into account, especially when preparing a response to 5G ACIA.

# 3	Dates of next TSG SA WG 3 meetings

SA3#110Bis-e	17 - 21 April 2023	Electronic meeting

SA3#111	22 - 26 May 2023		Berlin