3GPP TSG SA WG2 Meeting #156E	S2-2304012

# April 17-21, 2023																(Revision of S2-230MMMM)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1282 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to DCMF services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR replaces DCMF with a Unified Media Function. This unified media function includes DCMF service support, but can include other services such as ARMF. This enables resue of services that are common across different media fuctions but also caters to specifics that are applicable for individual media functions. During discovery or through configuration the consumer would know what media fuctions are supported in a unified media function. Additionally, this CR includes clarifications for the supported services for DCMF as well as missing information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace DCMF with unified media function. Include clarifications, and missing information, to these services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.1.1, AA.1.2, AA.1.3, AA.2.5, AA.2.5.1, AA.2.5.2.2, AA.2.5.2.3, AA.2.5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## AA.1.1	Architectural Support

Figure AA.1.1-1 shows the architecture to support SBA interactions between IMS entities.

Figure AA.1.1-1: System Architecture to support SBA in IMS

NOTE  1:	The ASF and MRF supports NF registration and discovery, but they do not provide services in this release of the specification.

Editor's note:	Whether the AS in Figure AA.1.1-1 needs to be changed to IMS AS is FFS.

Figure AA.1.1-2 shows the architecture using the reference point representation.

Figure AA.1.1-2: System Architecture to support SBA in IMS in reference point representation

NOTE 2:	In this Release of the specification the SBI capable IMS AS supporting data channel services is collocated with the TAS.

**** Next Change ****

## AA.1.2	Reference point to support SBA in IMS

Following reference points are realized by service-based interfaces in IMS:

N5:	Reference point between the PCF and an AF.

NOTE:	P-CSCF acts as an AF from PCF point of view. N5 Reference point is defined in TS 23.501 [93].

N70:	Reference point between an SBI capable I/S-CSCF and an SBI capable HSS.

N71:	Reference point between an SBI capable IMS AS and an SBI capable HSS.

DC1:	Reference point between an SBI capable IMS AS and ASF.

DC2:	Reference point between an SBI capable IMS AS and MF.

N72:	Reference point between ASF and an SBI capable HSS.

**** Next Change ****

## AA.1.3	Service based interface to support SBA in IMS

Npcf:	Service-based interface exhibited by PCF.

Nhss:	Service-based interface exhibited by an SBI capable HSS.

These SBI services provide equivalent functionality to the Diameter Rx and Cx/Sh reference points.

Nimsas:	Service-based interface exhibited by an SBI capable IMS AS.

Nmf:	Service-based interface exhibited by MF.

These SBI services provide functionality to support data channel management in IMS network.

To support co-existence of IMS nodes supporting SBA services and IMS nodes not supporting SBA services SBI enabled IMS nodes may support both SBI and non-SBI interfaces.

**** Next Change ****

## AA.2.5		MF Services

### AA.2.5.1		General

The following table illustrates the MF Services and Service Operations.

Table AA.2.X.1-1: NF services provided by the MF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nmf_MediaResourceManagement (MRM) | Create | Request/Response | IMS AS |
|  | Update | Request/Response | IMS AS |
|  | Delete | Request/Response | IMS AS |

**** Next Change ****

### AA.2.5.2			Nmf_MediaResourceManagement (MRM) service

#### AA.2.5.2.1	General

Service description: This service enables the consumer to create, update and delete media resources related to IMS Data Channel.

Editor’s note: 	Whether this service could be reused by ARMF is FFS.

The media resource represents a media context including one or multiple media terminations. A media termination includes media resources for one or multiple media streams representing a remote end point on the Mb interface.

Each media resource context is identified by a Media Resource Context ID.

Each media termination is identified by a Termination ID.

Each media stream is identified by a Media ID.

NOTE  1:	Termination is a concept widely used in media protocols like H.248. When a media stream, e.g. application data channel, pass through the DCMF, there is one termination for the input stream and one termination for the output stream. In this case, the two terminations can use the same Media ID.

Figure AA.2.5.2.1-1illustrates the above

Figure AA.2.5.2.1 : Media Context Resource View Example

**** Next Change ****

#### AA.2.5.2.2	Nmf_MRM_Create service operation

Service operation name: Nmf_MRM_Create

Description: The consumer NF requests the MF to create a media context including one or multiple media terminations and reserve media resources for anchoring one or multiple media streams of Mb interface in each termination. The consumer NF may also include media application function specification information requested by the application layer, e.g. ASF, to be applied on the media stream by the MF.

Inputs, Required:  List of Media Termination Descriptors.

Each Media Termination Descriptor of the list includes:

-	Command: Termination resource command to be applied. The command must be set to value ‘Add’ within a Mmf_MRM_Create operation.

-	Termination ID: A unique identity of the termination within the media context resource. The identity is allocated by the producer and must be set to value ‘null’ by the consumer when adding a termination to a context.

-	List of Media Stream Descriptors belonging to the Termination.

Each Media Stream Descriptor of the list includes:

-	Media ID: A unique identity of the media stream within the media context instance. The MediaID value is set by the consumer.

NOTE  1:	Different Termination IDs associated with the same Media ID represent the media stream flow for data forwarding.

-	 Remote Mb Specification: Media Specification specifying SDP parameters of a remote media endpoint e.g. media stream IP address and port allocated at  the remote UE.

NOTE  2:	In the case of reserving terimination media resource before receiving media negotiation information from the UE, Remote Mb Specification should be set to Null.

-	 Media Application Function Input Specification: Specification of the media application function to be applied to the media stream as requested by the application layer, which includes:

-		Media Application Function Type: The value is set based on the type of the media application function required, e.g. “DCMF”.

-	Media Application Function Description: Application specific media function  description specifying how the media function shall handle and process the media stream. Data content depends on type of media application function.
If the Media Application Function Type is “DCMF”, then the Media Application Function Description includes:

-		Media proxy configuration (HTTP or UDP) applicable to the media flow.

-		Remote MDC1/MDC2 media endpoint address.

-		Replacement HTTP URL allocated by the application layer representing the application list (e.g. graphical user interface) offered to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security setp up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

NOTE X: Some of the above attributes are optional depending on the scenario and proxy configuration

Inputs, Optional: None

Outputs, Required: Result indication for the entire list, Media Resource Context ID, List of allocated Media Termination Descriptor Resource information.  There is an entry correpsoding to every  entry in the list of media stream descriptor. Each entry includes:

- Media Resource Context ID uniquely identifies the media resources created by the MF. This identity shall be used as reference when updating or deleting the created media resource.

- List of allocated Media Termination Descriptor Resource information includes one or multiple Media Termination Descriptor Resources allocated. The list will include same number of terminations, in the same order as provided in the request. Each

Media Termination Descriptor Resource includes:

-	Termination ID: Unique identity allocated by the provider. The identity shall be used when updating/deleting the Media Termination

-	Command: Termination resource command applied. Will always be set to value ‘Add’ in responses to Nmf_MRB_Create operations.

-	List of Media Stream Descriptor Resource information elements allocated within the media termination. The list will include same number of media streams, in the same order as provided in the request.

Each Media Stream Descriptor Resource information includes:

-	Media ID: Unique identity of the media stream within the media context instance allocated by the consumer This is identical to the Media ID used in the input.

-		Local Mb Specification: Media specification specifying SDP parameters representing the local Mb media endpoint of the provider e.g. media stream IP address and port.

-	Media Application Function Output Description: Optional media application function description including information required by the application layer. The Media Application Function Output Description shall be relayed to the ASF by the consumer NF. Data content depends on type of media application function.  If the Media Application Function Type is “DCMF”, the Media Application Function Output Description includes:

-			DCMF MDC1/MDC2 media point information (IP address/port number, etc.) which shall be relayed to the ASF by the consumer NF.

Outputs, Optional: None.

**** Next Change ****

#### AA.2.5.2.3	Nmf_MRM_Update service operation

Service operation name: Nmf_MRM_Update

Description: The consumer NF requests the MF to update one or multiple media resources within a specific media resource context. Terminations and/or Media Streams can be added, modified, or deleted within an Update request.

Inputs, Required: Media Resource Context ID, List of Media Termination Descriptors to be updated.

Media Resource Context ID specifies the media context resource to be updated.

List of Media Termination Descriptors includes one of multiple media terminations impacted by the update request.

Each Media Termination Descriptor of the list includes:

-	Command: Termination resource command to be applied. Available commands are: ‘Add’, ‘Modify’, ‘Delete’.

-	Termination ID: A unique identity of the termination within the media context resource. The identity is allocated by the consumer and must be set to value ‘null’ when adding a termination to a context.

-	List of Media Stream Descriptors belonging to the Termination. The complete set of Media Stream Descriptors of a Media Termination must be specified in case a Media Termination is either added or modified. The consumer NF can remove media streams from a Media Termination by omitting the media stream from the list. The details of this list is identical to the input information in the Nmf_MRM_Create service.

Inputs, Optional: None.

Outputs, Required: Result indication, List of Media Termination Descriptor Resource information.

List of updated Media Termination Descriptor Resource information includes one or multiple Media Termination Descriptor Resources allocated. The list will include same number of terminations, in the same order as provided in the request.

Each Media Termination Descriptor Resource information includes:

-	Termination ID: A unique identity of the termination within the media context resource. The identity is allocated by the consumer.

-	Command: Termination resource command applied. Copied from the request.

-	List of Media Stream Descriptor Resource information elements allocated/updated within the media termination. The list will include same number of media streams, in the same order as provided in the request. The list will be set to ‘null’ when deleting a Media Termination. The details of this list is identical to the output information in the Nmf_MRM_Create service.

Outputs, Optional: None.

**** Next Change ****

#### AA.2.5.2.4	Nmf_MRM_Delete service operation

Service operation name: Nmf_MRM_Delete

Description: The consumer NF requests the MF to delete a specific media context resource including all existing terminations and media streams.

Inputs, Required: Media Context Resource ID.

Media Context Resource ID specifies the media context resource to be deleted.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

**** End of Changes ****