3GPP TSG-SA2 Meeting # 156E (e-meeting)	S2-2304016

Elbonia, April 17 - 21, 2023

Source: 	Ericsson

Title:	Dynamically Changing AM Policy for LBO roaming

Document for:	Discussion/Agreement

Agenda Item:	9.33

Work Item / Release:	TEI18_DCAMP_Ph2

Abstract of the contribution: This paper makes proposals how to support dynamically changing AM policy for inbound roamers in LBO case.

# 1 Discussion

## 1.1 Mini-WID TEI18_DCAMP_ph2

TEI18_DCAMP_Ph2 WID is agreed in SP-220799 with the objective to support dynamic AM policy for roaming UEs using LBO traffic routing (see below):

4 Objective

The objective is to enhance the dynamically changing AM policies, as follows:

Objective 1: To define the mechanism of supporting DCAMP in roaming scenario. This objective will focus on providing dynamic AM policy for any roaming UEs in LBO roaming scenario.

## 1.2 Discussion and Proposals

### 1.2.1 DCAMP in Rel-17 (only for non-roaming scenario)

In WID TEI17_DCAMP, the function of dynamically changing AM Policy (i.e., RFSP index and Service Area Restriction) is introduced for non-roaming users. Below are some excerpts from TS 23.502 v18.1.0 and observations:

4.15.6.9	Procedures for AF-triggered dynamically changing access and mobility management policies

4.15.6.9.1	General

Access and mobility management policies may be modified due to several inputs including the AF as described in clause 4.16.2. Clause 4.15.6.9 describes the procedures for triggering such modifications in scenarios belonging to "case B" of clause 4.16.2.0 that are initiated by the AF.

The following cases can be distinguished:

-	AF requests targeting an individual UE (identified by its SUPI or GPSI) without conditions related to the application traffic; these requests are routed (by the AF or by the NEF) to the PCF for the UE as described in clause 5.1.1.2 in TS 23.503 [20] This case is described in clause 4.15.6.9.2.

-	AF requests targeting an individual UE (identified by its GPSI), a group of UEs (identified by an Internal Group Identifier or an External Group Identifier), any UE accessing a combination of DNN and S-NSSAI, or all UEs using an application identified by an External Application Identifier. This case is described in clause 4.15.6.9.3. For such requests the AF shall contact the NEF and the NEF stores the AF request information in the UDR. The PCF(s) receive a corresponding notification if they had subscribed to the creation / modification / deletion of the AF request information corresponding to UDR Data Keys / Data Sub-Keys. This is further described in clause 4.15.6.9.3. The AF is not aware if the target UEs are with or without an already established AM Policy Association and with or without ongoing PDU Sessions.

4.15.6.9.2	Processing AF requests to influence access and mobility management policies targeting an individual UE

…

This procedure concerns only non-roaming scenarios, i.e. to cases where the involved entities serving the UE (AF, PCF, BSF, AMF) belong to the home PLMN.

4.15.6.9.3	Processing AF requests to influence access and mobility management policies

…

This procedure concerns only non-roaming scenarios, i.e. to cases where the involved entities serving the UE (i.e. AF, NEF, PCF, BSF, UDR, AMF) belong to the home PLMN, or the AF belongs to a third party with which the home PLMN has an agreement.

[Observation-1] Per clause 4.15.6.9.2 of TS 23.502, in non-roaming scenario, for an individual UE, an AF can request to influence access and mobility management policies.

[Observation-2] Per clause 4.15.6.9.3 of TS 23.502, in non-roaming scenario, an AF can provision/influence the access and mobility management policies in UDR via NEF. The target may be an individual UE, a group of UEs, or any UEs of non-roaming users.

### 1.2.2 Handling service specific parameters for inbound roamers

In SA2#155, the handling of URSP determination for inbound roamers is added in TS 23.502 CR3712 (S2-2303792) (agreed), see below from TS 23.502 v18.1.0:

4.15.6.7	Service specific parameter provisioning

…

The AF request sent to the NEF contains the information as below:

1)-	Service Description.

Service Description is the information to identify a service the Service Parameters are applied to. The Service Description in the AF request can be represented by the combination of DNN and S-NSSAI, an AF-Service-Identifier or an External Application Identifier.

2)	Service Parameters.

Service Parameters are the service specific information which needs to be provisioned in the Network and delivered to the UE in order to support the service identified by the Service Description.

VPLMN ID(s) that indicates the PLMN(s) where the AF guidance on URSP determination, and all its RSD(s), applies.

3)	Target UE(s) or a group of UEs or PLMN ID(s) of inbound roamers.

Target UE(s) or a group of UEs or PLMN ID(s) of inbound roamers indicate the UE(s) who the Service Parameters shall be delivered to. Individual UEs can be identified by GPSI, or an IP address/Prefix or a MAC address. Groups of UEs are identified by an External Group Identifiers as defined in TS 23.682 [23]. If identifiers of target UE(s) or a group of UEs or PLMN ID(s) of inbound roamers are not provided, then the Service Parameters shall be delivered to any UEs of the PLMN of the NEF using the service identified by the Service Description.

[Observation-3] The UEs of the inbound roamers can be identified by the home PLMN ID(s) of the inbound roamers.

### 1.2.3 DCAMP for LBO roaming and proposals

The LBO roaming architecture is illustrated as follows:

Figure 1 LBO roaming architecture

As illustrated in Figure 1, for the inbound roamers,

the subscription data is available in UDM in their HPLMN(s) but not available in VPLMN.

the policy data for a given UE is available in UDR in its HPLMN but not available in VPLMN, i.e., the V-PCF does not have per UE policy for the inbound roamers.

The above would mean, if the AF provide external UE Id or a group of UEs, the HPLMN (UDM) will be required to configure/maintain the translation from external UE identifier to internal UE identifier, or to configure the group of UEs that are roaming, which can be very challenging if not impossible.

To minimize the system impact in supporting dynamically changing AM policy for the inbound roamers using LBO, it is proposed not to target an individual UE or a group of UEs of inbound roamers, but to target any UE of the inbound roamers from the same PLMN or from any PLMNs, which is also aligned with the handling of Service Parameters for inbound roamers in eUEPO (see [Observation-3]).

[Proposal-1] It is proposed to agree on a principle that the dynamically changing AM policy can be applied to any UE of the inbound roamers identified by their home PLMN ID(s) but cannot be applied to an individual UE or a group of UEs of inbound roamers.

Based on the principle in [Proposal-1], the following are proposed:

[Proposal-2] It is proposed to support the AF requests to influence AM policies for any UE of the inbound roamers identified by PLMN ID(s).

# 2 Proposal

[Proposal-1] It is proposed to agree on a principle that the dynamically changing AM policy can be applied to any UE of the inbound roamers identified by their home PLMN ID(s) but cannot be applied to an individual UE or a group of UEs of inbound roamers.

Based on the principle in [Proposal-1], the following are proposed:

[Proposal-2] It is proposed to support the AF requests to influence AM policies for any UE of the inbound roamers identified by PLMN ID(s).

See proposed update in S2-2304017 (TS 23.501CR), S2-2304019 (TS 23.502CR) and S2-2304018 (TS 23.503CR).