3GPP TSG-SA2 Meeting #156E	  S2-2304037

# Elbonia, April 17 – 21, 2023										   						(revision of S2-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4192 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | High level feature description for AIMLsys |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The high-level description of the features described in 23.501 are focused to provide the enablers for the feature to work, and the enablers as such are defined somewhere else, including other TSs. The list of enablers is incomplete, as the Editor´s Note shows that there may be other enablers needed: Editor’s note: It is FFS with additional clauses to summarize the other respective 5GC assistance functionalities for the AI/ML operations in the application layer and the references to the relevant normative clauses once the normative work becomes stable. The respective 5GC assistance functionalities are: - Network resource performance and monitoring - Information exposure to authorized 3rd party - External parameter provisioning - Planned group data transfer support with QoS for application traffic - QoS and Policy requirements - Member Selection Assistance functionality - Network data analytics support |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rather than an overview, the clause describes enablers to support certain AIML operations. These AIML operations are listed, as stage 1 requirements are evolving, a stable design base is required. The following enablers are listed, and references to the clause that describe them is added. - AF request for a list of UEs that fulfil certain conditions. - AF request to provide an estimation of the time window to transfer data with QoS requirements to a list of UEs. - AF request to provide QoS for a list of UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | General archirecture principles are not captured in description of the item. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.46.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 5.46.1	General

This clause describes the list of 5GC enablers to support the following AI/ML operations in the Application layer: Distributed/Federated Learning, Model sharing and Model operation splitting

NOTE:	Requirements on AI/ML operations are specified in clause 6.40 of TS 22.261 [2].

The AF that aims to provide an AI/ML operation e.g. Distributed/Federated Learning, may request assistance from the 5GC as described in clause 5.46.2 by providing a list of UE, then subscribing to the NEF to be notified about the subset list of UEs that fulfil certain filtering criteria. Details of the procedures are described in clause 4.15.13 of TS 23.502 [3]. This subset list of UEs may become the list of candidate UEs for this AI/ML operation depending on the AF internal policies. Alternatively, the AF may select the list of candidate UEs for the AI/ML operation e.g. Distributed/Federated Learning, without NEF involvement as described in Annex I of TS 23.502 [3].

The AF that selected a list of candidate UEs, e.g. for an AI/ML operation of Distributed/Federated Learning, may request the network to provide the preferred time window for the AI/ML operation using the Planned Data Transfer with QoS requirements described in TS 23.503 [45].

At the time the AI/ML Operation. e.g. Distributed/Federated Learning, starts, the AF discovers a suitable NEF and requests the NEF to provide QoS for the list of candidate UEs, each UE identified by its UE IP address, that were selected as described in clause 4.15.13 of TS 23.502 [3]. The AF may subscribe to QoS Monitoring for those AF requests for QoS that result in a successful resource allocation. The AF provides QoS parameters that are derived from the performance requirements listed in clause 7.10 of TS 22.261 [2].

As result of the subscription to NEF to provide the subset list of UEs that fulfil certain filtering criteria, the AF may be notified about changes in the subset list of UEs, as such the AF may request a new preferred time window for the AI/ML operation using the Planned Data Transfer with QoS. The AF may request the NEF to provide QoS for the updated list of candidate UEs, each identified by UE IP addresses, that results in QoS resources previously allocated to some UE IP addresses may be released, while other UE IP addresses may require QoS for resource allocation and QoS monitoring.

The AF that aims to perform an application AI/ML operation may request assistance from the 5GC by subscribing to monitoring events from the NEF. For example, for Model Sharing operation, the AF may subscribe to NEF to be notified on the traffic volume shared between the UE and the AI/ML application server as described in clause 4.15.3.2.3 in TS 23.502 [3]. This may help the AI/ML application server to determine, e.g., how large the model should be, or whether it should select large models less frequently than small models, etc. In another example, for non-real time Distributed/Federated Learning operation the AF may subscribe to session inactivity time monitoring event e.g. to assist the AI/ML application server in scheduling UE(s) participating in Distributed/Federated Learning based on when they stop sharing data with the AI/ML application server.

In addition, the following is needed to support AI/ML operations in the Application Layer:

-	AF requests to the 5G System in the context of 5GS assistance to AI/ML operations in the application layer shall be authorized by the 5GC using existing mechanisms.

NOTE:	In this Release, assistance to AI/ML operations in the application layer is not supported to roaming UEs.

-	UPF can perform traffic detection for AI/ML traffic as defined in clause 5.8.2.4 and the network needs to support charging for AI/ML traffic.

Editor's note:	How the network and/or AF can differentiate AI/ML traffic is FFS.

-

Editor's note:	It is FFS with additional clauses to summarize the other respective 5GC assistance functionalities for the AI/ML operations in the application layer and the references to the relevant normative clauses once the normative work becomes stable. The respective 5GC assistance functionalities are:
	
-	Information exposure to authorized 3rd party.
-	External parameter provisioning.
	
	
	
-	Network data analytics support.

* * * End of Changes * * *