3GPP TSG-SA2 Meeting #156AH (e-meeting)	S2-2304039

# April 17 – 21, 2023, Elbonia

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3954 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving ENs in the Procedures for Consolidated-MBR Monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-03-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solving the ENs in clause 4.15.6.13.2: - Proper reference to the UPF Event Exposure needs further clarification. - Whether and how to leverage direct or indirect UPF event notification is FFS. - Whether and how to activate the Consolidated-MBR monitoring during the create procedure is FFS. Solving the ENs in clause 4.15.6.13.3: - Whether and how to leverage direct or indirect UPF event notification is FFS. - Proper reference to the UPF Event Exposure needs further clarification. - Whether and how to activate the Consolidated-MBR monitoring during the create procedure is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To solve the ENs, the following changes are made: - For "Proper reference to the UPF Event Exposure needs further clarification", the reference for UPF Event Exposure is added to step 12 and 13 in clause 4.15.6.13.2, and step 10 in clause 4.15.6.13.3. - For "Whether and how to leverage direct or indirect UPF event notification is FFS.", descriptions are added to steps 1, 5, 12 and 13 in clause 4.15.6.13.2, and steps 1 and 2 in clause 4.15.6.13.3. - For "Whether and how to activate the Consolidated-MBR monitoring during the create procedure is FFS.", descriptions are added to steps 1, 2, 5 and 13 in clause 4.15.6.13.2, and steps 1,2 and 11 in clause 4.15.6.13.3. - Modified the Table 4.15.6.13.1-1 in clause 4.15.6.13.1 for mapping the NEF service requests of the multi-member AF sessions and the NEF requests to PCF. - Modified the descriptions in clause 4.15.6.13.1 correspondly to the above changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for Consolidated-MBR Monitoring are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.13.1, 4.15.6.13.2, 4.15.6.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.15.6.13	Multi-member AF session with required QoS for a list of UEs

##### 4.15.6.13.1	General Descriptions

This clause describes the procedure to request QoS and to perform QoS monitoring for the communication between a list of UEs, identified by the UE IP address(es) and the AF.

Editor's note:	It is FFS whether to converge with GMEC to leverage common service interface to also support Multi-member AF session with required QoS for a list of UEs.

The NEF receives the Multi-member AF session request with required QoS for a list of UEs identified by their IP addresses. The NEF then converts the Multi-member AF session to the AF session with required QoS to each of the UE's serving PCF which then applies AF session with required QoS procedure as described in clause 4.15.6.6 "Setting up an AF session with required QoS procedure" to each UE, except for the TSCTSF related information.

The NEF receives the outcome of the individual AF session with required QoS corresponding to each UE's IP address, and consolidates them into single response before forwarding it to the AF.

When the AF subscribes to QoS Monitoring for the list of UEs identified by their respective IP addresses, the AF may provide a Consolidated MBR threshold that is stored in the NEF, or the NEF may be configured with a Consolidated MBR threshold. If the AF requests to monitor the Consolidated MBR, the NEF subscribes to QoS Monitoring to request Data Rate monitoring for each AF session with required QoS parameter(s) for each UE identified by its IP address. The list of the QoS Monitoring for Data Rate reports will be sent to the AF by NEF, only if the aggregated bit rate exceeds the Consolidated-MBR threshold. Otherwise, the list of QoS Monitoring for Data Rate report will be sent to the AF according to the reporting frequency. When the AF requests to monitor the Consolidated MBR, the NEF may include an indication of direct event notification to indicate that QoS Monitoring reports shall be sent by the UPF directly to the NEF.

The following describes the mapping of the 4 types of NEF service requests of the multi-member AF sessions and the NEF requests to PCF using Npcf_PolicyAuthorization service. There may be a different PCF for each of the UEs and their related IP addresses in the Nnef_MultiMemberAFSessionWithQoS Request.

Table 4.15.6.13.1-1

| Nnef_MultiMemberAFSessionWithQoS Request | Npcf_PolicyAuthorization request |
| --- | --- |
| Create | Npcf_PolicyAuthorization Create request for each of UE IP address received in the Nnef_MultiMemberAFSessionWithQoS Request. Npcf_PolicyAuthorization Subscribe to request QoS Monitoring request for each QoS Monitoring requests received in the Nnef_MultiMemberAFSessionWithQoS Request. |
| Update | Npcf_PolicyAuthorization Update request to update the QoS request for a UE IP address received in the Nnef_MultiMemberAFSessionWithQoS_Update Request. Npcf_PolicyAuthorization Create request for each new UE IP address received in the Nnef_MultiMemberAFSessionWithQoS Request. Npcf_PolicyAuthorization Delete request for each removed UE IP address received in the Nnef_MultiMemberAFSessionWithQoS Request. Npcf_PolicyAuthorization Subscribe to request QoS Monitoring request for each new QoS Monitoring requests received in the Nnef_MultiMemberAFSessionWithQoS Request. More than one of the above operations can be requested at the same time. |
| Revoke | Npcf_PolicyAuthorization Delete request for each removed UE IP address received in the Nnef_MultiMemberAFSessionWithQoS Request. |
| Notify | Npcf_PolicyAuthorization Notify to report the events that NEF subscribed to. |

The Consolidated Maximum Bit Rate (Consolidated-MBR) threshold is described as follows:

-	The Consolidated-MBR threshold provides the upper bound of the aggregate bit rate across all GBR QoS Flows corresponding to the list of PDU Sessions of the UEs who participate in a list of transmissions concurrently (e.g. Federated Learning (FL) operation) with active user planes. The NEF aggregates bit rate across the set of selected QoS Flows against the Consolidated-MBR threshold.

In the case of the QoS monitoring for a list of QoS flows, Consolidated-MBR monitoring or concurrent reporting for the list of QoS flows' data rate or both could be supported at the same time by the NEF according to the AF request. The NEF determines whether Consolidated-MBR from the AF or the locally configure one is to be used. When the AF provides the Consolidated-MBR Threshold, the NEF subscribes to QoS Monitoring for reporting data rates to NEF and may request direct reporting.

* * * Next Changes * * *

##### 4.15.6.13.2	Procedures for Creating a Multi-member AF session with required QoS for a list of UEs

Figure 4.15.6.13.2-1: Procedures for creating Multi-member AF sessions with required QoS for a list of UEs

1.	The AF sends a request to reserve resources for AF sessions for a list of UEs using Nnef_MultiMemberAFsessionWithQoS_Create request message (a list of UE address, AF Identifier, Flow description information or External Application Identifier, QoS Reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), QoS parameter(s) to be measured, Reporting frequency, Target of reporting, optional an indication of local event notification, Consolidated-MBR Threshold, DNN, S-NSSAI) to the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20]. The AF may provide QoS parameter(s) to be measured as defined in clause 5.46 of TS 23.501 [2], Reporting frequency, Target of reporting, optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20]. The AF may include an indication for activating Consolidated-MBR monitoring, the UL bitrate or DL bitrate as defined in clause 5.46 of TS 23.501 [2] in the QoS parameter(s) to be measured, Consolidated-MBR Threshold, and the Target of reporting set to the NEF for the Consolidated-MBR monitoring.

Editor's note:	The name of the service is FFS.

2.	The NEF authorizes the AF request and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all steps (except step 5) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed. The NEF generates the Transaction Reference ID and associate the list of UE address received from the AF to the Transaction Reference ID. If Consolidated-MBR monitoring is being indicated but the parameters are not provided by the AF, or the NEF determines to do Consolidated-MBR monitoring, the NEF may configure a Consolidated-MBR Threshold and request QoS Monitoring for reporting data rate, i.e, the UL bitrate or DL bitrate as defined in clause 5.46 of TS 23.501 [2] in the QoS parameter(s) to be measured, and direct reporting.

3-4.	The NEF finds BSF serving the UE IP address(es) using NRF and then for each UE IP address, the NEF uses Nbsf_Management_Discovery service operation, providing the UE IP address to discover each PCF for the PDU Session.

Steps 5-10 applies for each UE address in the list of UE addresses.

5.	The NEF provides the UE address and the received parameters in step 1 or the configured parameters in step 2 to the PCF in the Npcf_PolicyAuthorization_Create request. In the request, an indication for activating Consolidated-MBR monitoring is included if it is indicated by the AF in step 1 or determined by the NEF in step 2. An indication of direct event notification may also include in the request to indicate that QoS Monitoring reports shall be sent by the UPF directly to the NEF.

6.	Step 4 in Figure 4.15.6.6-1 applies except the PCF sending the response message to the NEF. The PCF generates authorized QoS Monitoring policy according to the information provided from the NEF in step 5 and provides PCC rules with the policy to the SMF as described in clause 6.1.3.21 of TS 23.503 [20]. The SMF configures the UPF to perform QoS Monitoring as described in clause 5.46 of TS 23.501 [2].

7.	The PCF sends the Npcf_PolicyAuthorization_Create response message to the NEF.

8.	If the NEF receives the Npcf_PolicyAuthorization_Create response, the NEF sends a Nnef_MultiMemberAFsessionWithQoS_Create response message (Transaction Reference ID, Result for list of UE address) to the AF. Result for list of UE address includes whether the request is granted or not for every UE address in the list of UE address.

9.	Step 6 in Figure 4.15.6.6-1 applies.

10.	Step 7 in Figure 4.15.6.6-1 applies.

Editor's note:	Whether and how to leverage direct or indirect UPF event notification is FFS.11.	The NEF sends Nnef_MultiMemberAFsessionWithQoS_Notify message with the aggregated resource allocation status events to the AF.

12.	If direct event notification is requested based on the parameters received in step 5, the QoS Monitoring events are reported by the UPF to the NEF using Nupf_EventExposure service as described in clause 5.2.26.2.

13.	The NEF sends Nnef_MultiMemberAFsessionWithQoS_Notify message with the aggregated QoS Monitoring events to the AF. If the NEF received the indication for activating Consolidated-MBR monitoring and know the Consolidated-MBR Threshold, the NEF aggregates the bitrate received from step 10 or step 12 and if the aggregated bit rate exceeds the Consolidated-MBR Threshold, the NEF sends the event report for Consolidated-MBR Monitoring which includes the aggregated QoS Monitoring event for bitrates and the indication whether the aggregated bitrate is exceeded or not. If the Reporting Frequency received from the AF is periodic, the NEF periodically sends the event report for Consolidated-MBR Monitoring to the AF.

The AF may send Nnef_MultiMemberAFsessionWithQoS_Revoke request containing the Transaction Reference ID to the NEF in order to revoke all the AF sessions related to the Transaction Reference ID. The NEF authorizes the revoke request and triggers the Npcf_PolicyAuthorization_Delete and the Npcf_PolicyAuthorization_Unsubscribe operations for every UE in the list of UE address associated with the Transaction Reference ID.

* * * Next Changes * * *

##### 4.15.6.13.3	Procedures for Updating a Multi-member AF session with required QoS for a list of UEs

Figure 4.15.6.13.3-1: Procedure for updating Multi-member AF session with required QoS for a list of UEs

1.	The AF which controls the operation for a multi-member AF Session invokes the Nnef_MultiMemberAFSessionWithQoS_ Update Request with a list of UEs' serving PCFs via the support of NEF to initiate the update the resource allocations and/or to direct the required QoS monitoring for a selected list of UEs to support a specific Application (e.g. FL operation).

When the AF requests to update the UE(s) in the list and/or to update either the resource allocation and/or to update the type of QoS monitoring, the AF initiates the Nnef_MultiMemberAFSessionWithQoS_Update request which includes the list of UE addresses, the AF Identifier, Transaction Reference ID, [Flow description information], [QoS Reference or individual QoS parameters], [Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20])]) to NEF for updating the reserved resources and/or to update the type of QoS monitoring. The Transaction Reference ID provided in the AF session with required QoS update request message is set to the Transaction Reference ID that was assigned by the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The Consolidated-MBR threshold for Consolidated-MBR monitoring (optional); in additional, the list of UE addresses subjected for Consolidated-MBR monitoring may be provided. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as specified in clause 6.1.3.22 of TS 23.503 [20]. The AF may include an indication for updating Consolidated-MBR monitoring.

NOTE 1:	If Consolidated-MBR Threshold is provided, the QoS parameter(s) to be measured indicating the Guaranteed Bitrate shall be provided.

NOTE 2:	When the AF request is for Consolidated-MBR monitoring is set for event trigger, the QoS Flow data rate reporting for the list of UEs provided to the AF by the NEF only when the Consolidated-MBR threshold is exceeded.

NOTE 3:	When the Consolidated-MBR threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subjected for Consolidated-MBR monitoring is also provided together with the Consolidate-MBR threshold, then such list has to be the subset of the list of UE addresses.

NOTE 4:	If AF wants to terminate the Consolidated-MBR monitoring for the list of AF sessions, AF does not include the Consolidate-MBR threshold in the AF request.

2.	NEF authorizes the Multi-member AF request, and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all following steps are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed. If updating Consolidated-MBR monitoring is indicated but the parameters are not provided by the AF, or the NEF determines to update the Consolidated-MBR monitoring, the NEF may configure a Consolidated-MBR Threshold and update the subscription to QoS Monitoring for reporting data rates, i.e, the UL bitrate or DL bitrate as defined in clause 5.46 of TS 23.501 [2] in the QoS parameter(s) to be measured, and direct reporting.

3.-5.	Once the UE(s)' serving PCF(s) are identified, the NEF converts the multi-member AF request into AF Session With QoS for a list of UEs. The UE IP address's serving PCF authorizes the NEF request and responds to NEF with Npcf_PolicyAuthorization response according to the success or failure of the authorization. In the NEF request, an indication of direct event notification may be included to indicate that QoS Monitoring reports shall be sent by the UPF directly to the NEF.

When the AF requests for Nnef_MultiMemberAFSessionWithQoS_Update in order to update the UE(s) in the list for the resource allocation and/or to update the type of QoS monitoring, the NEF refers to the local context of the corresponding to the list of UEs and determines for which the new UE is to be added to the list, for which of the existing UE is to be removed from the list and/or for which of the existing UE is to be updated by invoking one or more of the following Npcf_PolicyAuthorization procedures:

a)	If the Multi-member AF session update request involves a new UE is to be added to the list with resource allocation, after inquiring BSF via Nbsf_Management Discovery service operation to find out the address of the UE's serving PCF, the NEF initiates the Npcf_PolicyAuthorization_Create as described in clauses 4.15.6.6 and 5.2.6.9.4 excluding TSCTSF related procedure towards the PCF for the corresponding UE should be followed.

b)	If the Multi-member AF session update request involves an existing UE is to be removed to the list, the NEF initiates the Npcf_PolicyAuthorization_Delete as described in clause 5.2.6.9.4 excluding TSCTSF related info towards the PCF for the corresponding UE should be followed.

Editor's note:	Whether and how to support Consolidate_MBR monitoring concurrently when requesting the resource for the new UE is FFS.

c)	If the Multi-member AF session update request involves either the update the QoS resources or QoS monitoring or both for the existing UE in the group, the NEF initiates the Npcf_PolicyAuthorization_Update to the UE's serving PCF.

If the list of UE addresses subjected for the Consolidated-MBR monitoring is received from the AF, NEF updates the list of UEs subjected for Consolidated-MBR monitoring.

6.	NEF aggregates the authorization responses from the PCFs as described in step 6 of clause 4.15.6.6a and sends the AF with the Nnef_MultiMemberAFSessionWithQoS Update response corresponding to the earlier AF session request. The consolidated response for NEF provides specific success or failure response corresponding to each UE. NEF disassociates the UE IP address(es) to be removed and/or failed to be authorized by the PCF from the AF with the existing Transaction Reference ID.

7.	If NEF requested Npcf_PolicyAuthorization_ Update is authorized by the PCF as described in c) of step 5 above, the PCF processes the Npcf_PolicyAuthorization_Update request according to the actions described in step 4 of clause 4.15.6.6 for SM policy Association Modification procedure and/or QoS Monitoring as described in clause 6.1.3.21 of TS 23.503 [20]. The NEF shall send a Npcf_PolicyAuthorization_Subscribe message to the PCF to subscribe to notifications of Resource allocation status, QoS Monitoring for reporting data rate and may subscribe to other events described in clause 6.1.3.18 of TS 23.503 [20].

8.-9.	Dependent on the establishment of the transmission resources corresponding to the QoS update succeeded or failed, the UE's serving PCF sends Npcf_PolicyAuthorization_Notify message to the NEF about the Resource allocation status and NEF consolidates the notify messages from the respective UE IP addresses' serving PCFs before notifying the AF via Nnef_MultiMemberAFSessionWithQoS_Notify for the respective PCFs' outcome together with the AF Transaction Reference ID. NEF associates the UE IP address(es) which have been allocated with resources with the existing Transaction Reference ID.

10.	If the QoS Monitoring event for Data Rate Monitoring was subscribed by the PCF, and direct event notification is requested, the SMF requests UPF to report the data rate for the target QoS flow periodically to the NEF using Nupf_EventExposure service as described in clause 5.2.26.2; otherwise, if the QoS Monitoring event for Data Rate Monitoring is unsubscribed as indicated by the PCF, then the SMF requests the UPF to terminate the QoS Monitoring reporting for the QoS Flow.

11.	If receiving the Data Rate Monitoring report of the QoS Monitoring event for specific QoS flow from UPFs, and the Consolidated-MBR threshold was provided by the prior Nnef_MultiMemberAFSessionWithQoS request or updated by the AF in step 1 or by the NEF in step 4, the NEF sums up the data rates of all the active QoS flows according to the list of UEs which are subjected to Consolidate-MBR monitoring through the duration of the Reporting Frequency. If the given Application AI/ML traffic is asymmetric, the bit rate aggregation is done per uplink and downlink separately; otherwise, the bit rate aggregation is done per either uplink or downlink. NEF compares the aggregated bit rate of the uplink or downlink against the Consolidated-MBR threshold for the uplink or downlink, accordingly.

12.	In the case of event reporting for Consolidated-MBR monitoring, only if the aggregate bit rate exceeds the Consolidated-MBR threshold, the NEF will notify the Application AI/ML AF via Nnef_MultiMemberAFSessionWithQoS_Notify for the collected UPF QoS monitoring reports together with the status of the Consolidated-MBR monitoring and the AF Transaction Reference ID; otherwise, the NEF notify the AF periodically with the collected UPF QoS monitoring reports together with the status of the of the Consolidate-MBR monitoring, if requested, and the AF Transaction Reference ID.

The UPF QoS Monitoring event for Data Rate Monitoring continues until AF terminates the Consolidated-MBR monitoring. The UPF continues to perform the QoS Monitoring event for Data Rate Monitoring for the given QoS flow to the NEF periodically corresponding to the Reporting Frequency as described in steps 11-12 above.

* * * End of Changes * * *