3GPP TSG-WG SA2 Meeting #156                                           	S2-2304077

April 17-21, 2023 Elbonia

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 738 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extending the maximum response time for FL status report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS23.288 v18.1.0 contains the following ENs, in clauses 6.2C.2.2: Editor's note: Further information on how the server NWDAF behaves based on the maximum response time from client NWDAF is FFS. Editor's note: It is FFS whether Server NWDAF solicits a response from an FL clients which has not reported the FL status within the maximum response time. This CR proposes to add an operation that enables the server NWDAF to query the client NWDAF about the training status and in case extend the maximum response time to allow the client NWDAF to complete the training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TS23.288, clause 6.2C.2.2 is updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF cannot extend the time allowed to a client NWDAF to complete the training. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 6.2C.2.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF) sends a subscription request to NWDAF containing MTLF to retrieve a ML model, using Nnwdaf_MLModelProvision service as defined in TS 23.288 [5], including Analytics ID, model metric (e.g., accuracy, precision, recall, etc), pre-determined status (e.g. requested accuracy level and/or total training time) and/or periodically reporting condition (e.g. training round/time).

NOTE 1:	The requested accuracy level can be used to indicate the target accuracy level of the training process, and the FL server NWDAF may stop the training process when the requested accuracy level is achieved during the training process.

If the consumer (i.e. the NWDAF containing AnLF or the consumer of the NWDAF containing AnLF) provides the time when the analytics information is needed, the Server NWDAF can take this information into account to decide the maximum response time for its Client NWDAFs.

1.	Server NWDAF sends a request to the selected NWDAF containing MTLF(Client NWDAF) that participates in the Federated learning to perform the local model training for Federated Learning, including model metric. The request also includes the maximum response time before which the Client NWDAF has to report the interim local ML model information to the Server NWDAF.

2.	Each Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

3.	During Federated Learning training procedure, each Client NWDAF further trains the retrieved ML model from the Server NWDAF based on its own data, and reports interim local ML model information to the Server NWDAF. Each Client NWDAF also computes local model metric and includes it in the local training status report sent to the Server NWDAF, which optionally also contains other information about local training in the Client NWDAF, e.g. characteristics of local training dataset (e.g., area, sampling ratio, max/min of value of each dimension of data), etc.

NOTE 2:	The parameters in characteristics of local training dataset are up to the implementation.

The ML model, which is sent from the Client NWDAF(s) to the Server NWDAF during the FL training process, is the information needed by the Server NWDAF to build the aggregated model based on the locally trained models.

Editor's note:	The services (i.e. Nnwdaf_MLModelTraining, Nnwdaf_MLModelTrainingInfo) in steps 1, 3, 5a and 6 will be updated in the Figure 6.2C.2.1-1.

If the Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time provided by the Server NWDAF, the Client NWDAF may notify the Server NWDAF on the delay event with proper cause information (e.g. local ML model training failure, more time necessary for local ML model training) before the maximum response time elapses.

3a.	[Optional] If the Client NWDAF does not send the interim local ML model information to Server NWDAF within the maximum response time provided by the Server NWDAF, the Server NWDAF may send an ML model information request to obtain the expected time for local training to complete.

3b.	[Optional] The Client NWDAF sends a response to the Server NWDAF including the expected time to complete the training.

3c.	[Optional] The Server NWDAF may send to the Client NWDAF an extended maximum response time before which the Client NWDAF has to report the interim local ML model information to the Server NWDAF. Otherwise, the Server NWDAF may indicate Client NWDAF to skip reporting for this iteration.

4.	The Server NWDAF aggregates all the local ML model information retrieved at step 3, to update the global ML model. The Server NWDAF may also compute the global model metric, e.g. based on the local model metric(s) or by applying the global model on the validation dataset (if available).

If the Server NWDAF provides the maximum response time for the Client NWDAFs to provide the interim local ML model information in step 1, or the extended maximum response time in step 3c, the Server NWDAF decides either to wait for the Client NWDAFs which have not yet provided their interim local ML model within the (extended) maximum response time or aggregates only the retrieved local ML model information instances to update global ML model. The Server NWDAF makes this decision, considering the notification/response from the Client NWDAF or, if the notification/response is not received, based on local configuration.

5a.	[Optional] Based on the consumer request, the Server NWDAF updates the training status (e.g. global model metric) to the consumer periodically (e.g. a certain number of training rounds or every 10 min) or dynamically when some pre-determined status is achieved (e.g. the requested accuracy level is achieved or training time expires).

5b.	[Optional] Consumer decides whether the current model can fulfil the requirement, e.g. global model metric is satisfactory for the consumer. The consumer modifies subscription (stops or continues the training process) according to the consumer decision.

5c.	[Optional] According to the request from the consumer, Server NWDAF updates or terminates the current FL training process.

6.	If the FL procedure continues, Server NWDAF sends the aggregated ML model information to each Client NWDAF for next round model training.

7.	Each Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the Server NWDAF at step 6.

NOTE 3:	The steps 2-7 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is complete, the Server NWDAF may send the globally optimal ML model information to the consumer.

# * * * * End of change * * * *