3GPP TSG-WG SA2 Meeting # 156-e	S2-2304081

# E-Meeting, April 17 – 21, 2023																revision S2-230xxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3960 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | URSP rules for non-3GPP access path switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is currently not clear when the UE decides to switch non-3GPP access paths in a MA PDU session as this is left to UE implementation. Received URSP rules may indicate the non-3GPP access that can be used by the UE (either trusted non-3GPP access (TNGF) or untrusted non-3GPP access (N3IWF)). The UE (re-)evaluates the received URSP rules in order to determine which non-3GPP access it can use and to decide on switching the user plane resources from old to new non-3GPP access. The network/PCF may prefer a certain non-3GPP access (trusted/untrusted) over another non-3GPP access (untrusted/trusted) based on operator policies or local configurations, and then it decides to update the URSP rules to the UE to deliver that preference to the UE. This assists the UE to know when to possibly trigger the traffic switching between two non-3GPP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the requirements of non-3GPP access path switching to take into consideration the updated URSP rules. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No possibility for the network/operator to indicate its preference about which non-3GPP access is preferred, and ambiguity how/when the UE decides to switch non-3GPP access path. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.22.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.22.9.5	Registration procedures for non-3GPP access path switching

Editor’s note: A figure showing the below steps as a “delta” to the registration procedure is FFS.

If the UE supports non-3GPP access path switching and the network indicates that the network supports non-3GPP access path switching as described in clause 4.22.2, the UE may trigger a Mobility Registration Update via a new non-3GPP access to switch traffic from an old non-3GPP access (i.e., TNGF or N3IWF) to the new non-3GPP access (i.e., TNGF or N3IWF). In this case the Registration procedure described in clause 4.22.9.1 applies with the following differences and clarifications:

-	In step 1, if the UE decides, based on e.g., the received URSP rules or based on UE implementation, to switch the user plane from an old non-3GPP access to a new non-3GPP access where the Mobility Registration Update is sent, the UE indicates PDU Session ID(s) of the PDU Session(s) in the List Of PDU Sessions To Be Activated. This may include both PDU Session ID(s) corresponding to MA PDU Sessions and single access PDU Sessions.

NOTE 1:	The received URSP rules can include the preferred non-3GPP access (trusted or untrusted). This information can trigger the UE to switch the user plane traffic from an old to a new non-3GPP access which is indicated as preferred non-3GPP access.

The UE may also provide an ("Non-3GPP path switching while using old AN resources") indication in the Registration Request to indicate that the UP connection(s) via the old non-3GPP access can still be used for the MA PDU Session(s) during the Registration procedure. If the UP connection(s) via the old non-3GPP access cannot be used by the UE during the Registration procedure, the UE shall not provide an "Non-3GPP path switching while using old AN resources" indication.

The UE shall not perform non-3GPP access path switching if the PLMN of the selected new non-3GPP access is different from the PLMN of the old non-3GPP access.

-	In step 3, if the UE provided an ("Non-3GPP path switching while using old AN resources") indication in step 1 and the AMF supports to maintain two N2 connections for non-3GPP access during the Registration procedure, the AMF delays the release of the old N2 connection until the UP connection via the new non-3GPP access is established. Otherwise, the AMF may trigger AN release towards the old non-3GPP access before proceeding with the Registration procedure in the new non-3GPP access, as described in clause 4.12.4.2 for untrusted non-3GPP access and clause 4.12a.4.2 for trusted non-3GPP access with the following clarifications:

-	During the AN release procedure, the AMF should notify the SMF to release the UP resources for the activated PDU Sessions before sending the N2 UE Context Release Command to the old non-3GPP access.

-	In step 17, if the Registration procedure is triggered to switch traffic from the old non-3GPP access to the new non-3GPP access and the UE provided an ("Non-3GPP path switching while using old AN resources") indication in step 1, the AMF, if it supports maintaining two N2 connections for non-3GPP access, forwards this indication to the SMF in case the PDU Session is a MA PDU Session. If the SMF receives this indication, the SMF does not trigger release of the UP connection in the old non-3GPP access (if any).

-	After the UP connection via the new non-3GPP access is established, the UE and UPF start to send traffic via the new non-3GPP access.

- 	After the UP connection has been established in new non-3GPP access, the AMF also triggers AN release towards the old non-3GPP access (i.e. old N3IWF or TNGF), unless done previously.

NOTE 2: 	In order to support RAT restrictions for non-3GPP access in the above procedure, it is assumed that UDM has provided restricted non-3GPP RAT types, if any, in the RAT restriction parameter in AM subscription data. In order to ensure that RAT restrictions are not violated in case the PDU Sesssion is established in LBO roming scenarios, the AMF in VPLMN may be configured to not indicate support for non-3GPP path switch for inbound roaming UEs during MA PDU Session Establisment towards a SMF in VPLMN, unless it has been agreed in roaming agreements that non-3GPP path switch can be supported for such UEs.

* * * End of Changes * * *