3GPP TSG-SA WG2 Meeting #156 (e-meeting)	 S2-2304144

# Elbonia, April 17 – 21, 2023																		(rev of )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0205 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment with RAN on Xn/N2 based handover for inactive MBS session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 7.2.3.6 specifies the following: 7.2.3.6 Xn/N2 based handover for inactive MBS session If the MBS session is in Inactive state, comparing the handover procedure for the MBS session in Active state, the following additional procedures apply: - For Xn based handover, the information that MBS session is inactive is provided from the source RAN node towards the target RAN node. - For N2 based handover, the information that MBS session is inactive is provided from SMF towards the target RAN node. - For the MBS supporting NG-RAN node, the target NG-RAN establishes the shared tunnel with the MB-UPF as usual, if it hasn't been established before. However, as the MBS session is in Inactive state, the NG-RAN node will not allocate related radio resource. #1 The first 2 bullets above are not aligned with RAN specs, see below: Per TS 38.423, at Xn-based HO, the source RAN does not provide the information that MBS session is inactive, but provides Active MBS Session Information if available, see excerpt below: ===excerpt from TS 38.423=== 8.2.1 Handover Preparation 8.2.1.2 Successful Operation … For each MBS session for which the Active MBS Session Information IE is included in the MBS Session Information Item List IE, the target NG-RAN shall, if supported, use this information to setup respective MBS session resources. The target NG-RAN node shall, if supported, consider that the MBS sessions for which the Active MBS Session Information IE is not included are inactive. Per TS 38.413, at N2-based HO, SMF does not provide the information that MBS session is inactive to the target NG-RAN. The MBS related IEs that may be included in Handover Request are as follows (see also excerpt): MBS Active Session Information Source to Target List IE may be provided by source RAN to target RAN. MBS Session ID, area Session ID for location dependent MBS) and the information for associated QoS Flows (if applicable) may be provided by SMF to target RAN. ===excerpt from TS 38.413=== 8.4.2 Handover Resource Allocation 8.4.2.2 Successful Operation … In case there are MBS sessions the UE has joined, for all the MBS sessions the UE has joined, the SMF shall, if supported, include the MBS Session Setup Request List IE within the PDU Session Resource Setup Request Transfer IE in the HANDOVER REQUEST message. If the MBS Active Session Information Source to Target List IE is contained in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE within the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, assume the indicated MBS sessions to be active and establish MBS session resources as specified in TS 23.247 [44] and TS 38.300 [8], if applicable. The target NG-RAN node shall, if supported, consider that the MBS sessions the UE has joined which are not included in the MBS Active Session Information Source to Target List IE are inactive #2 The 3rd bullet in 7.2.3.6 does not correctly capture the difference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the actual differences compared to the handover procedures for with active MBS session: During Xn/N2-based HO, the source NG-RAN does not provide MBS Session Information to the target RAN. For Xn-based HO, SMF always triggers PDU Session modification to provide UE join info to NG-RAN; Even if the the shared delivery resource is established between the NG-RAN node and the MB-UPF, target NG-RAN does not allocate radio resource as the MBS Session is inactive. Add a note that MBS Session state Inactive is provided by MB-SMF to NG-RAN in clause 7.2.1.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with RAN spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 7.2.3.6	Xn/N2 based handover for inactive MBS session

If the MBS session is in Inactive state, the following differences apply as compared to the handover procedures for the MBS session in Active state:

-	The target NG-RAN is not provided with MBS session information by the source NG-RAN.

-	For Xn based handover, the SMF triggers modification of the associated PDU Session to the target NG-RAN node by including the multicast MBS session related information in N2 SM Information as specified in step 7 of clause 7.2.1.3.

-	For N2 based handover, establishment of shared delivery toward RAN node, if not performed yet, is triggered by MBS session related information contained in the associated PDU Session information provided by the SMF to the target RAN node.

-	For a MBS supporting target NG-RAN node, even if the shared delivery resource is established between the NG-RAN node and the MB-UPF, the NG-RAN node does not allocate related radio resource as the MBS session is in Inactive state.

NOTE 1: The MBS Session state Inactive is provided by MB-SMF to NG-RAN in the N2 SM information in step 7 of clause 7.2.1.4.

-	After a handover to a target NG-RAN node not supporting MBS, the SMF removes the associated QoS flow(s) information.

NOTE 2:	Whether the associated QoS Flow(s) are removed from UE, NG-RAN, or only resource in NG-RAN is removed is up to implementation.

* * * End of Changes * * *