SA WG2 Meeting #156-e	S2-2304162

# Electronic meeting,  2023-04-17 -- 2023-04-21

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0114 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Including a latency parameter related to DNAI mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It can be possible to reach an EAS from several DNAIs. See example below: The EAS can be reached from both DNAI1 and DNAI2, but DNAI2 has 1 more router hop. I.e. DNAI1 would be the first choice and DNAI2 a second choice for an edge application. Since the use of DNAI1 is not guaranteed, it would be good to give 2 (or more) choices of DNAIs to an AF. But for the AF to choose, some sort of distance associated with the DNAI to IP range could be provided to the AF. The proposal is to give expected latency between the DNAI and IP range. Since latency in a routed and or swithed network is not constant but can vary due to network load, re-routing due linkfailures etc, the word "expected" is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a latency parameter |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reduced information for the AF to select a desired DNAI |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502, CR 3973 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

## 6.8	Support for mapping between EAS address Information and DNAI

### 6.8.1	General

In order to make sure the AF can query for DNAIs, the 5GS may help determine proper DNAI(s) and notify the information to AF based on AF request providing EAS address information (i.e. IP address(es), EAS IP range(s) or FQDN(s)).

NEF/UDR is configured by OAM with the mapping information between EAS IP address information and DNAI. AF may subscribe the mapping information modification to NEF. AF may request immediate reporting. When the configuration information (relationship between DNAI and EAS address information) is changed, the UDR can notify the new mapping information to NEF. When the mapping information is stored in UDR, NEF may subscribe to mapping information in UDR, where NEF may request immediate reporting. If subscribed to information change, the UDR notifies the NEF with all the corresponding mapping information it has stored for the relevant DNN and/or S-NSSAI, and the NEF notifies the AF.

The EAS address and DNAI Mapping Information record in UDR is shown Table 6.8.1-1. The information elements for the NEF service Nnef_DNAIMapping_Subscribe is specified in clause 5.2.6.8 of TS 23.502 [3].

Table 6.8.1-1: Description of EAS address and DNAI Mapping Information

| Parameters | Description |
| --- | --- |
| EAS address information | IP address(es) of the EASs or the IP address ranges (IPv4 subnetwork(s) and/or IPv6 prefix(es)) or FQDN(s) where the EAS is deployed for each DNAI. [Mandatory] |
| DNAI(s) | DNAI(s) for the EAS Deployment information. [Mandatory] |
| DNN | DNN for the EAS Deployment Information. [Conditional] (NOTE 1) |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [Conditional] (NOTE 1) |
| Geographical Area | Geographical areas for EAS Deployment information. [Optional] |
| Expected Latency (NOTE 2) | Latency between DNAI and EAS address. |
| NOTE 1: At least one of DNN or S-NSSAI shall be present. NOTE 2: The Expected Latency is defined by the operator and can be either a quantitative value or relative value (against other latencies). The margin of errors is an operational configuration matter. |  |

Editor's note:	Whether and how UDR is used (e.g. How UDR data is to be structured) is FFS.

## END of CHANGES