SA WG2 Meeting #156E	S2-2304171

# Electronic meeting,  April 17-21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4222 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network exposure support for XR services and QoS Monitoring Policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solving the Editor Note in 5.37.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the EN: “Editor’s Note: It is FFS whether all QoS monitoring requests can be addressed with the limitation of one QoS Monitoring control information per PCC.” PCF derives one authorized QoS Monitoring policy for the PCC Rule as described in TS 23.503 [45] clause 6.1.3.21. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how the multiple QoS monitoring requests can be addressed with the limitation of one QoS Monitoring policy per PCC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 0956 CR0955 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.37.4	Network Exposure of 5GS information

5GS and XR/media services cooperate to provide a better user experience using External Network Exposure.

Based on the AF request, the 5GS can report measurements of QoS parameters based on the QoS Monitoring as defined in clause  5.45:

-	For the congestion information, based on the PCC rule from PCF, the SMF requests the NG-RAN to report via GTP-U header to UPF and to PSA UPF to expose the information described in clause 5.37.3.3 (this NG-RAN information can be provided to support ECN marking for L4S in PSA UPF as described in clause 5.37.3.3) via Nupf_EventExposure service or via SMF/PCF/NEF as described in clause 5.8.2.18.

-	For QoS Notification Control for GBR QoS Flow as defined in clause 5.7.2.4, upon SMF request, the NG-RAN may additionally support indicating that "GFBR can no longer (or can again) be guaranteed" via GTP-U to UPF and the PSA UPF exposes this information via Nupf_EventExposure or via SMF/PCF/NEF as described in clause 5.8.2.18.

-	Data rate information may be measured and exposed to the AF based on SMF request as one of the following:

-	measured and reported by PSA UPF via Nupf_EventExposure service or via SMF/PCF/NEF as described in clause 5.8.2.18.

-	measured by RAN and reported via SMF/PCF/NEF.

Editor's note:	It is for RAN WGs to confirm whether providing QoS Notification Control for GBR QoS Flow and data rate information can be included in Release 18.

-	Round trip delay for multiple QoS Flows of the XR service (e.g. the UL and DL are separated into two flows) in the same PDU Session is determined based on the information provided by the PSA UPF on the QoS Monitoring for delay measurement of individual QoS Flow as described in clause 5.33.3. The AF includes in the request the necessary information for the PCF to derive the associated QoS monitoring requirements for each PCC rule. The PCF provides the QoS Monitoring policies in the PCC rules for the XR service data flows. The PSA UPF reports the delay information per QoS Flow to the SMF. SMF reports to PCF. The PCF derives round trip delay information for the XR service data flows and exposes the information to the AF directly or via NEF.

NOTE:	How PCF calculates the requested round trip delay for multiple QoS flows from delays of individual QoS Flows is not defined in this specification.

QoS Monitoring is used to obtain measurements of QoS parameters of individual QoS Flows. PCF determines the measurements required based on input from AF and enables the measurements by generating an authorized QoS Monitoring Policy for the PCC Rule as specified in TS 23.503 [45] clause 6.1.3.21.

The AF may provide the Alternative QoS parameter set requirements and Averaging Window to the NEF/PCF for the GBR QoS Flow as specified in clause 4.15.6.6 of TS 23.502 [3].

Estimated bandwidth for 5QI may be exposed by NWDAF (according to information described in clause 6.9.2 of TS 23.288 [86]) to AF.

# * * * * End of changes * * * *