3GPP TSG-SA WG2 Meeting #156E (e-meeting)	S2-2304177

# Elbonia, 17 – 21 April, 2023												 		 (revision of S2-xxxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4226 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN related subscription for PEGC and PIN realization in 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As specified in clause 5.44.1, the UE acting as PEGC has subscription data related to PIN. However, it’s not clear how the PIN service is realized in subscription data for the PEGC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a PIN related service indication in SM subscription data Clarify PIN realization in 5GC with external/internal group ID in parameter provisioning from AF. Editorial updates |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing function. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.1, 5.44.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 5.44.1	General

Personal IoT Network (PIN) provides local connectivity between PIN elements i.e. UEs and/or non-3GPP devices. PIN elements communicate using PINE-to-PINE direct connection (the connection between two PIN elements) or PINE-to-PINE indirect connection (the connection between two PIN elements is relayed via one specific PIN element and/or 5GS). The management of the PINE-to-PINE direct connection is out of the scope of this specification. For the PINE-to PINE indirect connection, a UE acts as the specific PIN element with Gateway Capability (PEGC). The management of the PINE-to-PINE direct connection and PINE-to-PINE indirect connection without PDU session is out of the scope of this specification. With the support of the PEGC registered to 5G network, the PIN Elements have access to the 5G network services and may communicate with other PIN Elements via 5GC. A PEGC may support multiple PINs.

PIN and PIN elements are managed by specific PIN element with Management Capability (PEMC) and by an AF if AF deployed. A PIN includes at least one PEGC and at least one PEMC. The management of the PIN network and PIN Element (including the management role distribution between PEMC and AF) is out of the scope of this specification.

The PEGC is a UE with subscription data related to PIN within the 5GS and may register to 5GS. The PEMC does not have subscription data related to PIN within the 5GS and behaves as normal UE if it is registered in 5GS. See information in Annex O for the relation between PIN and 5GS.

An AF for PIN may be deployed to support the PIN service. The AF for PIN may communicate with PEMC and PEGC via application layer for management of the PIN which is transported as user plane data transparently to 5GS. The 5GC is enhanced to support the delivery of UE policy related to PIN service for UE acting as PEGC (as specified in clause 5.44.2) and to support the PDU session management for PIN service (as specified in clause 5.44.3). The external Group ID and internal group ID may be used by AF for PIN when AF provides parameters to 5GC.

The support of 5G-RG as a PEGC is not considered in this Release.

# * * * * 2nd change * * * *

#### 5.44.3.1	PDU Session Establishment for PIN

When a PDU Session associated with a PIN, based on indication in subscription data, is established by UE with PEGC, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The UE with PEGC may use IP address allocation methods as specified in clause 5.8.2.2 (e.g. IPv6 Prefix Delegation feature).

One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN if the PIN traffic is via PEGC/5GC. One PIN may be served by more than one PDU sessions in the PEGC.

Editor's note:	How and whether to handle the case where PINs share a PDU session and local switching is FFS.

Editor's note:	One PIN served by more than one PDU sessions in PEGC is FFS.

Editor's note:	The handling of the PEMC in 5GC in relation with PIN is FFS.

# * * * * End of change * * * *