# 3GPP TSG-SA WG2 Meeting #156E	S2-2304187

Electronic Meeting, 17 - 21 April, 2023	(Revision of S2-230xxxx)

Title:	[Draft]Reply LS on INACTIVE eDRX above 10.24sec and SDT

Response to:	LS on INACTIVE eDRX above 10.24sec and SDT (S2-2303947/R2-2302082)

Release:	Rel-18

# Work Item:	     NR_redcap_enh-Core

Source:	SA2

To:	RAN2, RAN3, CT1

cc:

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks RAN2 for informing the intention of allow SDT for extended DRX for RRC_INACTIVE sate beyond 10.24s.

SA2 would like to provide the following response:

- In Rel-18, CN based MT communication handling for UE in RRC_INACTIVE with eDRX>10.24s is introduced as one option for downlink signalling/data handling (e.g. MT data can be buffered in CN if it’s requested by NG-RAN). See more details in clause 5.31.7.2.1 in TS 23.501 and procedures in clause 4.8.1.1a and 4.8.2.2b in TS 23.502.

- It’s a NG-RAN decision if NG-RAN needs to inform the CN when UE is connected to network (e.g. depending on if NG-RAN has requested CN based MT communication handling).

- If CN based MT communication is requested by RAN and CN is informed that UE is connected to network (either via N2 UE Context Resume or Path Switch Request), regardless if it’s a resume with quick release or not, the CN delivers data/signalling towards RAN.

- The NG-RAN decides if the UE remains in RRC_CONNECTED or can be release to RRC_INACTIVE based on the receives amount of data from CN.

2. Actions:

To RAN2, RAN3, CT1

ACTION: 	SA2 kindly asks RAN2, RAN3 and CT1 take above information into consideration.

3. Date of Next SA2 Meeting:

SA2#157                   	May 22 - 26, 2023                                 	Berlin, DE

SA2#158   		August 21 - 25, 2023 				Goteborg, SE