3GPP TSG-WG SA2 Meeting #156E 	S2-2304212

# Online, Apr. 17-21, 2023	(revision of S2-2302505)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1265 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4 Describe option to support MRF registration and discovery using NRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TR 23.700-87 it has been concluded to include SBI based registration, discovery and selection of MRF using NRF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds new note below Figure AA.1.1-2 to clarify DCMF function may be performed by MRF. Adds new note to clarify Nnrf can be used to discover IMS nodes according to clause. Adds new clause describing discovery of IMS nodes not exposing SBA services. In this release only MRF is specified. Use of the Registration services to register an MRF is described. Use of the discovery service to discover or select an for MRF is described. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to discover MRF using NRF method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.1.1, AA.1.3, AA.XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Re-submission of unhandled TDoc S2-2302505 from SA2 #155 |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

## AA.1.1	Architectural Support

Figure AA.1.1-1 shows the architecture to support SBA interactions between IMS entities.

Figure AA.1.1-1: System Architecture to support SBA in IMS

NOTE 1:	The DCSF and MRF supports NF registration and discovery, but they do not provide services in this release of the specification.

Editor's note:	Whether the AS in Figure AA.1.1-1 needs to be changed to IMS AS is FFS.

Figure AA.1.1-2 shows the architecture using the reference point representation.

Figure AA.1.1-2: System Architecture to support SBA in IMS in reference point representation

NOTE 2:	In this Release of the specification the SBI capable IMS AS supporting data channel services is collocated with the TAS.

NOTE X: The DCMF in Figure AA.1.1-2, may instead be an MRF in the IMS domain that supports DC functions using existing IMS interfaces.

*** Second Change ***

## AA.1.3	Service based interface to support SBA in IMS

Npcf:	Service-based interface exhibited by PCF.

Nhss:	Service-based interface exhibited by an SBI capable HSS.

These SBI services provide equivalent functionality to the Diameter Rx and Cx/Sh reference points.

Nimsas:	Service-based interface exhibited by an SBI capable IMS AS.

Ndcmf:	Service-based interface exhibited by DCMF.

These SBI services provide functionality to support data channel management in IMS network.

To support co-existence of IMS nodes supporting SBA services and IMS nodes not supporting SBA services SBI enabled IMS nodes may support both SBI and non-SBI interfaces.

NOTE:	Nnrf may assist other IMS nodes to use NRF to perform registration, discovery, or selection, as described in clause AA.X.

*** Third Change ***

## AA.X NRF based Discovery and Selection for other IMS nodes

### AA.X.1 Service Based MRF Discovery and Selection

#### AA.X.1.1	General

While no SBI procedures are defined for MRF (and hence MRFC and MRFP), it is possible to utilize NRF to discover and select the MRF to use for an application or service. In networks where MRFC and MRFC are co-located it is only necessary to indicate that the service function is MRF, in networks where discreet MRFC and MRFP are deployed it is necessary to indicate which service function is needed – in some cases MRF, MRFC, and MRFP indications may co-exist inside the same network.

When an SBI capable IMS entity selects a MRF it shall use the IMS Media service list to ensure that the selected MRF supports the required media capabilities for the application.  The SBI capable IMS entity may utilize other available information to assist in selecting the MRF based on other implementation criteria (e.g. proximity of the IMS UE to MRF to minimize transmission delays).

A SBI capable IMS entity always selects an MRF (or MRFC or MRFP) within its own PLMN.

#### AA.X.1.2	MRF Registration

If an MRF has been configured to support NRF based discovery or selection it shall register with the NRF using the Nnrf_NFManagement_NFRegister Request message as defined in TS 23.502 [94]. The NF profile of the MRF registered in NRF includes necessary information for an SBI capable IMS entity interact with the MRF using non-SBI procedures.  In addition, the NF profile for MRF includes a text-based list of IMS Media services performed by the MRF (e.g. video transcoding, data channel services, voice conferencing, in-call announcements).

NOTE:	The list of Media services is implementation specific and individual entries in the list need to be configured to exactly correspond to the discovery requests made by AS or other SBA entity.

The MRF registration may include other information pertaining to the implementation (e.g.NF location to support routing media to nearest MRF, MSISDN prefix, or IP address for IP based “least-cost” routing).

#### AA.X.1.3	MRF Discovery and Selection

During the IMS procedure, an SBI capable IMS entity sends a Nnrf_NFDiscovery_Request to NRF as defined in TS 23.502 [94] to discover applicable MRF instances within a given PLMN.  The SBI capable entity may include a list of the service or services it is requesting to use, in which case the NRF will respond with the MRF (s) that support all the services listed. The SBI capable IMS entity may store all returned MRF instances and their NF profiles for subsequent use, including location and IP address (if supplied according to implementation).  If the IMS Media services list is blank the NRF shall return all applicable MRF according to the other selection parameters included in the request.

NOTE:	The SBI enabled IMS entity can also perform MRF discovery prior to receiving an IMS request without any including any applicable user, or IMS media service information in the request.

When separate MRFC and MRFP are used, and when there is no direct association between an MRFC and MRFP, then discovery and selection of the MRFP may be needed after the MRFC discovery; in such a case the MRFC may perform SBI discovery of the NRF MRFP, as if it were an AS.

*** End of Changes ***