3GPP TSG-SA WG2 Meeting #156E	S2-2304233

# E-meeting, April 17 - 21, 2023													 (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0089 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS support for Broadcast Remote ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the following conclusion described in clause 8.2 "Conclusions for Key Issue #2 - Support of Broadcast Remote ID" of TR 23.700-58, MBS needs to be supported for Broadcast Remote ID. - for scenarios that UAV UEs in coverage and register to the network, and to UAV-C that are 3GPP UEs, a mechanism using MBS location dependent broadcast service, based on solution #3. Because the scenario for BRID which a Tx UAV UE broadcasts BRID message and then Rx UAV UEs around the Tx UAV UE receive the BRID message is similar to the scenario for broadcast V2X message, MBS support for Broadcast Remote ID can be specified based on and by referring to MBS support for V2X specified in TS 23.287. Compared to MBS support for V2X including broadcast and multicast, it is considered that only broadcast is needed to support BRID using MBS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description to support BRID using MBS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No description to support BRID using MBS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 4.1, 4.2.4B (new), 4.3.3, 4.3.9, 5.5.X (new), 5.X (new), 5.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[5]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[7]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[11]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[12]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[13]	ASTM F3411.19: "Standard Specification for Remote ID and Tracking".

[14]	ASD-STAN prEN 4709-002:2022-03: "Aerospace series - Unmanned Aircraft Systems - Part 002: Direct Remote Identification".

[15]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[xx]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[yy]	3GPP TS 26.502: "5G multicast–broadcast services; User Service architecture".

[zz]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

* * * * Start of Next Change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A2X	Aircraft-to-anything

AAM	Area Airspace Manager

BRID	Broadcast Remote Identification

BVLOS	Beyond Visual Line of Sight

C2	Command and Control

DAA	Detect And Avoid

DDAA	Direct Detect And Avoid

MBS	Multicast/Broadcast Service

NRID	Networked Remote Identification

RID	Remote Identification

TPAE	Third Party Authorized Entity

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

USS	UAS Service Supplier

UTM	Uncrewed Aerial System Traffic Management

UUAA	USS UAV Authorization/Authentication

UUID	Universal Unique Identifier

* * * * Start of Next Change * * * *

4.1	General concept

The architecture enhancements for UAVs introduce the following functionality:

-	Authentication and authorization of a UAV with the USS during 5GS registration (optional).

-	Authentication and authorization of a UAV with the USS during PDU session establishment and PDN connection establishment.

-	Support for USS authorization of C2 Communication.

-	A reference model for UAV tracking, supporting three UAV tracking modes: UAV location reporting mode, UAV presence monitoring mode, and list of Aerial UEs in a geographic area. The 3GPP system supports geofencing (for in-flight UAV) and geocaging (for UAV on the ground intending to fly) functionality in USS by providing enablers such as location services, event notification to a subscribing USS, etc.

NOTE:	Geofencing/geocaging mechanisms are an air traffic control functionality performed by the USS and are out of scope of this specification. The 3GPP system provides enablers to support geofencing/geocaging functionality in USS, e.g. location services, enablement of C2 connectivity, event notification to a subscribing USS, etc. However, no specific geofencing/geocaging mechanisms are defined in 3GPP.

-	Support for Broadcast Remote ID using MBS.

* * * * Start of Next Change * * * *

4.2.4B	MBS for A2X architecture reference model

The reference architectures for MBS for A2X communication are based on the MBS reference architectures specified in clause 5.1 of TS 23.247 [xx].

Configuration options at Service and/or Application for MBS specified in Annex A of TS 23.247 [xx] and Interworking at reference points MB2 and xMB specified in Annex C of TS 23.247 [xx], can be applied for A2X communication via MBS.

The A2X Application Server acts as an AF/AS defined in clause 5.1 and Annex A of TS 23.247 [xx].

* * * * Start of Next Change * * * *

4.3.3	UAV

The UAV is a 3GPP UE supporting the UE functionality defined in TS 23.401 [6] and in TS 23.501 [2].

In addition:

-	a UAV that is configured for UAS services is provisioned with a single CAA-Level UAV ID;

-	a UAV that is configured for UAS services (i.e. is provisioned with a CAA-Level UAV ID) registers to the 3GPP system for UAS services (i.e. to take advantage of aerial features, connectivity with USS and for C2 connectivity) and provides the CAA-Level UAV ID and a UUAA Aviation Payload to 5GS or EPS. A UAV that has not performed a registration with aviation authorities shall not attempt to request for UAS services.

NOTE:	A UAV that is configured for UAS services but does not have an aerial subscription is not allowed by the network to register for UAS services.

-	a UAV that is configured for UAS services may support the following functions:

-	reports the A2X Capability and PC5 Capability for A2X (i.e. LTE PC5 and/or NR PC5) to 5GC over N1 reference point.

-	indicates A2X Policy Provisioning Request in UE Policy Container for UE triggered A2X Policy provisioning.

-	receives the A2X parameters from 5GC over N1 reference point.

-	supports procedures for A2X communication over PC5 reference point.

-	Configuration of parameters for A2X communication. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over A2X1 reference point from the A2X Application Server.

-	Provided with the following for receiving Broadcast Remote ID via MBS:

-	Announcement via existing announcement mechanisms specified in TS 23.247 [xx], TS 26.502 [yy] and TS 26.517 [zz]; or

-	MBS session announcement provisioned as specified in clauses 4.2.1.2.2 and 5.5.X.1.

-	Provisioned with MBS session announcement for receiving A2X Application Server information via MBS as specified in clauses 4.2.1.2.2 and 5.5.X.1.

* * * * Start of Next Change * * * *

4.3.9	A2X Application Server

The A2X Application Server implements a subset of the V2X AS functionality specified in TS 23.287 [11]:

-	includes AF functionality, and may support at least the following capabilities:

-	For A2X service parameters provisioning, the A2X AS provides the 5GC and the UAV UE (possibly via the UAVC) with parameters for A2X communications over PC5 reference point and parameters for A2X communications over MBS.

-	Act as an AF/AS in clause 5.1 and Annex A of TS 23.247 [xx] e.g. request NEF/MBSF for allocation/de-allocation of a set of TMGIs, initiate broadcast service from the 5GC by providing service information including QoS requirement to 5GC.

-	Perform MBS service area mapping as specified in clause 5.5.X.4.

NOTE:	The A2X AS and the USS serving a UAV can be the same or different entities.

* * * * Start of Next Change * * * *

5.5.X	Broadcast Remote ID using MBS

5.5.X.1	Policy/Parameter provisioning

The following set of information may be provisioned to the UAV UE for Broadcast Remote ID using MBS:

1)	Mapping of the A2X service type to A2X Application Server address information (consisting of IP address/FQDN and transport layer port#) for unicast.

2)	List of FQDNs or IP addresses of the A2X Application Servers, associated with served geographical area information and list of PLMNs that the configuration applies to.

3)	Mapping of the A2X service type to MBS session announcement for Broadcast Remote ID reception via MBS.

4)	MBS session announcement for receiving A2X Application Server information via MBS (i.e. for A2X Application Sever discovery using MBS).

In bullets 1) and 3), the A2X service type is Broadcast Remote ID.

5.5.X.2	Broadcast Remote ID reception via MBS

The mechanisms defined in TS 23.247 [xx] can be used to establish the suitable MBS sessions, and Broadcast Remote ID sent by UAV UE is routed by the A2X Application Server towards UAV UEs via broadcast MBS sessions. As defined in TS 23.247 [xx], MBS for roaming is not supported in this release.

For MBS reception of Broadcast Remote ID, announcement for A2X services is required for the UAV UE. To provide the UAV UE with the announcement, the following ways may be used:

-	Existing announcement mechanisms specified in TS 23.247 [xx], TS 26.502 [yy] and TS 26.517 [zz].

-	MBS session announcement provisioned as described in clauses 4.2.1.2.2 and 5.5.X.1.

The A2X Application Server providing the A2X service identified by A2X service type set to Broadcast Remote ID sends the Broadcast Remote ID via UDP/IP transport using information provided in the announcement.

Broadcast Remote ID transfer via MBS may be achieved by locating the MBS CN NFs (e.g. MB-UPF) closer to the gNB to meet the latency requirement as defined for 5QI 75 specified in TS 23.501 [2].

5.5.X.3	QoS handling

5QI 75 specified in TS 23.501 [2] is intended to be used for the delivery of Broadcast Remote ID over MBS.

5.5.X.4	MBS service area mapping

MBS service areas for A2X services may be configured at the A2X Application Server. Such service areas are not expected to change frequently. Alternatively, the A2X Application Server may obtain the UE location information and determine the MBS service area. The A2X Application Server performs the following procedures for managing the MBS sessions:

-	MBS session related procedures specified in TS 23.247 [xx] are used for managing MBS sessions.

-	The A2X Application Server derives MBS service area information (i.e. Cell ID list and/or TAI list, or geographical area information) of the target MBS service area for such MBS session related procedures.

-	The A2X Application Server may use the UAV UE location information obtained as below for the determination of the target MBS service area to deliver Broadcast Remote ID.

-	The UAV UE may provide its geographic location or Cell ID information over A2X1 reference point to the A2X Application Server.

-	A UAV UE may include its geographic location information in the Broadcast Remote ID, e.g. as defined in ASTM F3411.19 [13] or ASD-STAN prEN 4709-002 P1 [14].

-	A UAV UE may provide its geographic location information in the Broadcast Remote ID and Cell ID information in the signalling to the A2X Application Sever.

-	From such MBS session related procedures, the A2X Application Server knows which MBS session(s) are serving the target MBS service area. Hence, the A2X Application Server forwards a Broadcast Remote ID to the appropriate MBS session(s).

-	The A2X Application Server can apply a location dependent MBS service as defined in clause 6.2 of TS 23.247 [xx] for the distribution of different content data to different MBS service areas in such a manner that the areas are transparent to receiving UAV UEs and the received content of a UAV UE is determined by its location.

NOTE:	The A2X application in the UE discards the messages that are not relevant to the UE based on procedures internal to the UE.

* * * * Start of Next Change * * * *

5.X	A2X Application Server discovery

5.X.1	General

A UE needs to discover the A2X Application Server(s), when Broadcast Remote ID using MBS is used. The A2X Application Server address information may be configured on the UE or provisioned over N1 reference point, as specified in clause 5.5.X.1.

When the configuration contains the FQDN(s), the UE shall perform DNS to resolve the address(es) of the A2X Application Server. The UE may use the configured A2X Application Server information only in the designated geographical area. When the UE changes serving PLMN or crosses configured geographic areas, it should perform address resolution again.

NOTE:	When the A2X Application Server is notified by SMF indicating the DNAI change, as specified in clause 5.6.7 of TS 23.501 [2], the application layer can trigger the UE to perform DNS to resolve the address(es) of the A2X Application Server.

For a network that has deployed MBS, additional information to assist A2X Application Server discovery can be provided via broadcast MBS session. When a UE has the configuration for receiving A2X Application Server information via broadcast MBS session as specified in clause 5.5.X.1, it can perform the procedure specified in clause 5.X.3 to obtain additional local A2X Application Server information. The local A2X Application Server information obtained by MBS as described in clause 5.X.3 takes precedence over the A2X Application Server information in the UE. As defined in TS 23.247 [xx] broadcast service for roaming is not supported in this Release.

5.X.2	Multiple A2X Application Server and Localized A2X Application Server discovery and routing

Multiple A2X Application Servers may be involved in the A2X communication, each providing particular A2X services and/or serving a particular geographical region. Therefore, the A2X Application Server address information as specified in clause 5.5.X.1 can contain multiple servers' information. When multiple A2X Application Servers are configured, the application layer will choose the proper A2X Application Server to use.

When localized A2X Application Servers are deployed, Anycast may be used to conceal the server change from the UE. In this case, a FQDN is configured for a large region, e.g. the entire PLMN, and the UE only needs to resolve it once to an Anycast address. The UPF is responsible for routing the traffic to the appropriate local A2X Application Servers based on Anycast address.

5.X.3	A2X Application Server discovery using broadcast MBS session

The procedure for V2X Application Server discovery using broadcast MBS session as defined in clause 6.4.2 of TS 23.287 [11] is used for A2X Application Server discovery using broadcast MBS session with the following difference:

-	Throughout the procedure, V2X is replaced by A2X, e.g. V2X message to A2X message, V2X Application Server to A2X Application Server, etc.

The procedure for A2X Application Server discovery using broadcast MBS session may be used by the UE only when it is configured with the information to receive A2X Application Server information via MBS as specified in clause 5.5.X.1.

* * * * Start of Next Change * * * *

5.Y	MBS Service Description for A2X use

5.Y.1	General

The announced service information for A2X use should follow the guidance in clauses 5.Y.2 and 5.Y.3.

5.Y.2	MBS session announcement for A2X communication

As the A2X Application Server is out of scope of 3GPP, the exact information to be included in the MBS session announcement for A2X communication cannot be controlled. However, the A2X Application Server should make sure that at least the information listed in table 5.Y.2-1 is included in the MBS session announcement it provides.

The A2X message formats are handled by upper layer via Session Description Protocol (SDP).

Table 5.Y.2-1: Information for MBS session announcement for A2X communication

| Information element | Description |
| --- | --- |
| TMGI | TMGI information |
| MBS Service Type | Indicates that the MBS Session for the service is broadcast. |
| MBS service area information | Identifies the service area for the MBS session. |
| Frequency and/or MBS Frequency Selection Area (FSA) ID(s) (NOTE 1) | Identification of frequency if multi carrier support is provided |
| SDP information (NOTE 2) | SDP with IP multicast address and port number used for A2X communication via MBS. Depending on the A2X application, the A2X message can be carried directly on top of UDP, without any streaming protocols. |
| NOTE 1: The Frequency and/or FSA ID(s) are optional and only apply to broadcast MBS sessions. NOTE 2: Typical A2X application does not require Codec information in the SDP information. |  |

5.Y.3	MBS session announcement for A2X Application Server Discovery

The MBS session announcement for A2X Application Server Discovery is used to configure the UE for receiving local A2X Application Server information when it is provided over broadcast MBS session, as specified in clause 5.X.3.

The local Service Information contained in the broadcast is as described in clause 5.X.3, and should include similar information defined in clause 5.5.X.1.

NOTE:	Stage 3 defines the format of the local Service information.

Table 5.Y.3-1: Information for MBS session announcement for A2X Application Server Discovery

| Information element | Description |
| --- | --- |
| TMGI | TMGI information |
| MBS Service Type | Indicates that the MBS Session for the service is broadcast. |
| MBS service area information | Identifies the service area for the MBS session. |
| Frequency and/or MBS Frequency Selection Area (FSA) ID(s) | Identification of frequency if multi carrier support is provided |
| SDP information | SDP with IP multicast address and port number used for A2X Application Server discovery via MBS. The content of the message carries the local Service Information and should include following information: - Mapping of the A2X service types to A2X Application Server address information (consisting of IP address/FQDN and transport layer port#) for unicast, and MBS session announcement for A2X communication via MBS. |

* * * * End of Changes * * * *