3GPP TSG-SA WG2 Meeting #156-E	S2-2304238

# E-meeting, April 17 - 21, 2023											          (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0246 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of ENs with RAN dependency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following ENs with RAN dependency can be removed as no SA2 work is identified: Editor's note: Any additional update of procedure via Layer-2 UE-to-UE Relay, e.g. according to RAN's decision, will be included here. Editor's note: The procedures for Layer-2 Remote UE in CM-CONNECTED state switching the communication path to another Layer-2 UE-to-Network Relay will be aligned with RAN WG specifications. Editor's note: Mobility of 5G ProSe Layer-2 Remote UE between Direct and Indirect Network communication path will be defined by RAN WGs and alignment work (if any) will be made by SA2 based on RAN WGs conclusions. Editor's note: Alignment with RAN WGs is to be done. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the ENs as above, and add reference to RAN spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ENs are unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.3, 6.5.5.2, 6.7.3.3, 6.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.7.3.3	Procedure for Communication via Layer-2 UE-to-UE Relay

Figure 6.7.3.3-1: 5G ProSe UE-to-UE Relay Communication with integrated Discovery via Layer-2 UE-to-UE Relay

1.	The first hop and second hop PC5 unicast links are established as described in clause 6.7.3.2 and clause 6.4.3.7.4.

2.	For 5G ProSe UE-to-UE Relay Communication via Layer-2 UE-to-UE Relay, UE-1 establishes an end-to-end connection for unicast mode communication with UE-2 as described in clause 6.4.3.7.

* * * * 2nd Change * * * *

6.5.5.2	Procedures for Communication Path Switching between UE-to-Network Relays

This clause specifies the communication path switching procedures between two UE-to-Network Relays.

When performing the path switching between two 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, or between 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and 5G ProSe Layer-2 UE-to-Network Relay, the application layer procedures are used for service continuity support which is out of scope of this specification.

Path switching between two 5G ProSe Layer-3 UE-to-Network Relay with N3IWF is performed using the mechanism specified for Local mobility anchor within untrusted non-3GPP access networks using MOBIKE in clause 6.2.9 of TS 23.501 [4].

When Layer-2 Remote UE in CM-CONNECTED state switches the communication path to Layer-3 UE-to-Network Relay with N3IWF, Layer 2 Remote UE's PDU session(s) can handover to the indirect path via Layer-3 UE-to-Network Relay with N3IWF by reusing the procedures specified in clause 4.9.2 of TS 23.502 [5].

When Layer-3 Remote UE switches the communication path from Layer-3 UE-to-Network Relay with N3IWF to an indirect path via Layer-2 UE-to-Network Relay, Layer-3 Remote UE's PDU session(s) can handover to the indirect path via Layer-2 UE-to-Network Relay by reusing the procedures specified in clause 4.9.2 of TS 23.502 [5].

Path switching between two 5G ProSe Layer-2 UE-to-Network Relay is specified in TS 38.300 [12].

* * * * 3rd Change * * * *

6.5.2.3	Path switching between direct network communication path and indirect network communication path via 5G ProSe Layer-2 UE-to-Network Relay

Xn based (as defined in clause 4.9.1.2 of TS 23.502 [5]) and N2 based (as defined in clause 4.9.1.3 of TS 23.502 [5]) Handover procedure is used for inter-gNB indirect-to-direct and inter-gNB direct-to-indirect path switching for 5G ProSe Layer-2 Remote UE in CM-CONNECTED with RRC_CONNECTED state.

* * * * 4th Change * * * *

6.9.3	Multi-path transmission via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay

Multi-path transmission via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay is specified in TS 38.300 [12].

* * * * End of Changes * * * *