3GPP TSG-WG SA2 Meeting #156-E 	S2-2304239

E-meeting, April 17 - 21, 2023	(revision of S2-23xxxxx)

Source:	CATT

Title:	Procedures for Service Authorization and Provisioning to UE

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to add the Service Authorization and Provisioning to UE procedures into TS 23.586.

1. Introduction

This paper proposes to add the Service Authorization and Provisioning to UE procedures into TS 23.586 based on the following KI#1 conclusion in TR 23.700-86.

-	The procedures and signalling specified for ProSe in clause 6.2 of TS 23.304 [4] and for V2X in clause 6.2 of TS 23.287 [3] are used to provision the Ranging/SL positioning service authorization and policy/parameter provisioning to the UE.

2. Text Proposal

It is proposed to capture the following changes into TS 23.586.

The differences compared to TS 23.304 clause 6.2 are highlighted in yellow.

# * * * * First change* * * *

6.2	Procedures for Service Authorization and Provisioning to UE

6.2.1	General

The procedures for service authorization and provisioning to UE may be initiated by the PCF (as described in clause 6.2.2), by the UE (as described in clause 6.2.4), or by the AF (as described in clause 6.2.5).

6.2.2	PCF based Service Authorization and Provisioning to UE

For PCF based Service Authorization and Provisioning to UE, the Registration procedures as defined in clause 4.2.2.2 of TS 23.502 [3], UE Policy Association Establishment procedure as defined in clause 4.16.11 of TS 23.502 [3] and UE Policy Association Modification procedure as defined in clause 4.16.12 of TS 23.502 [3] apply with the following additions:

-	If the UE indicates Ranging/SL Positioning Capability in the Registration Request message and if the UE is authorized to use Ranging/SL Positioning service based on subscription data, the AMF selects the PCF which supports Ranging/SL Positioning Policy/Parameter provisioning as described in clause 6.2.3 and establishes a UE policy association with the PCF for Ranging/SL Positioning Policy/Parameter delivery. The AMF reports the authorized Ranging/SL Positioning Capability to the selected PCF, which may determine the Ranging/SL Positioning Policy/Parameter based on the UE's authorized Ranging/SL Positioning Capability.

The PCF may update the Ranging/SL Positioning Policy/parameters to the UE in following conditions:

-	UE Mobility, e.g. UE moves from one PLMN to another PLMN. This is achieved by using the procedure of UE Policy Association Modification initiated by the AMF, as defined in clause 4.16.12.1 of TS 23.502 [3].

-	When there is a subscription change in the list of PLMNs where the UE is authorized to perform Ranging/SL Positioning services. This is achieved by using UE Policy Association Modification initiated by the PCF procedure as defined in clause 4.16.12.2 of TS 23.502 [3].

-	When there is a change of service specific parameter (including path selection policy) as described in clause 6.2.5 (performing the procedure in clause 4.15.6.7 of TS 23.502 [3]).

-	When the timer associated with some Policy/parameter expires.

-	When the UE determines that the Ranging/SL Positioning Policy/parameter(s) is invalid and performs UE triggered Policy Provisioning procedure to the PCF.

If the serving PLMN is removed from the list of PLMNs in the service authorization parameters, the service authorization is revoked in the UE.

When the UE is roaming, the change of subscription resulting in updates of the service authorization parameters are transferred to the UE by H-PCF via V-PCF.

The UE may perform Ranging/SL Positioning UE triggered Policy Provisioning procedure to the PCF, after Registration procedure has been completed, as specified in clause 6.2.4 when the UE determines the Ranging/SL Positioning Policy/Parameter is invalid (e.g. Policy/Parameter is outdated, missing or invalid).

When the UE disables a Ranging/SL Positioning capability, the PCF may stop updating the corresponding Ranging/SL Positioning Policy/parameter(s) and when the UE enables a Ranging/SL Positioning capability the PCF may need to provide or update the corresponding Ranging/SL Positioning Policy/parameter(s).

6.2.3	PCF discovery

PCF discovery and selection mechanism defined in clause 6.3.7.1 of TS 23.501 [2] applies with the following addition to enable a PCF instance is selected for Ranging/SL Positioning service and for UE:

-	Based on the indication from the UE and/or UE subscription data during the Registration procedure as specified in clause 6.5.2, the AMF may include the Ranging/SL Positioning Capability indication in the Nnrf_NFDiscovery_Request message as the optional input parameter. If provided, the NRF takes the information into account for discovering the PCF instance.

6.2.4	Procedure for UE triggered ProSe Policy provisioning

The UE triggered Policy Provisioning procedure is initiated by the UE to request Ranging/SL Positioning Policy/Parameter from the PCF when UE determines the Ranging/SL Positioning Policy/Parameter is invalid in the following cases:

-	if the validity timer indicated in the Ranging/SL Positioning Policy/Parameter expires;

-	if there are no valid parameters, e.g. for the Ranging/SL Positioning Identifier a UE wants to use, for current area, or due to abnormal situation.

Figure 6.2.4-1: UE triggered Ranging/SL Positioning Policy provisioning procedure

1.	The UE sends the UL NAS TRANSPORT message carrying the UE Policy Container (UE Policy Provisioning Request to request Ranging/SL Positioning policies) to the AMF.

2.	The AMF sends Namf_Communication_N1MessageNotify request to the PCF including the UE Policy Container received from UE.

3.	The PCF receives UE Policy Container which indicates UE Policy Provisioning Request to request Ranging/SL Positioning policies. If the Ranging/SL Positioning policies are authorized based on AMF input, the PCF performs the UE Policy delivery procedure as defined in clause 4.2.4.3 of TS 23.502 [3].

6.2.5	AF-based service parameter provisioning for Ranging/SL Positioning over control plane

For Ranging/SL Positioning service parameter provisioning (i.e. creating, updating and deleting), the procedure defined in clause 4.15.6.7 of TS 23.502 [3] is performed with the following considerations:

-	The AF in TS 23.502 [3] is considered as Ranging/SL Positioning Application Server in this specification.

-	Service Description indicates Ranging/SL Positioning service domain information.

-	Service Parameters include parameters for Ranging/SL Positioning service. The detailed information on the parameters is described in clause 5.X.

NOTE:	It is assumed that the Ranging/SL Positioning service domain information is set based on the Service Level Agreement with the operator.

# * * * * End of changes * * * *