3GPP TSG SA WG2 Meeting #156-e	S2-2304259

# Elbonia, April 17 – 21, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3981 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | R18 AIMLsys_KI1_23502 CR_SingleSO_for_QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In 3GPP SA2#155 AIML WID introduced a new NEF service (Nnef_MultiMemberAFsessionWithQoS) for multiple UEs complicating both specifications and AF interaction with 5GC more. From a system architecture perspective, a preferred way forward is that the minimum number of service operations are defined for similar requirements. It is proposed that Nnef_AFsessionWithQoS is leveraged in all cases. - Although the backend handling of the single vs. a list of UEs is different, nothing prevents extending Nnef_AFsessionWithQoS to support a list of UEs or UE identified by address. The introduction of the new NEF service complicates the specifications and the interaction of an AF with 5GC. It is proposed to enhance Nnef_AFsessionWithQoS to support a list of UEs. The same paradigm has been followed in the past, where for example, Nnef_TrafficInfluence supports a single UE or multiple UEs. - The EN “It is FFS whether to converge with GMEC to leverage common service interface to also support Multi-member AF session with required QoS for a list of UEs.” is proposed to be removed as the GMEC API is also proposed to be combined with the existing Nnef_AFsessionWithQoS SO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Nnef_AFsessionWithQoS is extended to support as additonal input a list of UEs and the procedure described in 4.15.6.Y. - Nnef_MultiMemberAFsessionWithQoS is removed. - EN about converging GMEC API is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Complicated specifications hard to maintain and AF/5GC interactions. - Specifications tend to implement a set of uncordinated WIDs rather than a system architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.15.6.13	AF session with required QoS for a list of UEs idendified by a list of UE IP addresses

##### 4.15.6.13.1	General Descriptions

This clause describes the procedure to request QoS and to perform QoS monitoring for the communication between a list of UEs, identified by the UE IP address(es) and the AF.

The NEF receives the AF session request with required QoS for a list of UEs identified by their IP addresses. The NEF then converts the AF session to the AF session with required QoS to each of the UE's serving PCF which then applies AF session with required QoS procedure as described in clause 4.15.6.6 "Setting up an AF session with required QoS procedure" to each UE, except for the TSCTSF related information.

The NEF receives the outcome of the individual AF session with required QoS corresponding to each UE's IP address, and consolidates them into single response before forwarding it to the AF.

When the AF subscribes to QoS Monitoring for the list of UEs identified by their respective IP addresses, the AF also provide a Consolidated MBR threshold that is stored in the NEF. The NEF subscribes to QoS Monitoring for each AF session with required QoS parameter(s) for each UE identified by its IP address. The list of the QoS Monitoring for Data Rate reports will be sent to the AF only if the aggregated bit rate exceed the Consolidated-MBR threshold. Otherwise, the group of QoS Monitoring for Data Rate report will be sent to the AF according to the reporting frequency.

The following describes the mapping of the 4 types of NEF service requests of the AF sessions and the N5 service request.

Table 4.15.6.13.1-1

| Nnef_AFSessionWithQoS Request | N5 request |
| --- | --- |
| Create | Requesting to allocate network resources to support a list of UEs' communication with the AF and the UEs are identified by the UE IP address(es). In addition, the AF can subscribe to be notified of QoS Monitoring reports. |
| Update | Updating (a) the list of UEs, (b) the requested QoS (c) the type of QoS monitoring for the list of QoS flows. More than one of the above operations can be requested at the same time. |
| Revoke | Deleting the establishment of the AF session for a list of UEs. |
| Notify | Notifying AF with the individual or aggregated QoS monitoring report for a specific list of AF sessions |

The Consolidated Maximum Bit Rate (Consolidated-MBR) threshold is described as follows:

-	The Consolidated-MBR threshold provides the upper bound of the aggregate bit rate across all GBR QoS Flows corresponding to the list of PDU Sessions of the UEs who participate in a list of transmissions concurrently (e.g. Federated Learning (FL) operation) with active user planes. The NEF aggregates bit rate across the set of selected QoS Flows against the Consolidated-MBR threshold.

In the case of the QoS monitoring for a list of QoS flows, Consolidated-MBR monitoring or concurrent reporting for the list of QoS flows' data rate or both could be supported at the same time by the NEF according to the AF request.

NOTE 1:	If Consolidated-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the bit rate shall be provided.

NOTE 2:	When the event report is for Consolidated-MBR monitoring, the QoS Flow data rate reporting for the list of UEs provided to the AF by the NEF only when the Consolidated-MBR threshold is exceeded.

NEXT CHANGE (2)

##### 4.15.6.13.2	Procedures for Creating AF session with required QoS for a list of UEs

Figure 4.15.6.13.2-1: Procedures for creating AF sessions with required QoS for a list of UEs

1.	The AF sends a request to reserve resources for AF sessions for a list of UEs using Nnef_AFsessionWithQoS_Create request message (a list of UE address, AF Identifier, Flow description information or External Application Identifier, QoS Reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), QoS parameter(s) to be measured, Reporting frequency, Target of reporting, optional an indication of local event notification, Consolidated-MBR Threshold, DNN, S-NSSAI) to the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20]. The AF may provide QoS parameter(s) to be measured as defined in clause 5.46 of TS 23.501 [2], Reporting frequency, Target of reporting, optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20]. To activate the Consolidated-MBR monitoring, the AF shall include the UL bitrate or DL bitrate as defined in clause 5.46 of TS 23.501 [2] in the QoS parameter(s) to be measured, Aggregated-MBR Threshold, and the Target of reporting set to the NEF.

Editor's note:	Proper reference to the UPF Event Exposure needs further clarification.

Editor's note:	Whether and how to leverage direct or indirect UPF event notification is FFS.

Editor's note:	Whether and how to activate the Consolidated-MBR monitoring during the create procedure is FFS.

Editor's note:	The name of the service is FFS.

2.	The NEF authorizes the AF request and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all steps (except step 5) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed. The NEF generates the Transaction Reference ID and associate the list of UE address received from the AF to the Transaction Reference ID.

3-4.	The NEF finds BSF serving the UE IP address(es) using NRF and then for each UE IP address, the NEF uses Nbsf_Management_Discovery service operation, providing the UE IP address to discover each PCF for the PDU Session.

Steps 5-10 applies for each UE address in the list of UE addresses.

5.	The NEF provides the UE address and the received parameters in step 1 to the PCF in the Npcf_PolicyAuthorization_Create request.

6.	Step 4 in Figure 4.15.6.6-1 applies except the PCF sending the response message to the NEF. The PCF generates authorized QoS Monitoring policy according to the information provided from the NEF in step 5 and provides PCC rules with the policy to the SMF as described in clause 6.1.3.21 of TS 23.503 [20]. The SMF configures the UPF to perform QoS Monitoring as described in clause 5.46 of TS 23.501 [2].

Editor's note:	Proper reference to the UPF Event Exposure needs further clarification.

7.	The PCF sends the Npcf_PolicyAuthorization_Create response message to the NEF.

8.	If the NEF receives the Npcf_PolicyAuthorization_Create response, the NEF sends a Nnef_AFsessionWithQoS_Create response message (Transaction Reference ID, Result for list of UE address) to the AF. Result for list of UE address includes whether the request is granted or not for every UE address in the list of UE address.

9.	Step 6 in Figure 4.15.6.6-1 applies.

10.	Step 7 in Figure 4.15.6.6-1 applies.

Editor's note:	Whether and how to leverage direct or indirect UPF event notification is FFS.11.	The NEF sends Nnef_AFsessionWithQoS_Notify message with the aggregated resource allocation status events to the AF.

12.	Based on the parameters received in step 5, the QoS Monitoring events are reported to the NEF.

Editor's note:	Proper reference to the UPF Event Exposure needs further clarification.

13.	The NEF sends Nnef_AFsessionWithQoS_Notify message with the aggregated QoS Monitoring events to the AF. If the NEF received Consolidated-MBR Threshold, the NEF aggregates the bitrate received from step 10 and if the aggregated bit rate exceeds the Consolidated-MBR Threshold, the NEF sends the event report for Consolidated-MBR Monitoring which includes the aggregated QoS Monitoring event for bitrates and the indication whether the aggregated bitrate is exceeded or not. If the Reporting Frequency received from the AF is periodic, the NEF periodically sends the event report for Consolidated-MBR Monitoring to the AF.

The AF may send Nnef_AFsessionWithQoS_Revoke request containing the Transaction Reference ID to the NEF in order to revoke all the AF sessions related to the Transaction Reference ID. The NEF authorizes the revoke request and triggers the Npcf_PolicyAuthorization_Delete and the Npcf_PolicyAuthorization_Unsubscribe operations for every UE in the list of UE address associated with the Transaction Reference ID.

NEXT CHANGE (3)

##### 4.15.6.13.3	Procedures for Updating AF session with required QoS for a list of UEs

Figure 4.15.6.13.3-1: AF session with required QoS for a list of UEs

1.	The AF which controls the operation of AF Sessions for a list of UEs invokes the Nnef_AFSessionWithQoS_ Update Request with a list of UEs' serving PCFs via the support of NEF to initiate the update the resource allocations and/or to direct the required QoS monitoring for a selected list of UEs to support a specific Application (e.g. FL operation).

When the AF requests to update the UE(s) in the list and/or to update either the resource allocation and/or to update the type of QoS monitoring, the AF initiates the Nnef_AFSessionWithQoS_Update request which includes the list of UE addresses, the AF Identifier, Transaction Reference ID, [Flow description information], [QoS Reference or individual QoS parameters], [Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20])]) to NEF for updating the reserved resources and/or to update the type of QoS monitoring. The Transaction Reference ID provided in the AF session with required QoS update request message is set to the Transaction Reference ID that was assigned by the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The Consolidated-MBR threshold for Consolidated-MBR monitoring (optional); in additional, the list of UE addresses subjected for Consolidated-MBR monitoring may be provided. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as specified in clause 6.1.3.22 of TS 23.503 [20].

NOTE 1:	If Consolidated-MBR Threshold is provided, the QoS parameter(s) to be measured indicating the Guaranteed Bitrate shall be provided.

NOTE 2:	When the AF request is for Consolidated-MBR monitoring is set for event trigger, the QoS Flow data rate reporting for the list of UEs provided to the AF by the NEF only when the Consolidated-MBR threshold is exceeded.

NOTE 3:	When the Consolidated-MBR threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subjected for Consolidated-MBR monitoring is also provided together with the Consolidate-MBR threshold, then such list has to be the subset of the list of UE addresses.

NOTE 4:	If AF wants to terminate the Consolidated-MBR monitoring for the list of AF sessions, AF does not include the Consolidate-MBR threshold in the AF request.

Editor's note:	Whether and how to leverage direct or indirect UPF event notification is FFS.

2.	NEF authorizes the AF request, and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all following steps are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

3.-5.	Once the UE(s)' serving PCF(s) are identified, the NEF converts the AF request into AF Session With QoS for a list of UEs. The UE's serving PCF authorizes the NEF request and responds to NEF with Npcf_PolicyAuthorization response according to the success or failure of the authorization.

When the AF requests for Nnef_AFSessionWithQoS_Update in order to update the UE(s) in the list for the resource allocation and/or to update the type of QoS monitoring, the NEF refers to the local context of the corresponding to the list of UEs and determines for which the new UE is to be added to the list, for which of the existing UE is to be removed from the list and/or for which of the existing UE is to be updated by invoking one or more of the following Npcf_PolicyAuthorization procedures:

a)	If the AF session update request involves a new UE is to be added to the list with resource allocation, after inquiring BSF via Nbsf_Management Discovery service operation to find out the address of the UE's serving PCF, the NEF initiates the Npcf_PolicyAuthorization_Create as described in clauses 4.15.6.6 and 5.2.6.9.4 excluding TSCTSF related procedure towards the PCF for the corresponding UE should be followed.

Editor's note:	Proper reference to the UPF Event Exposure needs further clarification.

Editor's note:	Whether and how to activate the Consolidated-MBR monitoring during the create procedure is FFS.

b)	If the AF session update request involves an existing UE is to be removed to the list, the NEF initiates the Npcf_PolicyAuthorization_Delete as described in clause 5.2.6.9.4 excluding TSCTSF related info towards the PCF for the corresponding UE should be followed.

Editor's note:	Whether and how to support Consolidate_MBR monitoring concurrently when requesting the resource for the new UE is FFS.

c)	If the AF session update request involves either the update the QoS resources or QoS monitoring or both for the existing UE in the group, the NEF initiates the Npcf_PolicyAuthorization_Update to the UE's serving PCF.

If the list of UE addresses subjected for the Consolidated-MBR monitoring is received from the AF, NEF updates the list of UEs subjected for Consolidated-MBR monitoring.

6.	NEF aggregates the authorization responses from the PCFs as described in step 6 of clause 4.15.6.6a and sends the AF with the Nnef_AFSessionWithQoS Update response corresponding to the earlier AF session request. The consolidated response for NEF provides specific success or failure response corresponding to each UE. NEF disassociates the UE IP address(es) to be removed and/or failed to be authorized by the PCF from the AF with the existing Transaction Reference ID.

7.	If NEF requested Npcf_PolicyAuthorization_ Update is authorized by the PCF as described in c) of step 5 above, the PCF processes the Npcf_PolicyAuthorization_Update request according to the actions described in step 4 of clause 4.15.6.6 for SM policy Association Modification procedure and/or QoS Monitoring as described in clause 6.1.3.21 of TS 23.503 [20]. The NEF shall send a Npcf_PolicyAuthorization_Subscribe message to the PCF to subscribe to notifications of Resource allocation status and may subscribe to other events described in clause 6.1.3.18 of TS 23.503 [20].

Editor's note:	Whether and how to leverage direct or indirect UPF event notification is FFS.

8.-9.	Dependent on the establishment of the transmission resources corresponding to the QoS update succeeded or failed, the UE's serving PCF sends Npcf_PolicyAuthorization_Notify message to the NEF about the Resource allocation status and NEF consolidates the notify messages from the respective UEs' serving PCFs before notifying the AF via Nnef_AFSessionWithQoS_Notify for the respective PCFs' outcome together with the AF Transaction Reference ID. NEF associates the UE IP address(es) which have been allocated with resources with the existing Transaction Reference ID.

10.	If the QoS Monitoring event for Data Rate Monitoring is indicated by the SMF, the UPF reports the data rate for the target QoS flow periodically to the NEF; otherwise, if the QoS Monitoring event for Data Rate Monitoring is revoked by the SMF, the UPF terminate the UPF reporting.

11.	If receiving the Data Rate Monitoring report of the QoS Monitoring event for specific QoS flow from UPFs, and the Consolidated-MBR threshold was provided by the prior Nnef_AFSessionWithQoS request, the NEF sums up the data rates of all the active QoS flows according to the list of UEs which are subjected to Consolidate-MBR monitoring through the duration of the Reporting Frequency. If the given Application AI/ML traffic is asymmetric, the bit rate aggregation is done per uplink and downlink separately; otherwise, the bit rate aggregation is done per either uplink or downlink. NEF compares the aggregated bit rate of the uplink or downlink against the Consolidated-MBR threshold for the uplink or downlink, accordingly.

12.	In the case of event reporting for Consolidated-MBR monitoring, only if the aggregate bit rate exceeds the Consolidated-MBR threshold, the NEF will notify the Application AI/ML AF via Nnef_AFSessionWithQoS_Notify for the collected UPF QoS monitoring reports together with the status of the Consolidated-MBR monitoring and the AF Transaction Reference ID; otherwise, the NEF notify the AF periodically with the collected UPF QoS monitoring reports together with the status of the of the Consolidate-MBR monitoring, if requested, and the AF Transaction Reference ID.

The UPF QoS Monitoring event for Data Rate Monitoring continues until AF terminates the Consolidated-MBR monitoring. The UPF continues to perform the QoS Monitoring event for Data Rate Monitoring for the given QoS flow to the NEF periodically corresponding to the Reporting Frequency as described in steps 11-12 above.

NEXT CHANGE (4)

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_TrafficInfluenceData | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |
| Nnef_PDTQPolicyNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_UEMemberSelectionAssistance | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_AF_request_for_QoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_DNAIMapping | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | UpdateNotify |  | AF |

NEXT CHANGE (5)

#### 5.2.6.9	Nnef_AFsessionWithQoS service

##### 5.2.6.9.1	General

See clause 4.15.6.6, 4.15.6.13.

This service is also used to support subscription and notification of QoS Monitoring for the QoS parameter(s) to be measured defined in clause 5.37 of TS 23.501 [2].

This service is also used to support subscription and notification of BAT offset for the AF that supports adjusting burst sending time based on RAN feedback, as described in clause 5.27.2.5 of TS 23.501 [2].

This service is also used to support the QoS resource allocation for a list of UEs.

This service is also used to support subscription and notification of QoS Monitoring event for Data Rate Monitoring for a list of QoS flows, as described in clause 5.2.26.

##### 5.2.6.9.2	Nnef_AFsessionWithQoS_Create service operation

Service operation name: Nnef_AFsessionWithQoS Create

Description: The consumer requests the network to provide a specific QoS for an AF session.

Inputs, Required: AF Identifier, Target UE identifier (UE address i.e. IP address or MAC address, or a list of UE addresses i.e. IP addresses), Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: Multi-Model Service ID, Multi-modal Service Requirements, time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured as defined in clause 5.37 of TS 23.501 [2], Consolidated-MBR Threshold, a list of UE addresses subjected to Consolidated-MBR monitoring, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available, flow direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Periodicity, Time domain, Survival Time, BAT Window or Capability for BAT adaptation, Periodicity Range, RT Latency Indication as described in clause 6.1.3.22 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description (Indicates RTP/SRTP header type used by the flow).

NOTE-1: If Consolidated-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE-2: When the AF request is for Consolidated-MBR monitoring is set for event reporting, the QoS Flow data rate reporting for the list of UEs provided to the AF by the NEF only when the Consolidated-MBR threshold is exceeded.

NOTE-3: When the Consolidated-MBR threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subjected for Consolidated-MBR monitoring is also provided, then such list has to be the subset of the list of UE addresses.

Outputs, Required: Transaction Reference ID, result (corresponding to individual UE(s) in the list, if a list of UE is targeted).

Output (optional): None.

##### 5.2.6.9.3	Nnef_AFsessionWithQoS_Notify service operation

Service operation name: Nnef_AFsessionWithQoS Notify

Description: NEF reports the QoS Flow level event(s) to the consumer.

Inputs, Required: Reports of the events as defined in clause 6.1.3.18 of TS 23.503 [20] ], Transaction Reference ID if a list of UE is targeted.

NOTE: When the event report is for Consolidated-MBR monitoring, the QoS Flow data rate reporting for the list of UEs provided to the AF by the NEF only when the Consolidated-MBR threshold is exceeded.

Inputs, Optional: When the event report is for QoS Monitoring, includes QoS Monitoring report for the QoS parameter(s) to be measured defined in clause 5.37 of TS 23.501 [2], e.g. UL packet delay, DL packet delay, or round trip packet delay of the single UP path or two UP paths in the case of redundant transmission, as defined in clause 5.33.3.2 of TS 23.501 [2]. When the AF has provided Capability for BAT Adaptation or BAT Window, can include BAT offset as described in clause 5.27.2.5 of TS 23.501 [2]. When the delivery report is for Consolidated-MBR Monitoring, includes Consolidated-MBR Monitoring report.

Outputs, Required: None.

Output (optional): None.

##### 5.2.6.9.4	Nnef_AFsessionWithQoS_Revoke service operation

Service operation name: Nnef_AFsessionWithQoS Revoke

Description: The consumer requests the network to revoke the AF session with requested QoS or the AF session with requested QoS including Alternative Service Requirements.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Transaction Reference ID, result (corresponding to individual UE(s) in the list, if a list of UE is targeted).

Output (optional): None.

##### 5.2.6.9.5	Nnef_AFsessionWithQoS_Update service operation

Service operation name: Nnef_AFsessionWithQoS Update

Description: The consumer requests the network to update the list of UEs (identified by IP address) of the list of AF session, the type of QoS monitoring operation,  the Service Requirement(s) and/or additional Alternative Service Requirement(s) for an AF session.

Inputs, Required: Transaction Reference ID, AF Identifier, a list of UE addresses (i.e. IP addresses), Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier.

NOTE: AF Identifier, a list of UE addresses (i.e. IP addresses), Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier are only required when a list of UEs is targeted..

Inputs, Optional: Flow description information (as described in clause 6.1.3.6 of TS 23.503 [20]), QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured as defined in clause 5.37 of TS 23.501 [2], Consolidated-MBR Threshold, a list of UE addresses subjected to Consolidate-MBR monitoring,Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], flow direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Periodicity, Time domain, Survival Time, BAT Window or Capability for BAT adaptation, Periodicity Range, PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description (Indicates RTP/SRTP header type used by the flow).

NOTE-1: If Consolidated-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE-2: When the AF request is for Consolidated-MBR monitoring is set for event reporting, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Group-MBR threshold is exceeded.

NOTE-3: When the Consolidated-MBR threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subjected for Consolidated-MBR monitoring is also provided, then such list has to be the subset of the list of UE addresses.

NOTE-4:  If AF wants to terminate the Consolidated-MBR monitoring, AF does not include the Consolidate-MBR threashold in the AF request.

Outputs, Required: Result, Transaction Reference ID if a list of UE is targeted.

Output (optional): Failure Cause in case of failure.

NEXT CHANGE (6)

END OF CHANGES