SA WG2 Meeting #S2-156E	S2-2304300

17 – 21 April 2023, Electronic, Elbonia	(revision of S2-230xxxx)

Source:	MediaTek Inc.

Title:	Discussion on Equivalent SNPN used by UE for Localized Services

Document for:	Discussion

Agenda Item: 	9.24.2

Work Item / Release:	eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes to discuss how UE uses the Equivalent SNPNs allocated by the serving SNPN

- Discussion

The Equivalent SNPN list is allocated by a serving SNPN to which the UE is currently registered to and each SNPN identity in the Equivalent SNPN is as equivalent as the currently serving SNPN from the UE point of view.

1.1	The usage of Equivalent SNPN by UE

In Rel-18, for automatic network selection without considering the Localized Services by the UE, the UE shall select and attempt registration on available and allowable SNPNs in the following as captured below

- the SNPN the UE was last registered with (if available) or the equivalent SNPN (if available); - the subscribed SNPN, which is identified by the PLMN ID and NID for which the UE has SUPI and credentials.; - If the UEs supports access to an SNPN using credentials from a Credentials Holder then the UE continues by selecting and attempting registration on available and allowable SNPNs which broadcast the indication that access using credentials from a Credentials Holder is supported in the following order: - SNPNs in the user controlled prioritized list of preferred SNPNs (in priority order); - SNPNs in the Credentials Holder controlled prioritized list of preferred SNPNs (in priority order); - SNPNs, which additionally broadcast a GIN contained in the Credentials Holder controlled prioritized list of preferred GINs (in priority order); - SNPNs, which additionally broadcast an indication that the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, i.e. the broadcasted NID or GIN is not present in the Credentials Holder controlled prioritized lists of preferred SNPNs/GINs in the UE.

Based on the first bullet, the UE may select and attempt registration on an Equivalent SNPN included in the Equivalent SNPN list if available.

However, if UE needs to consider SNPN that provides access for the Localized Services, the UE needs to consider whether an Equivalent SNPN is applied the same validity information (time and/or location) as serving SNPN as described in below Editor’s note:.

Editor’s note:	Whether the Equivalent SNPN(s) has same validity information as the SNPN that providing access for Localized Services the UE was last registered is for FFS

To resolve the above EN, the simple way is to enable the UE to consider the validity information of the Equivalent SNPN is same as the serving SNPN. Therefore, no matter that the Equivalent SNPN is in Credentials Holder with/without the validity information. The UE only considers the Equivalent SNPN is as equivalent as the serving SNPN since this is allocated by the serving SNPN as equivalent. The validity information of the serving SNPN is applicable to the Equivalent SNPN.

2	Proposal

Based on the above proposal, when the UE is selecting an Equivalent SNPN, the UE shall consider the Equivalent SNPN has same validity information as the serving SNPN.

Please see CR in S2-2304301 for the normative text.