3GPP TSG-SA WG2 Meeting #156-e	S2-2304308

# 17 – 21 April, 2023, Electronic Meeting					                       (revision of S2-23XXXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0251 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription management for Layer-3 Remote UE multi-path communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Address remaining ENs in clauses 4.3.5 and 5.7 regarding subscription management for Layer-3 Remote UE's multi-path communication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the ENs in clauses 4.3.5 and 5.7 given that Layer-3 Remote UE does not require subscription management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The remaining ENs can not be addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.5, 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

### 4.3.5	UDM

In addition to the functions defined in TS 23.501 [4], the UDM performs the following functions:

-	Subscription management for 5G ProSe Direct Discovery and Communication.

-	Subscription management for 5G ProSe UE-to-Network Relay Discovery and Communication.

-	Subscription management for multi-path transmission via direct Uu path and via 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE.

-	Subscription management for 5G ProSe UE-to-UE Relay Discovery and Communication.

**** NEXT CHANGE ****

## 5.7	Subscription to 5G ProSe

The subscription information in the UDM contains information to give the user permission to use 5G ProSe.

At any time, the operator can amend or remove the ProSe UE subscription rights from subscription information in the UDM, or to revoke the user's permission to use 5G ProSe.

The following subscription information is defined for 5G ProSe:

-	subscription for open 5G ProSe Direct Discovery for NR PC5:

-	open 5G ProSe Direct Discovery Model A.

-	subscription for restricted 5G ProSe Direct Discovery for NR PC5:

-	restricted 5G ProSe Direct Discovery Model A;

-	restricted 5G ProSe Direct Discovery Model A with application-controlled extension;

-	restricted 5G ProSe Direct Discovery Model A with "on demand" announcing;

-	restricted 5G ProSe Direct Discovery Model B.

-	subscription for Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication for NR PC5.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-2 UE-to-Network Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-3 UE-to-Network Relay.

-	subscription for 5G ProSe Layer-2 Remote UE access via 5G ProSe Layer-2 UE-to-Network Relay.

-	subscription for 5G ProSe Layer-3 Remote UE access via 5G ProSe Layer-3 UE-to-Network Relay.

-	subscription for multi-path transmission via direct Uu path and via 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE.

-	UE-PC5-AMBR for NR PC5.

-	PC5 QoS parameters as defined in clause 5.6.1 used by NG-RAN.

-	the list of the PLMNs authorized for 5G ProSe services, including:

-	the list of the PLMNs where the UE is authorised for open 5G Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted 5G ProSe Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted 5G ProSe Direct Discovery Model B, i.e. to perform Discoverer operation or Discoveree operation or both.

-	the list of the PLMNs where the UE is authorised to perform Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication for NR PC5.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-3 UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 Remote UE.

**** END OF CHANGES ****