3GPP SA WG2 Meeting #156e	S2- 2304313

Electronic, 17-21 April 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3990 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selection of Common DNAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 23.700-48 concluded that the multiple SMF case should be supported, and the common DNAI selection should be independent of one single PDU session. For that purpose, coordination among multiple SMFs is specified under TS 23.548 clause 6.2.3.2.7. Following changes are needed for the alignment of the specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.3.6.2 is updated to cover the multiple SMF case for the selection of common DNAI as concluded in TR 23.700-48. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the proposed changes are not made, the solution of common DNAI selection is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start change ***

#### 4.3.6.2	Processing AF requests to influence traffic routing and/or service function chaining for Sessions not identified by an UE address

Figure 4.3.6.2-1: Processing AF requests to influence traffic routing for Sessions not identified by an UE address

NOTE 1:	The 5GC functions used in this scenario are assumed to all belong to the same PLMN (HPLMN in non-roaming case or VPLMN in the case of a PDU Session in LBO mode).

NOTE 2:	Nnef_TrafficInfluence_Create or Nnef_TrafficInfluence_Update or Nnef_TrafficInfluence_Delete service operations invoked from an AF located in the HPLMN for local breakout and home routed roaming scenarios are not supported.

0a.	The PCF(s) subscribe to modifications of AF requests (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = S-NSSAI and/or DNN and/or Internal Group Identifier or SUPI) from the UDR.

0b-1.  V-SMF supporting HR-SBO subscribes to notification of AF request by invoking Nnef_TrafficInfluenceData_Subscribe service (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = S-NSSAI and/or DNN and/or Internal Group Identifier and/or UE IP or SUPI) from V-NEF.

Editor's note:  For HR-SBO case, whether the AF influence procedure in VPLMN should inform HPLMN is FFS.

0b-2.  The V-NEF subscribes to notification of AF request from V-UDR.

1.	To create a new request, the AF invokes a Nnef_TrafficInfluence_Create service operation. The content of this service operation (AF request) is defined in clause 5.2.6.7. The request contains also an AF Transaction Id. If it subscribes to events related with PDU Sessions the AF indicates also where it desires to receive the corresponding notifications (AF notification reporting information).

To update or remove an existing request, the AF invokes a Nnef_TrafficInfluence_Update or Nnef_TrafficInfluence_Delete service operation providing the corresponding AF Transaction Id.

The Nnef_TrafficInfluence_Create (initiated by target AF) or Nnef_TrafficInfluence_Update (initiated by source AF or target AF) service operation may be used for the case of AF instance change. If Nnef_TrafficInfluence_Update service operation is invoked, the NEF is required to update the subscription resource. The Nnef_TrafficInfluence_Update service operation may include an updated notification target address. The updated subscription resource is used by the target AF.

NOTE 3:	If the source AF transfers the application context to the target AF, then target AF may create new subscription via Nnef_TrafficInfluence_Create operation or update existing subscription via Nnef_TrafficInfluence_Update. However, whether and how the application context transfer is done is out of this specification.

2.	The AF sends its request to the NEF. If the request is sent directly from the AF to the PCF, the AF reaches the PCF selected for the existing PDU Session by configuration or by invoking Nbsf_management_Discovery service.

The NEF ensures the necessary authorization control, including throttling of AF requests and, as described in clause 4.3.6.1, mapping from the information provided by the AF into information needed by the 5GC.

3.	(in the case of Nnef_TrafficInfluence_Create or Update): The NEF stores the AF request information in the UDR (Data Set = Application Data; Data Subset, Data Key = AF Transaction Internal ID, S-NSSAI and DNN and/or Internal Group Identifier(s) and/or Subscriber Category(s) or SUPI). The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing or AF traffic influence request information for service function chaining, as described in Table 5.2.12.2.1-1.

NOTE 4:	Both the AF Transaction Internal ID and, S-NSSAI and DNN and/or Internal Group Identifier(s) and/or Subscriber Category(s) or SUPI are regarded as Data Key when the AF request information are stored into the UDR, see Table 5.2.12.2.1-1. The Subscriber Category(s) is determined by NEF as described in clause 4.3.6.1.

(in the case of Nnef_TrafficInfluence_delete): The NEF deletes the AF requirements in the UDR (Data Set = Application Data; Data Subset, Data Key = AF Transaction Internal ID). The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing or AF traffic influence request information for service function chaining.

The NEF responds to the AF.

4a.	The PCF(s) that have subscribed to modifications of AF requests receive(s) a Nudr_DM_Notify notification of data change from the UDR. The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing or AF traffic influence request information for service function chaining.

4b.	The V-UDR notifies the subscribed V-NEF of the AF traffic influence request information.

5a.	The PCF determines if existing PDU Sessions are potentially impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding new policy information about the PDU Session by invoking Npcf_SMPolicyControl_UpdateNotify service operation as described in steps 5 and 6 in clause 4.16.5.

The PCF validates whether the SFC policy identifier (if available) corresponds to an authorized SFC for the AF based on local configuration. If the validation has succeeded the PCF maps the SFC policy identifier to the corresponding Traffic Steering Policy identifier (i.e. TSP ID) provided within a PCC rule. The PCF also includes the Metadata when provided by the AF in the PCC rule.

The PCF includes the Traffic Steering Policy ID(s) for AF influenced Traffic Steering Enforcement Control information and/or N6-LAN Traffic Steering Enforcement Control information in the relevant PCC rule as defined in clause 6.3.1 of TS 23.503 [20].

If the AF request includes a notification reporting request for UP path change, the PCF includes in the PCC rule(s) the information required for reporting the event, including the Notification Target Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

If the AF request includes an EAS Correlation indication or indication of traffic correlation, PCF includes in the PCC rule(s) an EAS Correlation indication or indication of traffic correlation, and a Traffic Correlation ID corresponding to a set of UEs that AF request aims at, also if AF request includes a common EAS IP address or common DNAI for a set of UEs, PCC rule includes the common EAS IP address or common DNAI.

To select the common DNAI for a collection of PDU Sessions associated with the same Traffic Correlation ID, the UE locations, network topology, or the current Analytics from NWDAF for service experience between the gNB and candidate UPFs can be used by the 5GC. Whenever another UE in the same set of UEs establishes a PDU Session with a PCC rule that matches with the same Traffic Correlation ID, or initiates a DNS query to the FQDN as received in the PCC Rule (when EAS discovery procedure in TS 23.548 [74] is used), the SMF is notified via the PCC Rule for the given Traffic Correlation ID.

A new common DNAI may be determined from the candidate DNAI(s) for the corresponding Traffic Correlation ID and the other SMF(s) with an active PDU session, serving a set of UE(s), are notified for a new common DNAI as explained in clause 6.2.3.2.7 in TS 23.548 [74]. This may be done e.g. one of the existing PDU Sessions is released, or one or more of the UEs in the set of UEs are moving. A threshold can be set to avoid frequent notifications.

In the case of AF influence on traffic routing, the PCF may, optionally, use service experience analytics per UP path, as defined in clause 6.4.3 of TS 23.288 [50], to provide an updated list of DNAI(s) to the SMF.

The PCF may use the "Subscriber categories" as defined in "PDU Session policy control subscription information" in table 6.2-2 of TS 23.503 [20] to determine whether the PDU Session is impacted by the AF request.

5b.	The V-NEF notifies the subscribed V-SMF of the AF traffic influence request information.

6.	When the updated policy information about the PDU Session is received from the PCF, the SMF may take appropriate actions to reconfigure the User plane of the PDU Session.

In the case of AF influence on traffic routing, the SMF may consider service experience analytics and/or DN Performance analytics per UP path (i.e. including UPF and/or DNAI and/or AS instance) as defined in clauses 6.4.3 and 6.14.3, respectively, of TS 23.288 [50] before taking such actions. Examples of actions are:

-	Determining a target DNAI,

-  Determining if a common DNAI needs to be used as a target DNAI.

-  Based on the target DNAI, adding, replacing or removing a UPF in the data path to e.g. act as an UL CL or a Branching Point e.g. as described in clause 4.3.5.

-	Allocate a new Prefix to the UE (when IPv6 multi-Homing applies).

-	Updating the UPF in the target DNAI/Common DNAI with new traffic steering rules.

-	Subscribe to notifications from the AMF for an Area of Interest via Namf_EventExposure_Subscribe service operation.

-	Determining whether to relocate PSA UPF considering the user plane latency requirements provided by the AF (see clause 6.3.6 of TS 23.548 [74]).

When the updated policy information about the PDU Session is received from the PCF, the SMF may take appropriate actions to assist the EAS discovery and re-discovery for PDU Session with Session Breakout connectivity model such as:

-	Retrieve the EAS deployment information as defined in clause 6.2.3.4.1 of TS 23.548 [74].

-	Providing DNS message handling rule to forward DNS messages of the UE and/or report when detecting DNS messages as defined in clause 6.2.3.2.2 of TS 23.548 [74].

In the case of N6-LAN Traffic Steering Control, the SMF may take appropriate actions to support the N6-LAN traffic steering control for SFC:

-	Provide N6-LAN traffic steering control parameters to UPF as described in clause 5.6.16 of TS 23.501 [2].

7.	The SMF may decide whether it is required to send the target DNAI or the common DNAI to the AMF for triggering SMF/I-SMF (re)selection and then inform the target DNAI information or the common DNAI information for the current PDU session or for the next PDU session to AMF via Nsmf_PDUSession_SMContextStatusNotify service operation.

*** End of change ***