3GPP TSG-WG SA2 Meeting #156-e	S2-2304322

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0215 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the procedures of KI#2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Documents submitted to SA2#155 meeting (S2-2303402, S2-2303107) tried to address the issue related to N3mb tunnel management. However, due to the reason that 1) Some updates also touches the Rel-17 and it would be better to submit the Rel-17/18 together, and 2) It was not determined the exact parts to be updated. i.e., whether the following sentence originally in step 5 should be moved to step 6 or step 4: “If NG-RAN prefers to use N3mb unicast transport (or if the LL SSM is not available in NG-RAN) between the NG-RAN and MB-UPF, NG-RAN provides its N3mb DL Tunnel Info.” In this document, it is proposed to finalize the N3mb tunnel management issue for KI#2, and update the specification in 7.3.1a and 7.3.2a accordingly under the following consideration: 1. If the broadcast session context needs to be updated, e.g., N3mb tunnel info, then update for step 4 is needed. 2. If the NGAP message is to be udpated, then the associating udpate for step 6 is needed. Note that the changes on clause 7.3.1 and 7.3.2 should be addressed in the other document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description of clause 7.3.1a and 7.3.2a to finalize the description for N3mb tunnel management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The N3mb tunnel management for KI#2 is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1a, 7.3.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

7.3.1a	MBS Session Start for resource sharing across multiple broadcast MBS Sessions during network sharing

To support resource sharing across multiple broadcast MBS Sessions during network sharing (see clause 6.17), in addition to clause 7.3.1, the following enhancement applies:

-	Step 2: The MB-SMF includes Associated Session ID in the N2 SM information in Namf_MBSBroadcast_ContextCreate.

-	Step 4: A shared NG RAN node identifies associated MBS sessions via a configured association of TMGIs or via the same Associated Session Identifier being received for the associated sessions. The associated MBS sessions shares radio resource configuration based on the existing Broadcast MBS Session context. If the NG-RAN determines there is already an existing broadcast MBS session delivering the same content, the shared NG-RAN node may decide to not allocate N3mb DL Tunnel Info. Procedure continues at step 6.

NOTE 1:	It is up to NG-RAN node implementation to decide whether to allocate the N3mb DL Tunnel.

-	Step 5: If the NG-RAN determines there is already an existing broadcast MBS session delivering the same content based on the Associated Session ID, the NG-RAN may skip joining the multicast group based on operator policy, i.e., step 5 can be skipped.

-	Step 6:	 For N3mb unicast transport, if the NG-RAN does not allocate N3mb DL Tunnel Info in step 4, the N3mb DL Tunnel Info is not present in MBS Session Resource Setup Response.

-	Step 8: If the N3mb unicast transport is used but N3mb DL Tunnel Info is not present, step 8 is skipped.

-	Step 9: If the NG-RAN determines that radio resource has been allocated for another broadcast MBS Session delivering the same content based on the Associated Session Identifier or the configured association of TMGIs, the NG-RAN advertises the TMGI of the broadcast MBS session, and reuses such already allocated radio resource for the MBS data transmission for the broadcast MBS session.

NOTE 2:	The 5G QoS Profile values for the broadcast MBS sessions are assumed to be pre-agreed amongst the multiple operators participating in the network sharing, and are the same irrespective of the PLMN from which the MBS Session Start is sent.

-	Step 10 ~ step 12:

The handling is the same as steps 5, 6, 7 and 8.

-	Step 14: If the user plane of a broadcast MBS session from its MB-UPF to the shared NG-RAN is not established, the NG-RAN will not receive the MBS packets from the MB-UPF.

-	Step 15: If the NG-RAN receives multiple DL media streams for the same content represented by the same Associated Session ID via multiple CNs, the NG-RAN should deliver only one DL media stream.

# * * * * Second change * * * *

7.3.2a	MBS Session Release for resource sharing across multiple broadcast MBS Sessions during network sharing

To support resource sharing across multiple broadcast MBS Sessions, in addition to clause 7.3.2, the following enhancement applies:

-	Step 6:

-	If the NG-RAN skipped joining the multicast group (LL SSM) in clause 7.3.1a, the NG-RAN does not send a Leave message (LL SSM),

-	Otherwise, if the NG-RAN joined the multicast group (LL SSM) previously, and if the NG-RAN determines there are other associated broadcast MBS Sessions based on the Associated Session Identifier, the NG-RAN may trigger procedure of Transport change for resource sharing across broadcast MBS Sessions in network sharing as specified in clause 7.3.7.

-	Step 7:

-	If the DL N3mb Tunnel Info for the broadcast MBS Session was not allocated previously in clause 7.3.1a, the NG-RAN does not have DL N3mb Tunnel Info to release.

-	Otherwise, if the DL N3mb Tunnel Info for the broadcast MBS Session was allocated previously, and if the NG-RAN determines there are other associated broadcast MBS Sessions based on the Associated Session Identifier, the NG-RAN may trigger the procedure of Transport change for resource sharing across broadcast MBS Sessions during network sharing as specified in clause 7.3.7.

# * * * * End of changes * * * *