3GPP TSG-WG SA2 Meeting #156e	S2-2304340

Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

# Title:	[DRAFT] LS reply on the use of PEI during an emergency PDU session

# Response to:	LS (S2-233953/R2-2302302) on LS on the use of PEI during an emergency PDU session from RAN2

# Release:	Rel-17

# Work Item:	NR_UE_pow_sav_enh-Core

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:	CT1

Contact Person:

Name:	Michele Zarri

Tel. Number:

E-mail Address:	Michele DOT Zarri AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank RAN2 for highlighting the current behaviour of the UE with regards to the use or PEI and UE ID based subgrouping during an emergency PDU session.

It is SA2 understanding that failing to disable the power saving mechanism while the UE has an active emergency PDU session is not aligned with the Stage 2 requirement expressed in TS 23.501, clause 5.12.4 and TS 24.501 clause 5.3.25 which read as follows:

TS 23.501, clause 5.12.4

" When the UE has an active emergency PDU Session:

-	The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

"

TS 24.501 clause 5.3.25

"When an emergency PDU session is successfully established over 3GPP access after the UE received the Negotiated PEIPS assistance information IE during the last registration procedure, the UE and the AMF shall not use PEIPS assistance information until the successful completion of the PDU session release procedure of the emergency PDU;

"

Indeed SA2 believe that any strategy for the reduction of power consumption that may negatively affect the time to connect an emergency call or emergency call back should not be used while the UE has an active emergency PDU session and is applicable to UE ID based subgrouping too.

2. Actions:

To RAN2, RAN3 groups.

ACTION: 	SA2 asks RAN2 and RAN3 to ensure that the behaviour of a UE with an active emergency PDU session specifically with regards to the use of PEI and UE ID based sub grouping and any other power saving mechanisms in general is aligned with the stage 2 requirement, i.e. these mechanisms should be uniformly disabled when the UE has an active emergency PDU session.

To meet these requirements, SA2 requests RAN3 to introduce an indication to disable/re-enable PEI in the NG paging message for CM-IDLE paging (Paging on NG) and for CM-CONNECTED paging (RAN paging on Xn).

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #157		May 22 – 26, 2023				Berlin, DE

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE