3GPP TSG-SA WG2#156E	S2-2304342

Electronic meeting, 17-21 April 2023	(was S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	Resolution of open issues for out-of-coverage reporting for Discontinuous Coverage

Document for:	Discussion/Approval

Agenda Item:	9.2.2

Work Item / Release:	5GSAT_Ph2/Rel.18

Abstract of the contribution: Proposes resolution of ENs for out of coverage reporting for satellite discontinuous coverage.

# Discussion

In SA2#155 a baseline was agreed for the procedure of the reporting UE out-of-coverage period in NAS and AMF reporting expected unavailability duration to the UE. Several open issues were documented and captured with related ENs. This CR is aiming to resolve the open issues related to out-of-coverage reporting and close the related ENs.

- Editor's note:	Capability negotiation between UE and network for the features introduced for discontinuous network coverage for satellite access is FFS.

It is proposed that there are two distinct separate features supported optionally by UE and AMF:

Feature 1: UE is able to determine its own out-of-coverage period

this feature is identical to SUECR and is using SUECR capability (from UE and AMF). No new change and capability is needed

Feature 2: UE is NOT able to determine its own out-of-coverage period but is able to determine that is about to lose coverage and AMF is able to assist it

this feature is new for satellite Discontinuous Coverage reporting and has its own capability from UE and AMF

Following up from that the " UE 5GMM Core Network Capabilities" have to be structured as follows:

1) Unavailability Period reporting (existing one defined in SUECR)

2) Support for discontinuous coverage without UE being able to determine its own UE out-of-coverage period.

Same applies to AMF support:

For Feature 1), the AMF signals the "Unavailability Period Supported" indication in Registration Accept message as defined in TS 23.501 cl. 5.4.1.4.

For Feature 2) there is a need for new capability from AMF, "AMF assistance to UE that cannot determine its out-of-coverage period"

- Editor's note:	It is FFS whether the AMF can provide an expected unavailability duration in the Registration Accept also if the UE provided an out-of-coverage time the Registration Request.

Assuming the format of satellite coverage availability information that is sent to the UE is standardised and therefore is reliable (from the perspective of PLMN), we don’t see the need for AMF to provide expected unavailability duration in the Registration Accept if the UE supports "feature 1" above and reports its own out-of-coverage period.

Clause 5.4.13.1 states:

To avoid signalling overload, the Mobility Registration Update procedure as described above shall be triggered during a time range before the UE loses coverage, as many UEs may lose coverage at the same time.

However, there is currently no definition or support for the time range. This is possible by extending Overload control in clause 5.4.13.5. This results in sending the Mobility Registration Update sometime before an Unavailability period would start which requires adding a Start time to clause 5.4.1.4. The presence of a Start time allows use of a periodic or mobility Registration Update to send an unavailability period to the AMF which can reduce signalling.

It is proposed to replace the existing text in clause 5.4.13.2 as proposed below:

A UE may use satellite coverage availability information for satellite access to support discontinuous coverage operations. Satellite coverage availability information can be provided to a UE by an external server via a PDU Session or SMS as defined in CT1 specifications.

Editor's Note: The specification that will define the protocol and format of satellite coverage availability information via PDU session or SMS will be defined by CT1.

# 2.	Text proposal

It is proposed to agree the changes vs. TS 23.501 proposed in S2-2304208. Similar changes will apply to TS 23.401 for EPS.