3GPP TSG-SA WG2 Meeting #156	S2-2304347

# Elbonia, April 17 - 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0093 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UAV authorization by USS via Uu interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing clarity on when the UAV can be authorized by the USS for direct C2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text added to clarify, when the UAV is in Uu access can be authorized by USS Changed the heading for clause 5.4.3 to reflect specific 3gpp capable UAV authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion about USS authorization when UE do not support 3gpp network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First change ****

## 5.4	Direct C2 Communication

### 5.4.1	General

A UAV that supports Direct C2 Communication may establish direct PC5 link with a UAV-C. Only NR PC5 is supported for Direct C2 Communication. The UAV that is engaged in Direct C2 Communication may or may not be capable of connection with 3GPP network. If the UAV is capable of 3GPP network connection, the UAV is authorized by the USS for establishing Direct C2 Communication with the UAV-C. The UAV-C with which the UAV performs Direct C2 Communication may be pre-configured in the UAV or provided by the network as described in clause 5.4.3. The UAV may be preconfigured with the A2X service type for direct C2 Communication, direct C2 pairing information (e.g. Application Layer ID of the UAV-C), a default Destination Layer-2 ID for initial signalling to establish unicast connection, authorization policy for direct C2 Communication.

**** Second change ****

### 5.4.3	Procedure for C2 authorization by the USS for using the A2X Direct C2 Communication service (3gpp capable UAV)

If the UAV is capable of 3GPP network connection, the UAV performs the C2 authorization as described in clause 5.2.5. If the UAV supports Direct C2 Communication and intends to request C2 authorization for Direct C2 Communication to the USS, it shall include an indication for Direct C2 Communication authorization in the authorization request.

In 5GS, the following procedures are used to request C2 authorization for Direct C2 Communication to the USS:

-	The procedure for C2 Authorization request during UUAA-SM procedure in 5GS as specified in clause 5.2.5.2.1 with the following enhancements:

-	In step 0: When the UAV needs to establish a direct PC5 link required for connectivity to UAV-C (i.e. Direct C2 Communication), the C2 Aviation Payload sent by the UAV includes an indication that the authorization is also for Direct C2 Communication. In addition, the UAV includes the direct C2 pairing information (if available) in the C2 Aviation Payload.

-	In step 4: If the authorization request for Direct C2 Communication was included in step 0 and the C2 authorization is successful, the USS may include direct C2 pairing information containing the UAV-C's Application Layer ID in the C2 Authorization Payload which is further forwarded to the UE in the Naf_Authentication_AuthenticateAuthorize response.

-	The procedure for UE initiated PDU Session Modification for C2 Communication as specified in clause 5.2.5.2.2 with the following enhancements:

-	In step 2: When the UAV needs to establish a direct PC5 link required for connectivity to UAV-C (i.e. Direct C2 Communication), the C2 Aviation Payload sent by the UAV includes an indication that the authorization is also for Direct C2 Communication. In addition, the UAV includes the direct C2 pairing information (if available) in the C2 Aviation Payload.

-	In step 7: If the authorization request for Direct C2 Communication was included in step 2 and the C2 authorization is successful, the USS may include direct C2 pairing information containing the UAV-C's Application Layer ID in the C2 Authorization Payload which is further forwarded to the UE in the Naf_Authentication_AuthenticateAuthorize response.

-	The procedure for UE initiated PDU Session Establishment for C2 Communication as specified in clause 5.2.5.2.3 with the following enhancements:

-	In step 1: When the UAV needs to establish a direct PC5 link required for connectivity to UAV-C (i.e. Direct C2 Communication), the C2 Aviation Payload sent by the UAV includes an indication that the authorization is also for Direct C2 Communication. In addition, the UAV includes the direct C2 pairing information (if available) in the C2 Aviation Payload.

-	In step 4: If the authorization request for Direct C2 Communication was included in step 1 and the C2 authorization is successful, the USS may include direct C2 pairing information containing the UAV-C's Application Layer ID in the C2 Authorization Payload which is further forwarded to the UE in the Naf_Authentication_AuthenticateAuthorize response.

In EPS, the following procedure is used to request C2 authorization for Direct C2 Communication to the USS:

-	The procedure for UE requested PDN connectivity for C2 authorization as specified in clause 5.2.5.3.1 with the following enhancements:

-	In step 1: When the UAV needs to establish a direct PC5 link required for connectivity to UAV-C (i.e. Direct C2 Communication), the C2 Aviation Payload sent by the UAV includes an indication that the authorization is also for Direct C2 Communication. In addition, the UAV includes the direct C2 pairing information (if available) in the C2 Aviation Payload.

-	In step 4: If the authorization request for Direct C2 Communication was included in step 1 and the C2 authorization is successful, the USS may include direct C2 pairing information containing the UAV-C's Application Layer ID in the C2 Authorization Payload which is further forwarded to the UE in the Naf_Authentication_AuthenticateAuthorize response.

**** End of change ****