3GPP TSG-SA WG2 Meeting #156	S2-2304348

# Elbonia, April 17 - 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0094 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of cross-rat authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Cross-rat authorization is not needed, as AMF will provide the per RAT type authorization to RAN. It is up to RAN to allocate resources for network schedule mode per PC5 RAT. It is mentioned the cross-RAT authorization is not provided by AMF to RAN in any new IE and it is implicit, so there is no need to mention that in all other sections as it adds to confusion” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed cross-rat authorization and aligned text for NGAP message and UE context in AMF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading text on AMF behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11.2, 5.11.3, 5.11.4, 5.11.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First change ****

### 5.11.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [3] with the following additions:

-	The UE includes the PC5 Capability for A2X (i.e. LTE PC5 only, NR PC5 only, both LTE and NR PC5) as part of the "5GMM capability" in the Registration Request message. The AMF stores this information for A2X operation. The PC5 Capability for A2X indicates whether the UE is capable of A2X communication over PC5 reference point and which specific PC5 RAT(s) it supports.

-	The AMF obtains the A2X Subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [3].

-	The AMF determines whether the UE is authorized to use A2X communication over PC5 reference point based on UE's PC5 Capability for A2X and the subscription data related to A2X service authorization information (i.e. "A2X services authorized" indication per PC5 RAT and UE-PC5-AMBR per PC5 RAT) received from UDM, and stores the A2X service authorization information as part of the UE context.

-	The PCF provides the PC5 QoS parameters to AMF, and the AMF stores them in the UE context.

-	If the UE is PC5 capable for A2X, and the UE is authorized to use A2X communication over PC5 reference point based on the subscription data, then the AMF shall include the following information in the NGAP message sent to NG-RAN:

-	a "A2X services authorized" indication per PC5 RAT, indicating the UE is authorized to use A2X communication over PC5 reference point.

-	UE-PC5-AMBR per PC5 RAT, used by NG-RAN for the resource management of UE's PC5 transmission for A2X services in network scheduled mode.

-	the PC5 QoS parameters, used by the NG-RAN for the resource management of UE's PC5 transmission for A2X services in network scheduled mode.

-	If the UE is authorized to use A2X communication over PC5 reference point, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the NAS signalling relies on the decision of NG-RAN, as specified in TS 23.502 [3].

### 5.11.3	Service Request procedure

The Service Request procedure for UE in CM-IDLE state is performed as defined in TS 23.502 [3] with the following additions:

-	If the UE is PC5 capable for A2X, and the UE is authorized to use A2X communication over PC5 reference point, then the AMF shall include a "A2X services authorized" indication per PC5 RAT in the NGAP message.

-	The AMF includes the UE-PC5-AMBR for A2X per PC5 RAT in the NGAP message to the NG-RAN as part of the UE context and NG-RAN may use in resource management of UE's PC5 transmission for A2X services in network scheduled mode.

-	The AMF sends the PC5 QoS parameters for A2X to NG-RAN via N2 signalling. The PC5 QoS parameters may be stored in the UE context after the registration procedure. If the UE is authorized to use A2X communication over PC5 reference point but AMF does not have PC5 QoS parameters available, the AMF fetches the PC5 QoS parameters from the PCF.

### 5.11.4	Subscriber Data Update Notification to AMF

The procedure of Subscriber Data Update Notification to AMF is performed as defined in TS 23.502 [3] with the following additions:

-	The Nudm_SDM_Notification service operation may contain the "A2X services authorized" indication per PC5 RAT and/or the UE-PC5-AMBR;

-	The AMF updates the UE Context with the above new A2X subscription data.

-	When the AMF updates UE context stored at NG-RAN, the UE context contains the A2X subscription data.

**** Second change ****

**** End of change ****