3GPP TSG-WG SA2 Meeting #156E 	S2-2304377

April 17 – 21, 2023, Elbonia	(revision of S2-230xxxx)

Source:	Sony

Title:	Update the feature Network assisted SL positioning without NAS connection.

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper address some open Editor’s notes and clarifies the feature Deferred MT-LR.

# 1. Introduction/Discussion

The new procedure in 6.5.2.1 clarifies open issues captured in EN.

As the feature is quite different for the two cases (MO-LR and deferred MT-LR) it is proposed to clearly separate them in the feature description.

# 2. Text Proposal

It is proposed to update clause 5.5.3 with the following changes.

# * * * * First change* * * *

### 5.5.3	Network assisted SL positioning without NAS connection

When Target UE cannot establish the NAS connection with AMF due to the Target UE being out of coverage, for 5GC-MO-LR or deferred 5GC-MT-LR, the following principles are applied:

For MO-LR:

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE may transmit its ranging measurements/results to the Located UE(s).

-	Located UE(s) may report the ranging/SL positioning measurement result to the LMF. This may include ranging measurements/results received from the Target UE. The endpoints for LPP messages are the LMF and the Located UE(s).

-	The LMF may use the received information to calculate the location of the Target UE and provide the resulting location via the Located UE to the Target UE.

For deferred MT-LR:

-	The UE has been configured in an earlier stage by the LMF when the UE had an NAS connection. The configuration includes e.g., deferred routing identifier, (H)GMLC contact address, LDR reference number.

-	The UE request the Located UE to forward an event report to the network. The UE also provides the deferred routing identifier which the AMF uses to forward the event report to the LMF. The event report includes the Ranging/SL positioning measurement result and the UE identity of the Located UE.

-	The LMF uses the location of Located UE(s) together with the Ranging/SL positioning measurement data or estimation results reported in the event report to estimate the location of the Target UE. The LMF send the result to the GMLC with the LDR reference number. The GMLC uses the LDR reference number to forwards the result to the applicable LCS client or Application Function (via NEF).

Editor's note:	It is FFS if an enhanced 5GC-MO-LR message is sent by the Located UE.

Editor's note:	Security aspects require confirmation from SA WG3.

.

Editor's note:	How LMF associates the ranging/SL positioning measurement or estimation result from different Located UEs is FFS.

# * * * * End of changes * * * *