3GPP TSG-WG SA2 Meeting #156E 	S2-2304378

April 17 – 21, 2023, Elbonia	(revision of S2-230xxxx)

Source:	Sony

Title:	Update Ranging/SL Positioning service exposure though 5GC network.

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper corrects two references in clause 5.6.2.3 for UE exposure using the user plane.

# 1. Introduction/Discussion

Correcting the references

# 2. Text Proposal

It is proposed to update clause 5.5.3 with the following changes.

# * * * * First change* * * *

#### 5.6.2.3	Ranging/SL Positioning service exposure though 5GC network

For Ranging/SL Positioning service exposure to a UE through network, the user plane-based procedure, or control plane-based procedure is used.

When using the user plane-based procedure:

-	The SL Positioning Client UE is provisioned with the Onboarding Enrolment Information during the registration procedure. The Onboarding Enrolment Information is specified in TS 33.122 [11].

-	The SL Positioning client UE invokes the Application exposure API over the application layer as specified clause 6.6.1.2.

-	The authorization of the SL positioning client UE for Ranging and sidelink positioning service invocation performed and checking that the Reference UE's and Target UE's privacy criteria are met, follows the same the procedures as for the AF in clause 6.6.2.

When using the control plane-based procedure:

-	The SL Positioning Client UE invokes the Ranging/SL Positioning service exposure via the 5GC-MO-LR procedure with Sidelink Positioning for relative positioning as defined in clause 6.2.x of TS 23.273 [8].

-	The Client UE sends the 5GC-MO-LR request to AMF and this triggers the AMF to send the Ranging/SL positioning request to the GMLC for the Ranging/SL Positioning service with including the SL positioning client UE ID.

-	The AMF sends the Ranging/SL positioning result to the SL positioning client UE when it receives the result from the GMLC.

Editor's note:	The procedure for Privacy handling on Ranging /SL Positioning service exposure through the 5GC network by 5GC-MO-LR will be aligned with SA3. This includes the authorization of the SL positioning client UE for Ranging and sidelink positioning service invocation of two other UEs.

NOTE:	In this release, for the control plane-based procedure, the SL positioning client UE and the UE who provides the exposure service are registered in the same PLMN.

# * * * * End of changes * * * *