3GPP TSG-SA WG2 Meeting #156	S2-2304384

Elbonia, March 17 - 21, 2023	(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4292 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provisioning Satellite Coverage Availability to the AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CR S2-2303858 introduced the provisioning of satellite coverage availability information to the UE and the AMF. This earlier CR left it open whether the AMF provisioning should re-use the AF and NEF parameter provisioning procedures or be done via O&M procedures. Satellite coverage area information is not subscriber specific. It has been agreed that the satellite coverage availability is provisioned to the AMF on geographical area basis and the AMF can use the single set of information for all UEs roaming to the target area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The existing provisioning procedures cannot be re-used as they are specified for different use case. Consequently, the required architecture is different and no UDM role exists in provisioning the satellite coverage availability information to the AMF. Since there is no readily available procedure, the provisioning is specified to be done via O&M procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GSAT_Ph2 work is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.4.13.3	Coverage availability information provisioning to the AMF

The AMF may use satellite coverage availability information to support satellite access by UEs with discontinuous coverage operation. Satellite coverage availability information may be provisioned to the AMF by O&M.

The satellite coverage availability information provisioned to the AMF describes when and where satellite coverage is expected to be available in an area and contains a list of TAIs with associated time indicating when there is coverage for each TA. The satellite coverage availability information is not UE specific and can be applied by the AMF for any UE in the affected area.

**** End of Changes ****