3GPP WG-SA2 Meeting #156E	S2-2304425

# April 17th – 21th, 2023                                     		     	                   (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4305 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Member-UE selection without the NEF assistance in TS 23.501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2022-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the result of Member Selection Terminology Discussions after SA2#155, the CR proposes to make text adjustment and use the term “Member-UE selection” for the Member-UE selection without the NEF assistance at the AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use the term “Member-UE selection” for the Member-UE selection without the NEF assistance at the A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not unified use for Member Selection Terminology. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.46.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First change ***

5.46.2	Member-UE selection assistance functionality for application operation

5G system may support Member-UE selection assistance functionality to assist the AF in selecting member UE(s) that can be used in application operations such as AI/ML based applications (e.g. Federated Learning). The Member-UE selection assistance functionality shall be hosted by NEF, and the features of the Member-UE selection assistance functionality hosted by NEF include (see clause 4.15.13 of TS 23.502 [3] for details of Member-UE selection procedures):

-	Receiving a request from the AF that shall include a list of target member UE(s), optionally (a) time window(s), and one or more filtering criteria as specified in Table 4.15.13.2-1 of TS 23.502 [3] (e.g. number of UEs, UE location, trajectory and direction, QoS requirements, Area of Interest, DNN, S-NSSAI, expected application operation time duration, preferred access/RAT type, Desired UE transmission latency performance).

-	Referring to the filtering criteria provided by the AF, NEF interacts with 5GC NFs using existing services to collect the corresponding data from relevant 5GC NFs (e.g. PCF, NWDAF and AMF) to derive the list(s) of candidate UE(s) (i.e. UE(s) among the list of target member UE(s) provided by the AF).

-	Providing the AF with the Member-UE selection assistance information, including one or more lists of candidate UE(s), and optionally other additional information (e.g. one or more recommended time window(s) for performing the application operation, QoS of each target UE, UE(s) location, Access/RAT type).

The Member-UE selection assistance information provided by the NEF can be used by the AF to select member UEs. (See clause 5.2.6.31 of TS 23.502 [3] for details of parameters)

NOTE:	AF can choose whether to use the Member-UE selection assistance functionality provided by NEF.

AF in either trusted or untrusted domain can select the member UE(s), e.g. participating in federating learning operation, by collecting the corresponding data using network exposure information as described in clause 4.15.13 of TS 23.502 [3], e.g. UE location reporting from the AMF, user plane information from the UPF and data analytics from NWDAF.

# *** End of Changes ***