3GPP TSG-WG SA2 Meeting #156-e	S2-2304445

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4311 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Address EN for LADN aspects for AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To solve the EN: Editor’s Note: If there is no overlapping between the LADN service area configured in AMF and service area received from UDM, how to handle this case is FFS. Since the LADN Service Area Information sent by the AMF to the UE may contain only a subset of the full LADN service area. When the AMF has both the LADN service area configured in AMF and LADN service area received from UDM, there are three options to solve the EN: - Option#1: set the intersection of the LADN service area received from UDM and the assigned Registration Area as the LADN service area. - Option#2: set the intersection of the LADN service area configured in AMF and the assigned Registration Area as the LADN service area. - Option#3: if there is no intersection of the LADN service area configured in AMF, LADN service area received from UDM and the assigned Registration Area, AMF can send the failure result and rejection code to AF via UDM/NEF, alternatively the UE can reject sending the LADN service area information to the UE or sends a blank LADN Inforamtion to UE. Consider that non-overlapping case occurs due to mis-alignment between local configuration and AF authorized subscription configuration, to minimize the system impacts, it is suggested to use the latest authorized AF provisioned subscribed LADN service area, opiton#1, the subscribed LADN service area received from UDM takes precendence over the local configured LADN service area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description to clarify the subscribed LADN service area received from UDM takes precendence over the local configured LADN service area. - Remove the editor’s note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsolved issues remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.5a, 5.20b.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.6.5a	Supporting LADN per DNN and S-NSSAI

The 5GS may support LADN per DNN and S-NSSAI, and the functions specified in clause 5.6.5 are used (with the extension to apply per DNN and S-NSSAI whenever applicable) with the following enhancements:

-	The UE indicates the support of LADN per DNN and S-NSSAI in the UE MM Core Network Capability of the Registration Request message.

-	The LADN Service Area can be provisioned for a group (e.g. 5G VN group) or an individual subscriber using UDM/NEF parameter provisioning service triggered by AF request as described in clause 4.15.6 of TS 23.502 [3].

-	The LADN Service Area Information is determined by AMF, based on the local configured LADN service area or the subscribed LADN service area received from UDM, and Registration Area that the AMF assigns to the UE.

-	If the AMF has both the local configured LADN service area and the subscribed LADN service area received from UDM, the subscribed LADN service area received from UDM takes precendence over the local configured LADN service area.

-	If UE indicates the support of LADN per DNN and S-NSSAI, the AMF provides the LADN Service Area Information per LADN DNN and S-NSSAI to the UE.

-	The AMF enforces the LADN Service Area Information per LADN DNN and S-NSSAI in the same way as is described in clause 5.6.5 with the difference that the LADN service area is defined per DNN and S-NSSAI.

-	The UE enforces the LADN Service Area Information per LADN DNN and S-NSSAI, if received from the AMF, in the same way as is described in clause 5.6.5 with the difference that the LADN service area is defined per DNN and S-NSSAI.

# * * * * Next change * * * *

### 5.20b.2	Support service area for a group

The procedure as defined in clause 4.15.6.2 of TS 23.502 [3] is applicable for provisioning of service area for a group with the following clarifications and enhancements:

-	The AF request additionally contains the LADN service area as part of DNN and S-NSSAI specific Group Parameters, and the service area is stored in UDR as subscription data and delivered to AMF as well as SMF.

-	If the AF provides the LADN service area in the form of geographical information, the NEF maps the geographical information to a list of TAs before sending the service area to the UDM.

LADN per DNN and S-NSSAI as defined in clause 5.6.5a is applicable for enforcement of LADN service area for a group with the following clarifications and enhancements:

-	If the group is provisioned with the LADN service area, the AMF configures the DNN of the group as LADN DNN and determines the LADN service area based on local configured LADN service area corresponding to the DNN (if any) and the LADN service area received from UDM.

-	If the group is provisioned with the LADN service area, the SMF configures the DNN of the group as LADN DNN.

Editor's note:	The SMF does not need to know whether there is a LADN Service Area for a DNN/S-NSSAI, SMF does not need to know the content of the LADN Service Area. Whether this should be reflected in the data provided by UDM is FFS.

# * * * * End of change * * * *