3GPP TSG SA WG2 Meeting #156E	S2-2304455

# Elbonia, April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4316 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LADN service area clarifications for enhanced group attribute management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution removes the ENs related to LADN aspects for a group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replacing the EN: “If there is no overlapping between the LADN service area configured in AMF and service area received from UDM, how to handle this case is FFS.” with a NOTE clarifying that in case of mismatch between the LADN service area configured in the AMF and the LADN service area received from UDM it depends on operator’s policies configured in the AMF which one takes precedence. - Replacing the EN: “The SMF does not need to know whether there is a LADN Service Area for a DNN/S-NSSAI, SMF does not need to know the content of the LADN Service Area. Whether this should be reflected in the data provided by UDM is FFS.” with a NOTE clarifying that the SMF does not need to know the content of the LADN service area. The AMF sends an indication to the SMF if the UE is inside the LADN service area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service area for a group functionality not fully specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20b.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 5.20b.2	Support service area for a group

The procedure as defined in clause 4.15.6.2 of TS 23.502 [3] is applicable for provisioning of service area for a group with the following clarifications and enhancements:

-	The AF request additionally contains the LADN service area as part of DNN and S-NSSAI specific Group Parameters, and the service area is stored in UDR as subscription data and delivered to AMF as well as SMF.

-	If the AF provides the LADN service area in the form of geographical information, the NEF maps the geographical information to a list of TAs before sending the service area to the UDM.

LADN per DNN and S-NSSAI as defined in clause 5.6.5a is applicable for enforcement of LADN service area for a group with the following clarifications and enhancements:

-	If the group is provisioned with the LADN service area, the AMF configures the DNN of the group as LADN DNN and determines the LADN service area based on local configured LADN service area corresponding to the DNN (if any) and the service area received from UDM.

NOTE 1:  In case of mismatch between the configured LADN service area in the AMF and the LADN service area received from UDM, it is up to operator’s policy configured at the AMF which information takes precedence.

-	If the group is provisioned with the LADN service area, the SMF configures the DNN of the group as LADN DNN.

NOTE 2:  The SMF does not need to know the LADN Service Area for a DNN/S-NSSAI. Upon a PDU Session Establishment request with LADN DNN or Service Request for the established PDU Session corresponding to LADN, the AMF indicates to the SMF if the UE is inside the LADN service area. The SMF subscribes to "UE mobility event notification" for reporting UE presence in Area of Interest by providing LADN DNN to the AMF as described in clauses 5.6.11 and 5.3.4.4.

* * * * End of Changes * * * *