3GPP TSG-WG SA2 Meeting #156                                           	S2-2304462

April 17-21, 2023 Elbonia

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 758 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consolidating ADRF MLmodelmanagement service in case of multiple ML models |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS23.288 v18.1.0 consolidates the input and output of MLmodelProvision service in case of single and multiple ML models. The ADRF MLmodelmanagement service still distinguishes the cases of single vs multiple ML models. This CR proposes to consolidate the two cases following the principle of MLmodelProvision service |  |  |  |  |  |  |  |  |
| Summary of change: |  | TS23.288, clauses 10.3.2 and 10.3.3 are updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ADRF MLmodelmanagement service still distinguishes the cases of single vs multiple ML models. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 10.3	Nadrf_MLModelManagement service

### 10.3.1	General

Service Description: This service enables the consumer to store and remove ML model(s) or ML model address(es) from an ADRF.

### 10.3.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store ML model(s). ML model(s) or ML model address(es) stored in NWDAF containing MTLF are provided to the ADRF in the request message.

Inputs, Required: ML model file(s) or Set of:

-	the tuple (NF instance ID of the NWDAF containing MTLF, one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in NWDAF containing MTLF).

NOTE:	The ADRF can download ML model files based on the ML model addresses provided by the NWDAF containing MTLF. How the ADRF downloads the ML model and locally stores the ML model is left for implementation.

Inputs, Optional: Validity period, Spatial validity.

Outputs Required: Result Indication,

one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: Storage Transaction Identifier.

### 10.3.3	Nadrf_MLModelManagement_Delete service operation

Service operation name: Nadrf_MLModelManagement_Delete

Description: This service operation instructs the ADRF to delete stored ML model file(s) or ML model address(es).

Inputs, Required: One of the following:

-	Storage Transaction Identifier; or

-	one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in ADRF.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 10.3.4	Nadrf_MLModelManagement_Retrieval Request service operation

Service operation name: Nadrf_MLModelManagement_Retrieval Request

Description: The consumer NF uses this service operation to request the ADRF to get the ML model(s) stored in ADRF.

Inputs, Required:

-	Analytics ID(s);

-	Storage Transaction Identifier; or

-	one or more tuples of unique ML Model identifier stored in ADRF.

Inputs, Optional: None.

Outputs Required:

-	address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: Validity Period.

# * * * * End of change * * * *