3GPP TSG-SA WG2 Meeting #156e	S2-2304467

# 17th – 21th April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2088 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing differentiated service for Non authenticable (NAUN3) devices connected behind a 5G RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in the conclusion for Key Issue #1 " Providing differentiated service for UE and Non-3GPP devices connected behind a 5G RG " in TR 23.700-17: - Normative work will proceed according to following principles: - FN RG are not considered - principles of solution 1 are endorsed as follows: - The RG is configured with the mapping between SSID, RG physical ports, etc… and Connection Capabilities whose values are defined by the operator. - Policies configured by the PCF for the RG are used to map from Connection Capabilities to parameters of the PDU session to use for the traffic of NAUN3 devices (DNN, S-NSSAI, PDU Session Type, SSC mode)) associated with a given Connection Capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add differentiated service for Non authenticable (NAUN3) devices connected behind a 5G RG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | differentiated service for Non authenticable (NAUN3) devices connected behind a 5G RG is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10b (new), 9.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

* * * Change * * * * all text is new

## 4.10b	Non-authenticable Non-3GPP (NAUN3) device behind 5G-RG

This clause defines support for NAUN3 devices behind a 5G-RG. This clause applies only to 5G-RG.

- The operator may define NAUN3 access categories that relate with the type of PDU Session that may be used by a NAUN3 device connecting to a 5G RG via a non-3GPP interface of the 5G-RG ; a non-3GPP interface of the 5G-RG associated with an NAUN3 access category may correspond to an SSID or to a physical (e.g; Ethernet) port of the 5G RG.

- The 5G-RG is locally configured with the mapping between non-3GPP interfaces of the 5G-RG such as SSID, RG physical ports, etc. and NAUN3 access category.  This local configuration is out of scope of 3GPP and may be done by the end-user.

- URSP (as defined in clause 9.5.2.1) configured by the PCF on the 5G-RG are used to map from NAUN3 access category to parameters of the PDU session (e.g. DNN, S-NSSAI, PDU Session Type, SSC mode)) to use for the traffic of NAUN3 devices using a non-3GPP interface of the 5G-RG. These URSP rules defined in 23.503 [4], may contain a time validity condition but contain only an NAUN3 access category  in their Traffic Descriptor.

NOTE X: The procedures for the connection of NAUN3 devices with a 5G-RG is not in the scope of 3GPP

* * * Next Change * * * *

#### 9.5.2.1	5G-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6. for 5G-RG.

If the PCF provides the URSP policy to the 5G-RG, the PCF should neither include NSWO indication nor any ANDSP policies. The 5G-RG shall ignore any NSWO indication or any ANDSP policies if received from the 5GC. The 5G-RG shall use the URSP policy as specified in TS 23.503 [4], for example for the association of application and PDU session, slices, etc.

The URSP indicates for the application of Auto-Configuration Server (ACS) which PDU session type, NSSAI and/or DNN is to be used. The 5G-RG establishes the connectivity to the management entity (e.g. ACS) via user plane connection on a PDU session according to the URSP.

UE Policy procedures defined in clause 6.1.2.2 of TS 23.503 [4] are applicable as follows:

-	Roaming is not applicable to W-5GAN access in this release of specification.

In order to support the case when NAUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF on 5G-RG.

UE Route Selection Policy information targetting an NAUN3 access category follows the structure defined in clause 6.2.2 of TS 23.503 [4] with following exception:

-	RSD elements associated of Table 6.6.2.1-2 are Not applicable for NAUN3 access category. Hence the (unique) TD of the (unique) URSP for an NAUN3 access category is assumed to always match

-	These URSP rules defined in 23.503 [4], may contain a time validity condition but contain only an NAUN3 access category in their Traffic Descriptor.

* * * End of Changes * * * *