3GPP TSG-WG SA2 Meeting #156-e	S2-2304490

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4329 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update PIN policy delivery for PIN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR aims to clarify that the PIN ID added in the URSP is the External Group ID for PIN but not the Internal Group ID which will be used by 5GC. Otherwise, it is unclear and confusing. Additionally, as discuss in the S2-2304928, single PDU session cannot be shared by multiple PINs. In order to identify the PIN that the PDU Session serves for, the RSD further includes the Internal Group ID. Otherwise, the same PDU session may be selected by UE to server for different PINs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the 5GS supports the provisioning of URSP rules that include a PIN ID (i.e. an External Group ID for PIN) as Traffic Descriptor and include an Internal Group ID as Route Selection Descriptor. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text in 23.501 is not clear and if the PIN ID is an Internal Group ID, it cannot work because the UE has no information to map the PIN ID allocated by AF to the Internal Group ID allocated by UDM. 5GC is not able to identify which PIN the PDU session established by the PEGC serves for. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.44.2	UE policy delivery for PIN

For a UE acting as a PEGC registered in the 5GS, the 5GS supports the provisioning of URSP rules that include a PIN ID (i.e. an External Group ID for PIN) as Traffic Descriptor and include an Internal Group ID as Route Selection Descriptor. URSP rules with a PIN ID in the Traffic Descriptor are sent to the UE based on the information provided from an AF for PIN as specified in TS 23.502 [3] and TS 23.503 [45] for policy delivery.

# * * * * End of changes * * * *