3GPP TSG-WG SA2 Meeting #156e	S2-2304494

Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on N5CW device using decorated NAI

Document for:	Discussion

Agenda Item:	9.24.2

Work Item / Release:	eNPN_Ph2 / Rel-18

Abstract: If N5CW device accesses the 5GC with decorated NAI, two different realm parts would be used. How to handle these two realm parts is unclear in the current specification. This discussion paper would like to discuss different options for handling the Decorated NAI provided by N5CW device.

1. Introduction

N5CW device uses decorated NAI for accessing SNPN with credentials owned by Credentials Holder has been agreed in the SA2#155 meeting. The Decorated NAI includes two different realm parts which indicate CH and the selected SNPN (i.e. the SNPN that N5CW device wants to connect). Currently, both AMF and AUSF do not support of handling the Decorated NAI format. Therefore, this discussion paper would like to provide analysis of different options of handling Decorated NAI and propose a possible solution to solve it with least impacts on 5GC.

2. Discussion

The figure 1 and figure 2 show the example of possible deployment scenario where a TWAN or a TWAP is shared among different SNPNs. Users of different CH can connect to the TWAN.

Figure 1: TWAN shared among 3 SNPNs supporting different CHs

Figure 2: TWAP shared among different SNPNS supporting different CHs

The Decorated NAI format for SUCI shall take the form of a NAI and shall have the format

‘Homerealm!username@otherrealm’

As specified in clause 28.7.9 of the TS 23.003.

The Decorated NAI is used to select SNPN which is used to connect to the Credentials Holder based on the ‘otherrealm’.

In the existing mechanism, AMF selects an AUSF based on SUPI or SUCI as described in clause 6.3.4 of TS 23.501. Home Network Identifier (e.g. MNC and MCC, realm) of SUCI/SUPI along with the selected NID in the case of SNPN is needed for AMF to select AUSF.

When UE access SNPN with CH, AMF selects the AUSF and in case of the CH supporting AAA server, the AUSF selects the UDM to retrieve the SUPI for selecting the NSSAAF and the AAA server. While if the CH supports the UDM, the AMF selects the AUSF to perform the authentication.

If the Decorated NAI has been provided to the AMF, the AMF cannot distinguish the “homerealm” and the “otherrealm” in the Decorated NAI. Therefore, the AMF cannot select the corresponding AUSF for the N5CW device based on the Decorated NAI.

There are two options:

Option A: TWIF selects the AMF based on Decorated NAI and removes the decoration from received NAI.

TWIF can create a 5GC Registration Request message on behalf of the N5CW device. The other realm included in the Decorated NAI is used by the TWAN to select the AMF. And the TWIF uses the NAI used by N5CW device (i.e. ‘username@homerealm’) in the Registration Request message.

In this option, AMF, AUSF, UDM and AAA Server receive a NAI used by N5CW device and select the CH based on the ‘homerealm’.

Option B: TWIF selects the AMF based on Decorated NAI and forward Decorated NAI.

In this option, AMF, AUSF and/or UDM shall be able to understand the decorated NAI. One of them shall be able to understand the decorated NAI. One of them need to be able to manage the decorated NAI and to remove the decoration. This is a new function of the NF.

These two options have different impact to the access network and 5G core.

- Option A, it has impact to the TWIF. TWIF provides a NAI used by the N5CW device (i.e. ‘username@homerealm’) to the AMF. The existing mechanism can be used in the 5GC part.

- Option B, it has more impact to the 5GC. AMF, AUSF and/or UDM shall have a new functionality to understand the Decorated NAI. And one of those NF shall have the ability to remove the decoration.

When N5CW device uses Decorated NAI for accessing the 5GC, the existing specified mechanism cannot select the appropriate AUSF based on the Decorated NAI. Therefore, the two options of the enhancement have been provided to solve this problem.

Considering that:

- the deployment scenario supports only 1 intermediate network (the SNPN).

- that per clause 5.30.2.13 in TS 23.501 a non-3GPP access network may advertise (e.g. with ANQP), not only the PLMNs with which 5G connectivity is supported (as specified in clause 6.3.12.2), but also the SNPNs with which 5G connectivity is supported and the related parameters and indications defined in clause 5.30.2.2 (i.e. human-readable network name(s), GIN(s), indication whether access using credentials from a Credentials Holder is supported, indication whether SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN, etc.).

The use cases are:

- The UE knows that SNPN supports the Credential Holders and that its own credentials are those of a Credential Holders, then the selected SNPN and the selected WLAN are intermediate network, hence it build the Decorated NAI “Homerealm!username@otherrealm” where the “otherrealm” indicates those of the selected SNPN.

- The UE selects a SNPN (among those advertised) which is different from the home network (they have different identifiers) and select a WLAN then the Selected SNPN and the selected WLAN are intermediate network, hence it build the Decorated NAI “Homerealm!user@otherrealm” where the “otherrealm” indicates those of the selected SNPN

- The UE only select a WLAN, since for example the SNPN are not advertised, then the UE send the normal NAI “usernam@homerealm”. The TWIF selects an AMF in a SNPN based on local configuration, for example via association of homerealm with a specific SNPN. This does not guarantee that the corrects SNPN will be selected unless the TWAN (WLAN & TWIF) belongs to the SNPN or it is not shared among other SNPNs.

Then in any of the above the TWIF remove the Decoration and send a regular NAI to the SNPN, which is able to select the CH based on current specification.

3. Conclusion and proposal(s)

Based on above consideration it is proposed that

- If the UE selects a SNPN which is different from the home network the UE sends a Decorated NAI build as “homerealm!username@otherrealm”

- If the TWIF receives a Decorated NAI (“homerealm!username@otherrealm”), it selects an AMF in the SNPN indicated by “otherrealm”. The TWIF removes the Decoration and forward to AMF in the Registration Request with the regular NAI “username@homerealm”