3GPP TSG-WG SA2 Meeting #156-e	S2-2304511

# Elbonia, April 17 – 21, 2023		(revision of S2-2302656)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0331 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure of Network based Sidelink Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - This CR adds the Procedure of Network based Sidelink Positioning (i.e., Procedure of UE Positioning using Sidelink Positioning with involving NF) based on the KI#5 conclusion in TR 23.700-86. - In clause 5.5.2 of TS 23.586, the LMF may interact with GMLC to get the location of Located UE. This paper proposes the NL5 supporting the LMF and GMCL interaction, and also LMF as a consumer of GMLC service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Procedure of UE Positioning using Sidelink Positioning with involving NF is added. - Update NL5 reference point to support the LMF and GMCL interaction; To add LMF as a consumer of GMLC service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure of UE Positioning using Sidelink Positioning with involving NF is not supported, and Not clear LMF being as a consumer of GMLC service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17, 6.1.X (new), 6.2a (new), 4.4.6, 8.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.17	Support of Ranging and Sidelink Positioning

Ranging and Sidelink Positioning as defined in TS 23.586 [40] is supported. The following procedures have been specified to support the Network based Sidelink Positioning:

- SL-MT-LR involving LMF as defined in clause 6.18;

- SL-MO-LR involving LMF as defined in clause 6.19.

Editor's Note: This clause will be updated with full list of procedures for Ranging/SL Positioning support by stage 2 freeze deadline.

# * * * * Second change * * * *

### 4.4.6	NL5 Reference Point

The NL5 reference point supports location requests sent by an NEF or other NF (e.g., LMF) to a GMLC.

# * * * * Third change* * * *

## 8.4	GMLC Services

### 8.4.1	General

The following table shows the GMLC Services and GMLC Service Operations.

Table 8.4.1-1: List of GMLC Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ngmlc_Location | ProvideLocation | Request / Response | GMLC, NEF, NWDAF, LMF |
|  | LocationUpdate | Request / Response | AMF, GMLC |
|  | LocationUpdateNotify | Notify | NEF, NWDAF |
|  | CancellLocation | Request / Response | GMLC, NEF, NWDAF |
|  | EventNotify | Notify | GMLC, NEF, NWDAF |

# * * * * Forth change (all new) * * * *

### 6.X.1	5GC-MT-LR procedure with Sidelink Positioning

Figure 6.X.1-1: 5GC-MT-LR procedure with Sidelink positioning

1.	5GC-MT-LR procedure is triggered and Target UE’s LMF receives a location request for Target UE as specified in clause 6.1. The location request may include a scheduled location time (Time T) and a coordinate ID. The coordinate ID indicates the local coordinate of which the Target UE location is expressed.

2.	Target UE’s LMF decides to use Target UE Positioning using Sidelink Positioning.

Editor’s note:	It is FFS that how does the LMF determine to apply Target UE Positioning using Sidelink Positioning.

Target UE’s LMF triggers the discovery of Located UE(s) for positioning assistance. The Located UE discovery request message may includes a candidate list of Located UE. If received in step 1 the coordinate ID, LMF sends it to Target UE.

NOTE 1:	LMF can decide when to send the request of step 2 based on the Time T if existing.

3.	Target UE performs the Located UE discovery, using the procedure described in clause 6.4 of TS 23.586 [40]. If received in step the coordinate ID, Target UE discovers a Located UE that supports the coordinate ID.

4.	The Target UE sends the Located UE discovery response to LMF, including the identity of the selected Located UE(s).

Editor’s note:	It is FFS what the the identity of the selected Located UE(s) is and the related security aspect need the coordination with SA WG3. Depending on which UE identity, the LMF or the GMLC may need to resolve this by query another NF.

5.	Target UE’s LMF triggers the location procedure for each selected Located UE and sends the Provide Location Request to GMLC. The request message may include the Time T.

6.	GMLC triggers the positioning procedure for each selected Located UE, as specified in clause 6.1, and Located UE positioning is scheduled with Time T. GMLC gets the Located UE location estimation.

7.	GMLC responses the location estimation of each selected Located UE to Target UE’s LMF.

8.	Target UE’s LMF sends UL Positioning message to the Target UE with the Time T and Ranging/SL positioning QoS parameters.

NOTE 2:	Step 8 can be triggered at the same time of step 5.

9.	Target UE performs Ranging/Sidelink positioning procedure with each selected Located UE, as described in clause 6.Z of TS 23.586 [40], and Ranging/Sidelink positioning procedure is scheduled with the Time T if existing. Ranging/SL positioning QoS parameters are used for Ranging/Sidelink positioning control. Target UE obtains the Ranging/Sidelink positioning measurement data. Also, Target UE may obtain the Located UE position in local coordinate, if coordinate ID is received in step 2.

10.	Target UE provides the Ranging/Sidelink positioning measurement data to the Target UE’s LMF. Target UE may provide the Located UE position in local coordinate in this step.

11.	Target UE’s LMF determines the Target UE location estimation based on the Ranging/Sidelink positioning measurement data received in step 10 and the location estimation of each selected Located UE received in step 7. Target UE’s LMF may need to compensate the time difference of the positioning of Target UE and each Located UE. LMF may determine the Target UE location in local coordinate, if the coordinate ID is received in step 1.

Target UE’s LMF provides the Target UE location estimation to the AMF and the following procedures as specificed in clause 6.1.

# * * * * Fifth change (all new) * * * *

### 6.Y.2	5GC-MO-LR procedure with Sidelink Positioning

Figure 6.Y.2-1: 5GC-MO-LR procedure with Sidelink positioning

1.	Target UE performs the Located UE discovery for positioning assistance, using the procedure described in clause 6.4 of TS 23.586 [40].

2.	5GC-MO-LR procedure is triggered, and Target UE sends 5GC-MO-LR Request to Target UE’s LMF, additionally including the identity of the selected Located UE(s) and a scheduled location time (Time T).

Editor’s note:	It is FFS what the the identity of the selected Located UE(s) is and the related security aspect need the coordination with SA WG3. Depending on which UE identity, the LMF or the GMLC may need to resolve this by query another NF.

3-5.	Step 3-5 are the same as steps 5-7 defined in clause 6.X.2.

6.	Target UE performs Ranging/Sidelink positioning procedure with each selected Located UE, as described in clause 6.Z of TS 23.586 [40], and Ranging/Sidelink positioning procedure is scheduled with the Time T. Target UE obtains the Ranging/Sidelink positioning measurement data.

NOTE:	Step 7 can be triggered by Target UE itself based on the Time T.

7.	Target UE provides the Ranging/Sidelink positioning measurement data to the Target UE’s LMF.

8.	Target UE’s LMF determines the Target UE location estimation based on the Ranging/Sidelink positioning measurement data received in step 7 and the location estimation of each selected Located UE received in step 5.

9.	Target UE’s LMF sends 5GC-MO-LR Response to Target UE including the Target UE location estimation.

# * * * * End of changes * * * *