3GPP TSG-WG SA2 Meeting #156e	S2-2304513

Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

Source:	Huawei, HiSilicon

Title:	Procedures for Service Authorisation and Provisioning to RAN

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: this contribution is to specify the Procedures for Service Authorisation and Provisioning to RAN.

1. Introduction/Discussion

The procedure for authorisation and provisioning for Ranging/Sidelink positioning to RAN is necessary and shall be  specified in the normative phase, based on the conclusion of KI#8. From the perspective of “functional description and information flows”, it adds the procedure for authorisation and provisioning for Ranging/Sidelink positioning to RAN as one basic procedure.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change (All text new)* * * *

6.3	Procedures for Service Authorization to NG-RAN

6.3.1	General

In order to support PC5 radio resource control in NG-RAN, Ranging/SL Positioning service Authorisation information and QoS parameters for Ranging/SL Positioning need to be made available in NG-RAN. This clause describes the corresponding procedures and aspects.

6.3.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [3] clause 4.2.2.2 with the following additions:

-	The UE includes the Ranging/SL Positioning Capability as part of the 5G ProSe Capability/V2X capability in the "5GMM capability" in the Registration Request message. The AMF stores the Ranging/SL Positioning Capability for Ranging/SL Positioning operation. The Ranging/SL Positioning Capability indicates whether the UE supports one or more of the Ranging/SL Positioning capabilities as defined in clause 4.3.1.

-	The AMF obtains the Ranging/SL Positioning subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [3].

-	The AMF determines whether the UE is authorised to use Ranging/SL Positioning services based on UE's Ranging/SL Positioning Capability and the Ranging/SL Positioning Service Authorisation included in the subscription data received from UDM as specified in clause 5.1. The AMF stores the authorized Ranging/SL Positioning Capability.

-	The AMF sends the authorized Ranging/SL Positioning Capability for Ranging/SL Positioning operation to PCF. Based on the received Ranging/SL Positioning Capability from the AMF, the PCF provides the QoS parameters for Ranging/SL Positioning to AMF. The AMF stores such information as part of the UE context.

Editor’s note: The details of the QoS parameters of Ranging/SL positioning used by NG-RAN is FFS and will be coordinated with RAN WG2.

-	If the UE is authorised to use Ranging/SL Positioning services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"Ranging/SL Positioning authorised" information, indicating the UE is authorized to use Ranging/SL positioning service in the serving PLMN.

-	the QoS parameters for Ranging/SL Positioning used by the NG-RAN for the resource management of UE's Ranging/SL Positioning services in network scheduled mode.

-	If the UE is authorised to use Ranging/SL Positioning services, the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [3].

6.3.3	Service Request procedure

The Service Request procedures for UE in CM-IDLE state are performed as defined in TS 23.502 [3] clause 4.2.3.2 and clause 4.2.3.3 with the following additions:

-	If the UE is authorised to use Ranging/SL Positioning services, then the AMF shall include "Ranging/SL Positioning authorised" information in the NGAP message as described in clause 6.3.2.

-	The AMF sends the QoS parameters for Ranging/SL Positioning to NG-RAN via N2 signalling. The QoS parameters for Ranging/SL Positioning may be stored in the UE context after the registration procedure. If the UE is authorised to use Ranging/SL Positioning services but AMF does not have QoS parameters for Ranging/SL Positioning available, the AMF fetches the QoS parameters for Ranging/SL Positioning from the PCF.

Editor’s note: The details of the QoS parameters of Ranging/SL positioning used by NG-RAN is FFS and will be coordinated with RAN WG2.

6.3.4	N2 Handover procedure

The N2 based handover procedures for UE are performed as defined in TS 23.502 [3] clause 4.9.1.3 with the following additions:

-	If the UE is authorised to use Ranging/SL Positioning services, then the target AMF shall send the "Ranging/SL Positioning authorised" information and QoS parameters for Ranging/SL Positioning to the target NG-RAN in the NGAP Handover Request message.

Editor’s note: The details of the QoS parameters of Ranging/SL positioning used by NG-RAN is FFS and will be coordinated with RAN WG2.

6.3.5	Xn Handover procedure

The Xn based handover procedures for UE are performed as defined in TS 23.502 [3] clause 4.9.1.2 with the following additions:

-	If the "Ranging/SL Positioning authorised" information is included in the UE context, then the source NG-RAN shall include a "Ranging/SL Positioning authorised" information and QoS parameters for Ranging/SL Positioning in the XnAP Handover Request message to the target NG-RAN.

-	If the "Ranging/SL Positioning authorised" information is included in the UE context, then the AMF shall send the "Ranging/SL Positioning authorised" information and QoS parameters for Ranging/SL Positioning to the target NG-RAN in the Path Switch Request Acknowledge message.

Editor’s note: The details of the QoS parameters of Ranging/SL positioning used by NG-RAN is FFS and will be coordinated with RAN WG2.

6.3.6	Subscriber Data Update Notification to AMF

The procedure of Subscriber Data Update Notification to AMF is performed as defined in TS 23.502 [3] clause 4.5.1 with the following additions:

-	The Nudm_SDM_Notification service operation may contain the Ranging/SL Positioning Service Authorisation.

-	The AMF updates the UE Context with the above new Ranging/SL Positioning subscription data.

-	When the AMF updates UE context stored at NG-RAN, the UE context contains the Ranging/SL Positioning subscription data.

6.3.7	Delivery of QoS parameters for Ranging/SL Positioning to NG-RAN

The UE Policy Association Establishment procedure and UE Policy Association Modification procedure, as defined in TS 23.502 [3], is used to provide the AMF with QoS parameters used by NG-RAN. When receiving the Ranging/SL Positioning Capability in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the QoS parameters used by NG-RAN corresponding to a UE.

The (V-)PCF provides the information to the AMF as follows:

-	In the roaming case, the H-PCF includes the QoS parameters used by NG-RAN in the Npcf_UEPolicyControl_Create Response message or Npcf_UEPolicyControl UpdateNotify Request message sent to V-PCF in an N2 PC5 policy container, and V-PCF relays this N2 PC5 policy container as the N2 container in the Namf_Communication_N1N2MessageTransfer message sent to AMF.

-	In the non-roaming case, the PCF includes the QoS parameters used by NG-RAN in an N2 container in Namf_Communication_N1N2MessageTransfer message sent to AMF.

When the AMF receives the N2 PC5 policy container from (V-)PCF, the AMF stores such information as part of the UE context.

In the UE Configuration Update procedure triggered by UE Policy Association Establishment or UE Policy Association Modification:

-	The AMF forwards the QoS parameters in the NGAP message to the NG-RAN if a N2 PC5 policy container is received in the Namf_Communication_N1N2MessageTransfer message.

NOTE 1:	If the QoS parameters are provided to both NG-RAN and UE, both the N2 PC5 Policy Container and the UE Policy Container are included in the Namf_Communication_N1N2MessageTransfer message.

NOTE 2:	Non-UE specific QoS parameters, e.g. default QoS parameters, can also be locally configured in NG-RAN. How such configuration is performed is out of scope of this specification.

Editor’s note: The details of the QoS parameters of Ranging/SL positioning used by NG-RAN is FFS and will be coordinated with RAN WG2.

# * * * * End of changes * * * *