3GPP TSG-WG SA2 Meeting #156e	S2-2304514

Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

Source:	Huawei, HiSilicon

Title:	Procedure for Ranging/SL Positioning service exposure to UE

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: this contribution is to specify the Procedure for Ranging/SL Positioning service exposure to UE.

1. Introduction/Discussion

The procedure for Ranging/SL Positioning service exposure to UE is necessary and shall be specified in the normative phase, based on the conclusion of KI#6. From the perspective of “functional description and information flows”, it adds the procedure for Ranging/SL Positioning service exposure to UE as one basic procedure.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change (All text new)* * * *

6.6.1	Procedures for Service exposure to SL Positioning Client UE

6.6.1.1	Procedures for Ranging/SL Positioning service exposure though PC5

The SL Positioning Client UE, Reference UE, and Target UE are involved in the Ranging/SL positioning control. The SL Positioning Client UE requests the Reference/Target UE via PC5 interface for Ranging/SL positioning service.

UE2

NG-RAN

UE1

SL Positioning Client UE

2. SL Positioning Request

4. SL Positioning Response

3. Ranging/SL Positioning Control

1. Ranging/SL Positioning Discovery

Figure 6.6.1.1-1: Service exposure to SL Positioning Client UE via PC5

1.	The application layer of the SL Positioning Client UE triggers the request for the Ranging/SL Positioning service. The SL Positioning Client UE performs the Ranging/SL Positioning Discovery procedure for the UE1 and/or UE2 and the UE1 is discovered.

2.	The SL Positioning Client UE sends SL Positioning Request to UE1, to ask for the SL Positioning result of UE1 and UE2. The identifications of UE1 and UE2 are included in the request.

3.	UE1 initiates the Ranging/SL Positioning Control with the UE2 and obtains the result. The NG-RAN node is involved if UE is in network coverage and the scheduled resource allocation is used. See clause 5.3 for the details.

4.	The UE1 returns the Ranging/SL Positioning result to the SL Positioning Client UE via the PC5 interface.

6.6.1.2	Procedure for Ranging/SL Positioning service exposure though 5GC network

For Ranging/SL Positioning service exposure procedure through 5GC network, the procedures of 5GC-MO-LR involving LMF is used as described in clause 6.X of TS 23.273 [8].

# * * * * End of changes * * * *