3GPP TSG-WG SA2 Meeting #156E	S2-2304557

# April 17 - 21, 2023, Electronic Meeting	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4343 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional information in PMF-Suspend Duplication Request message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE actions when duplication is suspended are not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of “access leg to use when traffic duplication is suspended” (in the PMF-Suspend Duplication Request message. - Addition of new Note 3, describing behaviour when access leg to use is not included in the PMF-Suspend Duplication Request message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UPF has no ability to tell the UE which access leg to use if it wants to suspend traffic duplication on a specific access leg |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Begin of changes * * * *

#### 5.32.5.6	Suspend and Resume Traffic Duplication

As part of the Redundant Steering Mode, UPF can decide to suspend traffic duplication for a UE by sending PMF- Suspend Duplication Request message to the UE. How the UPF determines to suspend traffic duplication is implementation specific.

NOTE 1:	Suspension can e.g. happen in case of locally detected UPF congestion. In that way the UPF can stop receiving duplicated traffic via 3GPP and non-3GPP access simultaneously.

The UPF may indicate in PMF-Suspend Duplication Request message:

- the type of traffic (i.e. GBR or non-GBR) for which traffic duplication is being suspended;

- the access leg to use when traffic duplication is suspended (i.e. 3GPP access or non-3GPP access).

Once the UE receives PMF-Suspend Duplication Request message from the UPF, the UE shall stop duplicating the type of traffic for which traffic duplication is suspended.

NOTE 2:	If the UPF does not provide the type of traffic (GBR or non-GBR) in the PMF-Suspend Duplication Request message, traffic duplication is suspended for all traffic for which traffic duplication is being performed.

NOTE 3:	If the UPF does not provide the access leg to use (3GPP access or non-3GPP access) in the PMF-Suspend Duplication Request message, the UE may determine which access leg to use for the uplink transmissions.

UPF may decide to resume traffic duplication for a UE by sending PMF-Resume Duplication Request message. How the UPF determines to resume traffic duplication is implementation specific.

NOTE 4:	Resume of traffic duplication can e.g. happen in case UPF has detected that local congestion has diminished.

Once the UE receives PMF-Resume Duplication Request message from the UPF, the UE may again duplicate the type of traffic for which traffic duplication is resumed based on the provided Redundant steering mode policies and UE implementation.

# * * * * End of changes * * * *