3GPP TSG-SA2 Meeting #156E	S2-2304565

# April  17 – 21, 2023, Elbonia									 								  (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0095 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of UE requesting UE policies from PCF in REGISTRATION REQUEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Inc., LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For V2X and ProSe, CT1 decided NOT to support requirements for UE requesting UE policies from PCF in REGISTRATION REQUEST in any release (see C1-227137) and SA2 subsequently removed those requirements from SA2 specs. Whereas, for Rel-18 UAS, SA2 introduced similar requirements for A2X services, which if CT1 has already ruled out as not agreement can be reached to support it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove any requirements for UE to request UE policies from PCF via Registration Request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unfulfilled stage 2 requirements which will never be implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 5.10.2	PCF based A2X Service Authorization and Provisioning to UE

For PCF based Service Authorization and Provisioning to UE, the Registration procedures as defined in clause 4.2.2.2 of TS 23.502 [3], UE Policy Association Establishment procedure as defined in clause 4.16.11 of TS 23.502 [3] and UE Policy Association Modification procedure as defined in clause 4.16.12 of TS 23.502 [3] apply with the following additions:

-	If the UE indicates A2X capability in the Registration Request message and if the UE is authorized to use A2X service based on subscription data, the AMF selects the PCF which supports A2X Policy/Parameter provisioning and establishes a UE policy association with the PCF for A2X Policy/Parameter delivery. PCF discovery and selection mechanism defined in clause 6.3.7.1 of TS 23.501 [2] applies and the AMF may include the A2X capability indication in the Nnrf_NFDiscovery_Request message as the optional input parameter. If provided, the NRF takes the information into account for discovering the PCF instance.

-	If the AMF receives the PC5 capability for A2X in the Registration Request message from UE, the AMF further reports the PC5 capability for A2X to the selected PCF. The PCF may determine the A2X Policy/Parameter for specific PC5 RAT based on the received UE's PC5 capability for A2X.

The PCF may update the A2X Policy/parameters to the UE in following conditions:

-	UE Mobility, e.g. UE moves from one PLMN to another PLMN. This is achieved by using the procedure of UE Policy Association Modification initiated by the AMF, as defined in clause 4.16.12.1 of TS 23.502 [3].

-	When there is a subscription change in the list of PLMNs where the UE is authorized to perform A2X communication over PC5 reference point. This is achieved by using UE Policy Association Modification initiated by the PCF procedure as defined in clause 4.16.12.2 of TS 23.502 [3].

-	When there is a change of service specific parameter as described in clause 4.15.6.7 of TS 23.502 [3].

If the serving PLMN is removed from the list of PLMNs in the service authorization parameters, the service authorization is revoked in the UE.

When the UE is roaming, the change of subscription resulting in updates of the service authorization parameters are transferred to the UE by H-PCF via V-PCF.

The UE may perform UE triggered Policy Provisioning procedure to the PCF, after Registration procedure has been completed, as specified in clause 5.10.3 when the UE determines the A2X Policy/Parameter is invalid (e.g. Policy/Parameter is outdated, missing or invalid).

End Change