3GPP TSG-SA WG2 Meeting #156e	S2-2304578

# April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0224 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Connection resume with local multicast session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Connection resume in RRC Inactive procedure has been added to the mobility procedures for MBS. For local multicast service, connection resume procedure need further enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add enhancements on connection resume procedure to support local multicast service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Fail to support local multicast service in connection resume procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.4.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

7.2.4.3.x	Connection Resume procedure with local MBS session

The Connection Resume procedure for the UE is performed as defined in clause 7.2.3.7 with the following additions:

-	the target RAN, source RAN, SMF and AMF behave as specified in clause 7.2.4.3.3 with the following differences:

-	the source RAN, if supporting MBS, provides MBS Session ID and MBS service area to the target RAN during the UE Context Retrieval procedure;

-	if the target NG-RAN supports MBS and the UE is in the MBS service area, the SMF provides the MBS session information to the target RAN, by updating the PDU session with the N2 SM information after the Path Switch procedure;

-	if the UE is out of the MBS service area and the target NG-RAN does not support MBS, the SMF deletes the associated QoS flow (if any) from NG-RAN node and the UPF after the Path Switch procedure.

* * * End of Changes * * * *