3GPP TSG-SA WG2 Meeting #156E	S2-2304597

# Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4035 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of NSSF notification for Network Slice Replacement and Network Slice instance Replacement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 23.501, AMF subscribes NSSF for Network Slice Replacement. However, this is not specified in TS 23.502. For Network Slice instance Replacement, NSSF sends notification based on implicit subscription. However, comparing to Network Slice Replacement, the procedure is quite different and there is no need to have different procedure for the simlar mehcanism. See further information in DP S2-2304598. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify NSSF service operation to support Network Slice Replacement and Network Slice instace Replacement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of NSSF notification for Network Slice Replacement and Network Slice instance Replacement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.16.1, 5.2.16.3.1, 5.2.16.3.3, 5.2.16.3.4, 5.2.16.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 5.2.16.1	General

The following table illustrates the NSSF Services.

Table 5.2.16.1-1: NF Services provided by NSSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnssf_NSSelection | Get | Request/Response | AMF, NSSF in a different PLMN, SMF, NWDAF |
| Nnssf_NSSAIAvailability | Update | Request/Response | AMF |
|  | Subscribe | Subscribe/Notify | AMF, NSSF |
|  | Unsubscribe |  | AMF, NSSF |
|  | Notify |  | AMF, NSSF |
|  | Delete | Request/Response | AMF |

NOTE:	The SMF uses the Nnssf_NSSelection_Get service operation during EPS interworking.

* * * * Start of 2nd Change * * * *

5.2.16.3	Nnssf_NSSAIAvailability service

##### 5.2.16.3.1	General

Service description: This service enables to update the AMFs and the NSSF on the availability of S-NSSAIs and NSAGs on a per TA basis. This service is also used to provide an Alternative S-NSSAI and/or NSI ID to NF Service Consumer in case when the NSSF determines that an S-NSSAI and/or NSI ID has to be replaced.

NOTE:	The NSSF can determine the serving network and Access Type from the TAI, as described in TS 29.571 [70].

* * * * Start of 3rd Change * * * *

##### 5.2.16.3.3	Nnssf_NSSAIAvailability_Notify service operation

Service operation name: Nnssf_NSSAIAvailability_Notify

Description: This service operation enables the NSSF to update a NF Service Consumer (e.g. AMF) with any S-NSSAIs restricted per TA and, if needed, subsequently lift any restriction per TA. This service operation is also enables to notify an NF Service Consumer (e.g. AMF) with network slice replacement information.

Inputs, Required: SubscriptionId

Inputs, Conditional Required:

If this service operation is invoked by subscription for changes in the status of the NSSAI availability information, then the following inputs are required:

-	a list of TAIs and the S-NSSAIs for which the status is changed (restricted/unrestricted) per each TAI.

If this service operation is invoked by subscription for event notification for slice replacement, then the following inputs are required:

-	For Network Slice Replacement: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAI to be replaced.

-	For Network Slice instance Replacement: a new NSI ID and the corresponding mapping to the old NSI ID to be replaced.

Inputs, Optional: NSI IDs in the S-NSSAI for which the status is changed (restricted/unrestricted) per each TAI.

Outputs, Required: None.

Outputs, Optional: None.

* * * * Start of 4th Change * * * *

##### 5.2.16.3.4	Nnssf_NSSAIAvailability_Subscribe service operation

Service operation name: Nnssf_NSSAIAvailability_Subscribe

Description: This service operation enables a NF Service Consumer (e.g. AMF) to subscribe to a notification of any changes in status of the NSSAI availability information (e.g. S-NSSAIs and optionally NSI IDs available per TA and the restricted S-NSSAI(s) and optionally NSI IDs per PLMN in that TA in the serving PLMN of the UE) upon this is updated by another AMF. This service operation also enables the NF Service Consumer (e.g. AMF) to subscribe to a notification when an S-NSSAI and/or an NSI ID needs to be replaced to Alternative S-NSSAI and/or new Network Slice instance.

Inputs, Required: Callback URI of the NF Service Consumer

Inputs, Conditional Required:

If this service operation is invoked to subscribe for change in status of the NSSAI availability information, then the following inputs are required:

-	list of TAIs supported by the NF service consumer, event to be subscribed.

If this service operation is invoked to subscribe to a notification for slice replacement:

-	Event ID(s), i.e. Network Slice Replacement and/or Network Slice instance Replacement.

-	List of S-NSSAIs for Network Slice Replacement

-	List of NSI IDs for Network Slice instance Replacement

Inputs, Optional: Expiry time.

Outputs, Required: SubscriptionID,

Outputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following outputs are required:

-	Expiry time (if present in the request, may be included in the response based on operator's policy and taking into account the expiry time present in the request (i.e. should be less than or equal to that value); if not present in the request, may be included in the response based on operator's policy. Whatever the case, if not included in the response, this means that the subscription is valid without an expiry time).

If this service operation is invoked to subscribe to a notification for slice replacement:

-	For Network Slice Replacement: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAI to be replaced.

-	For Network Slice instance Replacement: a new NSI ID and the corresponding mapping to the old NSI ID to be replaced.

Outputs, Optional: A list of TAIs and, for each TAI, the S-NSSAIs and optionally NSI IDs supported by the AMF and 5G-AN and authorized by the NSSF for the TAI and a list of S-NSSAIs and optionally NSI IDs restricted per PLMN for the TAI.

* * * * Start of 5th Change * * * *

##### 5.2.16.3.5	Nnssf_NSSAIAvailability_Unsubscribe service operation

Service operation name: Nnssf_NSSAIAvailability_Unsubscribe

Description: This service operation enables a NF Service Consumer (e.g. AMF) to unsubscribe to a notification of any previously subscribed changes to the NSSAI availability information. This service operation also enables the NF Service Consumer (e.g. AMF) to unsubscribe to a notification when an S-NSSAI and/or an NSI ID needs to be replaced to Alternative S-NSSAI and/or new Network Slice instance.

Inputs, Required: SubscriptionId.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

* * * * End of Changes * * * *